| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0110-v1 |
| --- | --- |

| Title: | Non-EE2: An improved method for IntraTMP fusion |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shuang Peng, Dong Jiang, Jucai Lin, Chunhuang Zheng, Kai Fu, Peng Zhang | Tel: Email: | jiang_dong@dahuatech.com lin_jucai@dahuatech.com |
| Source: | Dahua (ZheJiang Dahua Technology CO.,LTD.) |  |  |

_____________________________

- Abstract

This contribution proposes an improved method for IntraTMP fusion. In this method, the filtered matched blocks are used in the IntraTMP fusion process. The proposed method was implemented on top of ECM-9.0, it reportedly provides {Y, U, V} BD-reduction with encoder and decoder runtimes as follows:

AI: -0.0x%, -0.0x%, -0.0x%, 10x% (EncT), 10x% (DecT)

Introduction

In the last meeting, an IntraTMP fusion method was included in ECM-9.0 [1]. In the fusion process, the SAD based weight derivation method or the wiener filter weight based derivation method is used. For the SAD based derivation method, the fused predictor pfusion is calculated by:

pfusion=n=1Nwnpn

where N is the number of matched blocks selected for fusion, pn is the nth matched block to be fused, wn is the corresponding fusion weight and it is calculated as follows:

wn= sum-SADnN-1*sum

sum= n=1NSADn

In addition, a linear filter method for IntraTMP prediction was also included to improve the matched blocks.

To improve the fusion process, the block filtered with the linear filter is used for the SAD based derivation method.

Proposed method

In this contribution, the proposed method includes the following aspects.

a) Generating the filtered matched blocks

During the IntraTMP search process, a candidate list is generated for 42 matched blocks with the smallest template SAD. The matched blocks and the corresponding template is filtered with the linear filter.

b) Reordering the filtered matched blocks

In this process, the filtered template SAD for each matched block is calculated. Moreover, the order of the matched blocks is sorted by the filtered SADs and up to 15 candidates are retained.

c) Fusing the filtered matched blocks

This process similar to the IntraTMP fusion method with the SAD based derivation, but the blocks to be fused are filtered and the SADs are updated by the filtered SADs.

The proposed method is signalled with a CU level flag and is considered as a sub-mode of the IntraTMP fusion method.

Experimental results

The proposed method is implemented on top of ECM-9.0 software under the common test conditions [2]. Table 1 presents a summary of the results under AI configuration.

Results of the proposed method under AI configuration

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.05% | 0.01% | -0.04% | #DIV/0! | #DIV/0! |
| Class C | -0.04% | 0.01% | 0.09% | 103.2% | 101.9% |
| Class E | -0.09% | -0.06% | -0.12% | 105.3% | 98.2% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.00% | 0.09% | 0.04% | 104.1% | 101.3% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Conclusions

This contribution suggests using the filtered matched blocks to improve the prediction performance of IntraTMP fusion. The results show that the proposed method can bring more coding gains.

References

- ECM-9.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-9.0.

- M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing”, JVET-AD2017, Apr. 2022.

Patent rights declaration(s)

Dahua Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).