| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0112-v2 |
| --- | --- |

| Title: | EE1-5.1: Deep Reference Frame Generation for Inter Prediction Enhancement |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weijie Bao, Wenhui Meng, Jianghao Jia, Yuantong Zhang, Huairui Wang, Zhenzhong Chen Wuhan University Zizheng Liu, Xiaozhong Xu, Shan Liu Tencent | Email: | {baoweijie, whmeng, jiajh2021, yuantongzhang, wanghr827, zzchen}@whu.edu.cn {zizhengliu, xiaozhongxu, shanl} @tencent.com |
| Source: | Wuhan University, Tencent |  |  |

_____________________________

- Abstract

This contribution reports the EE1-5.1 results of NN-based inter prediction. Based on JVET-AD0162 [1], this contribution has simplified the network architecture to reduce the parameters and implemented the float32 SADL inference process. It is reported that this method can achieve X.XX%/X.XX%/X.XX% bitrate savings for the Y/U/V components under the RA configuration when compared to the anchor VTM-11.0_NNVC-5.0, all without the utilization of any NNLF coding tools.

In the v2 version, we have updated the description of our modifications and experimental results.

Introduction

JVET-AD0162 [1] proposes a deep reference frame generation method to enhance VVC inter prediction, which was introduced into EE in the 30th JVET meeting. During the encoding and decoding process, the network receives two reconstructed frames from the decoded picture buffer as inputs and generates a new frame. The generated frame will be put into the reference picture lists as an additional reference for the current encoding picture, which enhances the inter prediction.

In this contribution, the network architecture is simplified, and the SADL inference process is implemented based on the method in [1]. The main modifications are summarized as:

a) Remove the PFRB module and ResBlocks.

b) Replace the IFRNet module with the small IFRNet module.

c) Reduce the number of intermediate channels in some certain module.

d) Implement the float32 SADL inference process.

Proposed Method

Framework

Fig. 1: The framework of the proposed DRF method

The framework of the proposed DRF method is depicted in Figure 1. In this approach, two reconstructed frames chosen from the DPB are inputted into the deep reference frame generation network to synthesize a new frame. This synthesized frame is then added to both reference picture list0 and list1, using the same POC as the current encoding frame. It serves as an additional reference frame for subsequent encoding processes.

Network architecture

Fig. 2: The architecture of the DRF network

Figure 2 illustrates the architecture of the proposed DRF network. In this contribution, architectural simplifications have been made to reduce the number of network parameters. Specifically, the PFRB module and ResBlocks have been eliminated, opting instead for the utilization of the smaller IFRNet module for optical flow estimation. Besides, the number of intermediate channels in some modules is decreased. These modifications have resulted in a reduction in the number of parameters from 7797K to 3800K and a decrease in the multiply-accumulate operations from 986 kMAC/pixel to 504 kMAC/pixel.

SADL implementation

Using the extended SADL library [2], a SADL inference method of JVET-AD0162 [1] has been proposed. Currently, only the float32 inference model is completed, and the SIMD optimization is not supplied.

Network information in training stage

Table 1: Network information for the NN-based video coding tool testing in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GTX 3090 x 2 x 24GB |
|  | Framework | PyTorch v1.9.0 |
|  | Number of GPUs per Task | 2 |
|  | Epoch | 200 |
|  | Batch size | 3Kx24 |
|  | Loss function | L1 |
|  | Training time (for 1 model) | 60h |
|  | Training data information | Vimeo90K |
|  | Training configurations for generating compressed training data (if different to VTM CTC) | VTM-11.0_NNVC-5.0, QP{22, 27, 32, 37, 42} |
| Optional | Number of iterations |  |
|  | Patch size | 256x256 |
|  | Learning rate | 1e-4 |
|  | Learning rate update strategy |  |
|  | Optimizer | ADAM |
|  | Preprocessing |  |
|  | Mini-batch selection process |  |
|  | Training data update strategy |  |
|  | Other information |  |

Network information in inference stage

Table 2: Network information for the NN-based video coding tool testing in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment |  |
|  | GPU Type | CPU only |
|  | Framework | SADL |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 3800K |
|  | Total Number of Parameters (All Models) | 3800K |
|  | Parameter Precision (Bits) | 32 |
|  | Memory Parameter (MB) | 14.9 |
|  | Multiply Accumulate (kMAC/pixel) | 504 |
| Optional | Total Conv. Layers |  |
|  | Total FC Layers |  |
|  | Total Memory (MB) |  |
|  | Batch size |  |
|  | Patch size | 480x480 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information |  |

Results

The method has been implemented on the VTM-11.0_NNVC-5.0, and the experimental results of the proposed DRF method under RA configuration are presented in Table 3. It is important to note that these experiments were conducted without employing any NNLF modules or other NN-based methods.

As depicted in Table 3, the proposed method demonstrates a BD-rate reduction of X.XX%/X.XX%/X.XX% for the Y/U/V components compared to the anchor. The performance is superior to the method in JVET-AD0162 [1]. However, considering that the proposed method reduces the number of network parameters by nearly half, it offers a better trade-off between performance and network complexity. Additionally, it is worth mentioning that the SADL implementation does not utilize any SIMD optimization, resulting in significantly higher computational times for both encoding and decoding processes.

Table 3: Performance of the proposed method (RA, full length)

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_NNVC-5.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | -3.20% | -7.25% | -6.55% | -3.77% | -9.25% | -8.30% | 920% | 1285555% |
| Class C | -4.48% | -6.97% | -7.20% | -5.98% | -9.62% | -9.94% | 570% | 914553% |
| Class E |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |
| Class D | -5.53% | -5.94% | -6.48% | -5.72% | -10.42% | -9.95% | 989% | 1673983% |
| Class F | -1.40% | -1.76% | -1.27% | -2.20% | -3.25% | -3.32% | 1471% | 1526972% |

Conclusions

This contribution presents a deep reference frame generation (DRF) method for VVC inter prediction enhancement. Leveraging the VTM-11.0_NNVC-5.0 platform, the proposed method can bring X.XX%/X.XX%/X.XX% coding efficiency improvements with a computational complexity of 504 kMAC/pixel under RA configuration, which achieves better trade-off compared to the JVET-AD0162.

Reference

[1] J. Jia, D. Ding, W. Meng, Z. Chen, Z. Liu, X. Xu, and S. Liu. “EE1-2.1-related: DRF Model without QP Input”. JVET-AD0162, 30th Meeting, Antalya, TR, 21–28 April 2023.

[2] W. Bao, D. Ding, J. Wang, J. Jia, and Z. Chen. “AHG14: The extension of SADL library”. JVET-AD0365, 30th Meeting, Antalya, TR, 21–28 April 2023.

Patent rights declaration(s)

Wuhan University does not have any current or pending patent rights relating to the technology described in this contribution.

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).