| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0114-v2 |
| --- | --- |

| Title: | AHG10: Low-Delay configuration improvements |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jack Enhorn Per Wennersten | Email: | jack.enhorn@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This document proposes changes to CTC low-delay configurations including Intra QP offset, periodic QP offsets every 64th picture, and changes to GOP structure and reference picture lists.

Reported results on CTC on top of VTM and ECM are as follows:

VTM LD-B Y/U/V: -5,62%/-10,34%/-11,24%

VTM LD-P Y/U/V: -4,94%	-9,76%	-10,58%

ECM LD-B Y/U/V: -4,49%/-9,08%/-9,47% (estimated)

ECM LD-P Y/U/V: -3,57%/-7,94%/-8,29%

# Introduction

For the current Random Access configuration, the compression efficiency is greatly helped by the fact that periodic intra pictures are boosting the quality and increasing the reusability of samples. For the low-delay configuration however, the increased quality of the intra picture is both less dramatic from the start (-1 QP compared to -3 of RA) and quickly dissipates with the distance from this first and only intra picture.

Figure 1: QP-offset structure in CTC low-delay B configuration

In this proposal, an encoder-only method for periodic lower QP pictures is presented. It has a modified reference structure to fully utilize these boosted pictures as references.

# Proposed method

Firstly, the intra picture QP offset is aligned with RA at -3 compared to the base QP. This is implemented by simply changing the configuration file:

IntraQPOffset                     : -3

Secondly, every 64th picture is boosted to an offset of QP -2. Two configuration parameters for the offset and period of the QP changes are added, this is an encoder-only software change:

LowdelayBoostPeriod       : 64

LowdelayBoostQPOffset  : -3  (added on top of GOP configuration QP offset which is  +1 resulting in -2)

Thirdly, the reference picture structure is modified so all pictures will reference these boosted pictures that occur every 64 pictures. This enables all pictures to fully utilize the potential of these reference samples. Currently the low-delay configuration reference picture lists include every first picture of the GOP-8 structure referencing back up to 33 pictures. In the proposed method the GOP is extended to 16 frames with doubling the distance of the reference pictures further back up to 66 pictures back.

Lastly, both due to the fact that every 8th picture in this proposed configuration is not used as frequently in the reference picture lists as in the anchor and also to counter an increase of the average bitrate, the QP of middle pictures of the GOP-16 is increased by 2. The starting picture of a GOP, i.e., every 16th picture, is kept at QP +1.

Figure 2: QP-offset structure of proposed low-delay B configuration

Example QPs and reference picture lists (base QP 22):

| Anchor | Proposed method |
| --- | --- |
| POC 48 ( QP 23 ) [L0 47 40 32 24] [L1 47c 46 44 16] POC 49 ( QP 28 ) [L0 48 40 32 24] [L1 48c 46 44 16] POC 50 ( QP 26 ) [L0 49 48 40 32] [L1 49c 48 46 24] POC 51 ( QP 28 ) [L0 50 48 40 32] [L1 50c 48 46 24] POC 52 ( QP 26 ) [L0 51 48 40 32] [L1 51c 50 48 24] POC 53 ( QP 28 ) [L0 52 48 40 32] [L1 52c 50 48 24] POC 54 ( QP 26 ) [L0 53 48 40 32] [L1 53c 52 48 24] POC 55 ( QP 28 ) [L0 54 48 40 32] [L1 54c 52 48 24] POC 56 ( QP 23 ) [L0 55 48 40 32] [L1 55c 54 52 24] POC 57 ( QP 28 ) [L0 56 48 40 32] [L1 56c 54 52 24] POC 58 ( QP 26 ) [L0 57 56 48 40] [L1 57c 56 54 32] POC 59 ( QP 28 ) [L0 58 56 48 40] [L1 58c 56 54 32] POC 60 ( QP 26 ) [L0 59 56 48 40] [L1 59c 58 56 32] POC 61 ( QP 28 ) [L0 60 56 48 40] [L1 60c 58 56 32] POC 62 ( QP 26 ) [L0 61 56 48 40] [L1 61c 60 56 32] POC 63 ( QP 28 ) [L0 62 56 48 40] [L1 62c 60 56 32] POC 64 ( QP 23 ) [L0 63 56 48 40] [L1 63c 62 60 32] | POC 48 ( QP 23 ) [L0 47 32 16 0] [L1 47c 44 40 32] POC 49 ( QP 28 ) [L0 48 32 16 0] [L1 48c 44 40 32] POC 50 ( QP 26 ) [L0 49 32 16 0] [L1 49c 48 44 32] POC 51 ( QP 28 ) [L0 50 48 32 16] [L1 50c 48 44 0] POC 52 ( QP 26 ) [L0 51 48 32 16] [L1 51c 48 44 0] POC 53 ( QP 28 ) [L0 52 48 32 16] [L1 52c 48 44 0] POC 54 ( QP 26 ) [L0 53 48 32 16] [L1 53c 52 44 0] POC 55 ( QP 28 ) [L0 54 48 32 16] [L1 54c 52 48 0] POC 56 ( QP 25 ) [L0 55 48 32 16] [L1 55c 52 48 0] POC 57 ( QP 28 ) [L0 56 48 32 16] [L1 56c 52 48 0] POC 58 ( QP 26 ) [L0 57 48 32 16] [L1 57c 56 48 0] POC 59 ( QP 28 ) [L0 58 48 32 16] [L1 58c 56 48 0] POC 60 ( QP 26 ) [L0 59 48 32 16] [L1 59c 56 48 0] POC 61 ( QP 28 ) [L0 60 48 32 16] [L1 60c 56 48 0] POC 62 ( QP 26 ) [L0 61 48 32 16] [L1 61c 60 56 0] POC 63 ( QP 28 ) [L0 62 48 32 16] [L1 62c 60 56 0] POC 64 ( QP 20 ) [L0 63 48 32 16] [L1 63c 60 56 0] |

# Results

Following are the CTC results of the method on top of VTM-20.0:

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-20.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -2,28% | -6,73% | -7,10% | 107% | 100% |
| Class C | -6,48% | -9,72% | -9,33% | 111% | 98% |
| Class E | -10,06% | -17,19% | -20,67% | 104% | 99% |
| Overall | -5,62% | -10,34% | -11,24% | 107% | 99% |
| Class D | -2,55% | -8,99% | -9,24% | 105% | 98% |
| Class F | -7,60% | -8,23% | -8,00% | 104% | 98% |

|  | Low delay P |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-20.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1,13% | -5,70% | -6,32% | 103% | 99% |
| Class C | -6,19% | -9,32% | -8,95% | 98% | 100% |
| Class E | -9,60% | -17,11% | -19,87% | 97% | 101% |
| Overall | -4,94% | -9,76% | -10,58% | 99% | 100% |
| Class D | -2,23% | -10,02% | -9,62% | 101% | 100% |
| Class F | -5,30% | -5,57% | -5,21% | 102% | 100% |

Following are the results of the method on top of ECM-9.0:

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B* |  | -1,23% |  |  | -3,88% |  |  | -5,45% |  |  | 104% |  |  | 100% |  |  |
| Class C | -6,05% | -10,04% | -8,74% | 99% | 99% |
| Class E | -7,84% | -16,44% | -17,15% | 105% | 100% |
| Overall |  | -4,49% |  | -9,08% |  |  | -9,47% |  |  | 103% |  |  | 100% |  |  |
| Class D | -3,02% | -7,06% | -7,73% | 101% | 99% |
| Class F | -2,62% | -2,63% | -2,36% | 99% | 100% |
| Class TGM | 1,23% | 1,98% | 2,21% | 99% | 99% |

*Estimated, anchor results copied for BQTerrace and BasketballDrive QP 22

|  | Low delay P |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  | 0,05% |  | -2,56% |  |  | -3,82% |  |  | 99% |  |  | 99% |  |  |
| Class C | -5,53% | -8,73% | -7,56% | 97% | 98% |
| Class E | -7,00% | -15,84% | -16,71% | 99% | 100% |
| Overall |  | -3,57% |  |  | -7,94% |  |  | -8,29% |  |  | 99% |  |  | 99% |  |  |
| Class D | -2,28% | -6,82% | -7,22% | 99% | 100% |
| Class F |  | -2,83% |  |  | -2,39% |  |  | -1,77% |  |  | 98% |  |  | 99% |  |  |
| Class TGM |  | -2,46% |  |  | -2,74% |  |  | -2,32% |  |  | 105% |  |  | 102% |  |  |

(New decoder time measurement done since v1 of this document)

Encoder times are not reliable due to a heterogeneous cluster environment. Decoder times more reliable. As no significant additional calculations are proposed, any differences in times must be due to different QPs/bitrates. The average bitrate of the full CTC is within ±1% of average anchor bitrate.

The difference in BD-Rate gains between VTM and ECM are mainly due to VTM CTC uses 10 second sequences while ECM CTC restrict encoding to 5 seconds of the sequences. VTM results for 5 second sequences are as following:

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-20.0 5s |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,90% | -5,03% | -5,66% | 112% | 104% |
| Class C | -6,02% | -10,42% | -9,24% | 112% | 113% |
| Class E | -7,96% | -15,33% | -18,85% | 104% | 101% |
| Overall | -4,37% | -9,40% | -10,15% | 110% | 106% |
| Class D | -2,78% | -8,67% | -8,90% | 108% | 112% |
| Class F | -2,41% | -2,38% | -1,48% | 103% | 101% |

|  | Low delay P |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-20.0 5s |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0,09% | -3,60% | -4,61% | 100% | 98% |
| Class C | -5,41% | -9,53% | -8,74% | 97% | 99% |
| Class E | -7,61% | -15,14% | -18,16% | 98% | 95% |
| Overall | -3,67% | -8,46% | -9,37% | 99% | 97% |
| Class D | -2,41% | -9,35% | -8,01% | 99% | 95% |
| Class F | -2,71% | -2,23% | -2,36% | 97% | 95% |

Neither encoder nor decoder times are reliable for the VTM 5 second test results.

Subjective benefits have been observed with higher background quality and faster recovery of areas when movements slow down or become static. In the example below from the KristenAndSara sequence, the hand to the left has moved over the face in the background and then back.

Figure 3: Anchor

Figure 4: Proposed method

# Conclusion

In this proposal a method was presenting with periodic QP-offsets and updated reference picture structure for low-delay coding. The objective gains are significant and subjected benefits have also been observed. It is proposed to adopt the method into CTC and software of VTM and ECM.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).