| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0120 |
| --- | --- |

| Title: | Non-EE2: Intra Angular Prediction Extension |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jiaye Fu, Jiaqi Zhang, Chuanmin Jia, Siwei Ma (PKU) Ying Gao, Cheng Huang (ZTE) | Email: | jiaye.fu@ucdconnect.ie huang.cheng5@zte.com.cn |
| Source: | PKU, ZTE |  |  |

_____________________________

- Abstract

This document illustrates test results of the proposed method.

On top of the ECM-9.0, simulation result of the proposed method is reported as:

AI: -0.02%, -0.01%, 0.21%, 124%, 106%

Introduction

In ECM, current angular intra prediction is based on linear projection. 1 of 67 intra prediction modes is chosen to be the angle of projection, and the location of reference pixel are calculated, then the value of projecting pixel is calculated through 6-tap intra filter and the value of reference pixel.

Proposed method

In this contribution, it is proposed to extend the current linear projection function into quadratic projection function that takes two intra prediction modes. When two intra prediction modes are belonging to the set of vertical intra prediction mode, the location of reference pixel is inferred by the location of predicting pixel and the quadratic function generated by two intra prediction modes that shows below:

c=x0-tan β-tan α2*h*y02-tan α *y0

where α,  β are intra prediction modes that belongs to the set of vertical intra prediction mode, and h is the height of the coding block, x0,  y0 are the horizontal and vertical ordinate of the predicting pixel, and c is the horizontal location of reference pixel. When two intra prediction modes are belonging to the set of horizontal intra prediction mode, the location of reference pixel is inferred by the location of predicting pixel and the quadratic function generated by two intra prediction modes that shows below:

c=y0-tan β-tan α2*w*x02-tan α *x0

where α,  β are intra prediction modes that belongs to the set of vertical intra prediction mode, and w is the width of the coding block, x0,  y0 are the horizontal and vertical ordinate of the predicting pixel, and c is the horizontal location of reference pixel. After that, the six-tap intra filter is used to calculate the value of predicting pixel with the value of reference pixel.

Experimental results

The proposed method is implemented on top of the ECM-9.0 software [1] and tested based on the common test condition [2]. The test results are summarized in Table 1.

Table I: Test results of proposed method under CTC

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.29% | 0.23% | 1.89 | 1.36 |
| Class A2 | 0.01% | 0.06% | 0.22% | 1.46 | 1.20 |
| Class B | -0.04% | 0.58% | 0.14% | 1.12 | 1.02 |
| Class C | 0.04% | -0.26% | 0.02% | 1.02 | 0.95 |
| Class E | -0.03% | -0.45% | 0.58% | 1.03 | 0.99 |
| Overall | -0.02% | -0.01% | 0.21% | 1.24 | 1.06 |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

Conclusion

In this contribution, intra angular prediction extension is proposed. The proposed method brings -0.02% coding gain for AI with 124% and 106% encoding and decoding time.

References

- ECM-9.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-9.0

- V. Seregin, J. Chen, R. Chernyak, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)”, JVET-AD2024, April 2023.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AD2017, April, 2023.

Patent rights declaration(s)

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).