| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0124-v1 |
| --- | --- |

| Title: | Non-EE2: Fixes related to Intra TMP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian-Liang Lin Po-Han Lin Yao-Jen Chang Vadim Seregin Marta Karczewicz | Tel: Email: | jilin@qti.qualcomm.com pohanlin@qti.qualcomm.com yjchang@qti.qualcomm.com vseregin@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In ECM-9.0, it is asserted that the cost list for Intra TMP candidates is not updated correctly and the BV storage in Intra TMP is not aligned with the IBC. This contribution proposes to fix those issues, and the simulation results on top of ECM-9.0 are summarized as follows:

AI(Y/U/V): X.XX%/ X.XX %/ X.XX %, EncT XX %, DecT XX %

RA(Y/U/V): X.XX %/ X.XX %/ X.XX %, EncT XX %, DecT XX %

# Introduction

In JVET-AD0086, multiple-candidate Intra TMP was adopted to ECM. An Intra TMP candidate list is constructed from candidates derived by L-shape, left or above template based on the template matching cost. However, during the candidate list construction by updating the candidates derived from left and above templates, only the candidate BVs are updated in the candidate list, and the candidates’ template matching costs are not updated correctly. Therefore, the cost of the reordering L-shape candidates is incorrect, and the cost of L-shape candidates is misassigned to the left or above candidates.

If a block uses Intra TMP or IBC, the BV of this block will be stored to the spatial and history BV buffers. The buffers will be referenced by IBC AMVP, IBC merge, and chroma DBV modes so that the previous coded BV information can help to increase the coding efficiency of the current block. However, the BV storing procedures for IBC and Intra TMP are different even if they are using similar concept to generate the predictor from block vector. When fractional BV is used in IBC and Intra TMP for the current block, IBC block will store the true fractional BV to the BV storage, whereas Intra TMP block only stores the integer part of the BV. Besides, the BV from the IBC block is updated in the history BV buffer, but the BV from Intra TMP block is not.

# Proposed method

The proposed fix 1 is to correctly update the candidates’ template matching cost. The template matching cost is calculated using SAD between the template of current block and the template of the reference block. Since the numbers of pixels in the L-shape, left and above templates are different, the template matching costs from left and above templates are normalized according to the template sizes.

In the proposed fix 2, a true fractional per Intra TMP BV is stored in the BV and history buffers as for IBC blocks.

# Experiment result

The proposed fixes were implemented on top of ECM-9.0 and tests under the common test conditions [3].

## Test results of fix 1 on top of ECM-9.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.05% | 0.00% | 100% | 101% |
| Class A2 | 0.00% | -0.07% | 0.05% | 100% | 98% |
| Class B | 0.00% | 0.08% | 0.09% | 101% | 100% |
| Class C | 0.00% | -0.03% | -0.02% | 101% | 98% |
| Class E | 0.00% | 0.06% | 0.04% | 100% | 98% |
| Overall | 0.00% | 0.01% | 0.03% | 100% | 99% |
| Class D | 0.02% | 0.02% | -0.02% | 100% | 96% |
| Class F | 0.00% | -0.03% | 0.01% | 99% | 97% |
| Class TGM | -0.02% | -0.04% | 0.02% | 100% | 98% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

## Test results of fix 2 on top of ECM-9.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM9 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.04% | -0.04% | 97% | 97% |
| Class A2 | 0.00% | 0.05% | 0.02% | 99% | 103% |
| Class B | -0.02% | 0.02% | 0.00% | 100% | 104% |
| Class C | -0.01% | -0.08% | 0.02% | 102% | 111% |
| Class E | -0.01% | 0.14% | 0.10% | 99% | 105% |
| Overall | -0.01% | 0.03% | 0.02% | 100% | 104% |
| Class D | 0.01% | 0.09% | -0.07% | 100% | 104% |
| Class F | -0.07% | -0.04% | -0.08% | 99% | 100% |
| Class TGM | -0.10% | -0.11% | -0.14% | 99% | 102% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM9 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.01% | -0.08% | 0.04% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.02% | 0.01% | 0.17% | 101% | 103% |
| Class F | -0.07% | -0.05% | -0.11% | 98% | 101% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

## Test results of combination of fix 1 and fix 2 on top of ECM-9.0

# References

[1] F. Wang, etc., “EE2-1.20i/j: Combination of IntraTMP tests”, JVET-AD0086, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 30th Meeting, Antalya, TR, 21–28 April 2023.

[2] ECM-9.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-9.0.

[3] M. Karczewicz, et. al., “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, February 2022.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).