| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0128-v1 |
| --- | --- |

| Title: | AHG9: On NNPFC extensibility and base filter signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang 8910 University Center Lane San Diego, CA 92122, USA | Email: | yekui.wang@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following changes to the NNPFC SEI message:

- The metadata extension mechanism, enabled by the syntax elements nnpfc_metadata_extension_num_bits and nnpfc_reserved_metadata_extension, is specified to be applied only when nnpfc_mode_idc is equal to 0, by making the following changes:

  - The signalling of the syntax element nnpfc_metadata_extension_num_bits is skipped when nnpfc_mode_idc is not equal to 0.

  - It is constrained that, when nnpfc_metadata_extension_num_bits is not present, its value is inferred to be equal to 0.

- Update the constraint on when an NNPFC SEI message shall be a repetition of the first NNPFC SEI message in the CLVS to be as follows, such that it is clearly specified that the current CLVS is the CLVS containing the NNPFC SEI message nnpfcB, and that the first NNPFC SEI message nnpfcA is within the current CLVS:

When an NNPFC SEI message nnpfcB is not the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, which contains nnpfcB, and the value of nnpfc_base_flag is equal to 1, the NNPFC SEI message shall be a repetition of the first NNPFC SEI message nnpfcA with the same nnpfc_id value, in decoding order, within the current CLVS, i.e., the payload content of nnpfcB shall be the same as that of nnpfcA.

# On NNPFC extensibility

## Introduction

When nnpfc_mode_idc is equal to 1, the general SEI extension mechanism, enabled by the syntax element sei_reserved_payload_extension_data in the sei_payload( ) syntax and the syntax condition for presence of this syntax element, can be applied, same as for all other SEI messages.

When nnpfc_mode_idc is equal to 0, that general SEI extension mechanism cannot be applied due to the use of more_data_in_payload( ) for the loop of the nnpfc_payload_byte[ i ] syntax elements. To enable future extensibility for the case of nnpfc_mode_idc equal to 0 in a backward compatible manner, the metadata extension mechanism, enabled by the syntax elements nnpfc_metadata_extension_num_bits and nnpfc_reserved_metadata_extension, was included.

However, currently the metadata extension mechanism also applies when nnpfc_mode_idc is equal to 1, which adds complexity unnecessarily.

It is therefore proposed that the metadata extension mechanism, enabled by the syntax elements nnpfc_metadata_extension_num_bits and nnpfc_reserved_metadata_extension, is specified to be applied only when nnpfc_mode_idc is equal to 0.

## Proposal

The proposed text changes are as follows (additions are highlighted in turquoise, and removals are highlighted in yellow strikethrough in red fonts.

### 8.28.2.1 Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| nnpfc_id | ue(v) |
| nnpfc_base_flag | u(1) |
| nnpfc_mode_idc | ue(v) |
| if( nnpfc_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_reserved_zero_bit_a | u(1) |
| nnpfc_tag_uri | st(v) |
| nnpfc_uri | st(v) |
| } |  |
| nnpfc_property_present_flag | u(1) |
| if( nnpfc_property_present_flag ) { |  |
| ... |  |
| if( nnpfc_mode_idc = = 0 ) |  |
| nnpfc_metadata_extension_num_bits | ue(v) |
| if( nnpfc_metadata_extension_num_bits > 0 ) |  |
| nnpfc_reserved_metadata_extension | u(v) |
| } |  |
| /* ISO/IEC 15938-17 bitstream */ |  |
| if( nnpfc_mode_idc = = 0 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_reserved_zero_bit_b | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| nnpfc_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

### 8.28.2.2 Neural-network post-filter characteristics SEI message semantics

...

nnpfc_metadata_extension_num_bits equal to 0 specifies that nnpfc_reserved_metadata_extension is not present. nnpfc_metadata_extension_num_bits greater than 0 specifies the length, in bits, of nnpfc_reserved_metadata_extension. When nnpfc_metadata_extension_num_bits is not present, its value is inferred to be equal to 0.

nnpfc_metadata_extension_num_bits shall be equal to 0 in this edition of this document. Values in the range of 1 to 2 048, inclusive, for nnpfc_metadata_extension_num_bits are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall allow any value of nnpfc_metadata_extension_num_bits in the range of 0 to 2 048, inclusive. Values of nnpfc_metadata_extension_num_bits greater than 2 048 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

nnpfc_reserved_metadata_extension shall not be present in bitstreams conforming to this edition of this document. However, decoders conforming to this edition of this document shall ignore the presence and value of nnpfc_reserved_metadata_extension. When present, the length, in bits, of nnpfc_reserved_metadata_extension is equal to nnpfc_metadata_extension_num_bits.

...

# On base NNPF signalling

## Introduction

The following constraint is specified on the value of nnpfc_base_flag:

When an NNPFC SEI message nnpfcB is not the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS and the value of nnpfc_base_flag is equal to 1, the NNPFC SEI message shall be a repetition of the first NNPFC SEI message nnpfcA with the same nnpfc_id value, in decoding order, i.e., the payload content of nnpfcB shall be the same as that of nnpfcA.

However, it should be clearly specified that the current CLVS is the CLVS containing the NNPFC SEI message nnpfcB, and it should also be clearly specified that the first NNPFC SEI message nnpfcA is within the current CLVS.

## Proposal

The proposed text changes are as follows (additions are highlighted in turquoise, and removals are highlighted in yellow strikethrough in red fonts.

### 8.28.2.2 Neural-network post-filter characteristics SEI message semantics

...

nnpfc_base_flag equal to 1 specifies that the SEI message specifies the base NNPF. nnpf_base_flag equal to 0 specifies that the SEI message specifies an update relative to the base NNPF.

The following constraints apply to the value of nnpfc_base_flag:

–	When an NNPFC SEI message is the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, the value of nnpfc_base_flag shall be equal to 1.

–	When an NNPFC SEI message nnpfcB is not the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, which contains nnpfcB, and the value of nnpfc_base_flag is equal to 1, the NNPFC SEI message shall be a repetition of the first NNPFC SEI message nnpfcA with the same nnpfc_id value, in decoding order, within the current CLVS, i.e., the payload content of nnpfcB shall be the same as that of nnpfcA.

When nnpfc_base_flag is equal to 0, the following applies:

–	This SEI message defines an update relative to the preceding base NNPF in decoding order with the same nnpfc_id value. Updates are not cumulative but rather each update is applied on the base NNPF, which is the NNPF specified by the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS. The NNPF defined by this SEI message is obtained by applying the update defined by this SEI message relative to the base NNPF with the same nnpfc_id value.

–	This SEI message pertains to the current decoded picture and all subsequent decoded pictures of the current layer, in output order, until the end of the current CLVS or up to but excluding the decoded picture that follows the current decoded picture in output order within the current CLVS and is associated with a subsequent NNPFC SEI message, in decoding order, having nnpfc_base_flag equal to 0 and that particular nnpfc_id value within the current CLVS, whichever is earlier.

...

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).