| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0131_v1 |
| --- | --- |

| Title: | Non-EE2: Variance based Classification for In-loop Filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenbin Yin1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | yinwenbin.hit@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

Abstract

In this contribution, a variance-based classification for in-loop filtering is proposed. In the proposed method, variance is computed and used as an additional classification rule for BF and ALF. The existing classification methods in BF and ALF are kept unchanged.

On top of ECM-9.0, simulation results of the proposed methods are reported as below:

AI: *-0.02%, -0.02%, 0.03%, 100.0%, 100.1%.

RA: *-0.04%, -0.12%, 0.07%, 100.0%, 99.8%.

LB: %, %, %, %, %.

Introduction

In adaptive loop filter (ALF) of ECM-9.0, there are 3 classifiers known as band-based, residual-based, and texture-based. The band-based classification divides samples into 25 groups according to the sample value range. The residual-based classification divides residuals into 25 groups according to the absolute values. The texture-based classification considers the gradient information to classify each unit into 5 texture directions and 5 texture strengths, resulting in 25 classes in total too. The classifiers are switchable at CTU level selected by the encoder.

In bilateral filter (BF) of ECM-9.0, the filter strength is only related to prediction mode, size and QP of current TU. The texture strength inside a TU is not considered.

Proposed Method

In this contribution, a variance-based classification for BF and ALF is proposed.

For BF, the variance of a TU is utilized to identify the texture strength. More filter strengths are introduced into BF. The filtering strength is determined by the variance.

For ALF, each classification unit is classified into 2 levels of texture strength based on variance and boundary position jointly. Then the texture strength levels are further combined with the existing classifiers to output the final classification results.

Simulation Results

The proposed method was implemented on top of the ECM-9.0 [1]. Simulations were performed following the common test conditions [2]. More detailed results could be found in the attached excel sheets. The results compared to ECM-9.0 are shown in TABLE 1.

TABLE 1 Performance of the Proposed Method

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 | -0.01% | -0.01% | 0.01% | 99.7% | 100.1% |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% | -0.04% | 0.03% | 99.1% | 100.2% | -0.02% | -0.05% | 0.15% | 100.3% | 99.6% |  |  |  |  |  |
| Class C | -0.04% | -0.01% | 0.06% | 100.9% | 99.6% | -0.11% | -0.18% | 0.05% | 99.6% | 100.0% | -0.15% | -0.31% | -0.40% | 99.7% | 100.0% |
| Class E | -0.05% | 0.02% | 0.09% | 100.3% | 100.7% |  |  |  |  |  | 0.03% | -1.04% | 0.78% | 100.5% | 100.0% |
| Overall |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.02% | -0.07% | 0.13% | 99.2% | 100.0% | -0.09% | -0.23% | 0.15% | 99.9% | 100.3% | -0.19% | 0.56% | -0.21% | 100.7% | 100.9% |
| Class F | -0.02% | -0.02% | 0.07% | 99.7% | 102.1% | -0.09% | -0.01% | -0.15% | 101.0% | 100.9% |  |  |  |  |  |

Conclusions

In this contribution, a variance-based classification for in-loop filtering is proposed. The proposed methods can achieve promising coding performance with negligible encoding and decoding time increase. It is recommended to include the proposed method in the next round of EE2.

Reference

- ECM-9.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-9.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-Y2017, by teleconference, January 2022.

Patent Rights Declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).