| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0132-v1 |
| --- | --- |

| Title: | EE2-1.1: Partitioning prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Patrice Onno | Tel: Email: | 33 299876800 guillaume.laroche@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution presents the results of EE2 Tests 1.1 on a temporal partitioning prediction. In this temporal prediction method, initially presented in the contribution JVET-AD0130, the partitioning splits allowance and partitioning depths are derived for each block according to the partitioning parameters of the current frame and parameters obtained from a temporal area.

Three EE2 tests are presented. Test 1.1a corresponds to a first runtime level where the encoding runtime is in average lower than the Anchor for all classes. Test 1.1b corresponds to a second run time level where only classes A and B of RA configuration are changed compared to Test 1.1a. And Test 1.1c corresponds to an encoder only modification of Test 1.1a.

Compared to ECM-9.0, the average BDR gains and runtimes reported in this contribution are as follows:

EE2 Test 1.1a:

-0.08%	-0.07%	-0.07%  -   97.6%	102.5% for the RA configuration,

0.02%	0.05%	-0.02%  -   97.5%	101.2% for the Low Delay B configuration.

EE2 Test 1.1b:

-0.13%	-0.16%	-0.19%  -   99.7%	100.8% for the RA configuration,

EE2 Test 1.1c:

0.03%	0.07%	0.13%	96.6%	100.0% for the RA configuration,

XXX % XXX %	- XXX %  -  XXX%, XXX% for the Low Delay B configuration.

# Proposed method

For each block, the allowances of splits partitioning are predicted according to the minimum QT split and the average QT split obtained from a temporal area. So, when the current QT depth is strictly inferior to the temporal minimum QT depth minus 1, only the QT split is allowed and when the current QT depth is strictly inferior to the temporal average QT depth minus 1, No split, QT Split and TT split are allowed and BT split is allowed if TT is selected in a parent node.

In the same way, the maximum multi-tree depth is predicted for each block. If the temporal maximum multi-tree depth is strictly inferior to the maximum multi-tree depth of the current block, it is decreased except if the current QP is strictly inferior to the QP of the reference frame. If the temporal maximum multi-tree depth is superior to the maximum multi-tree depth of the current block and if the current depth is equal to the current QT depth, the maximum multi-tree depth is incremented. When the maximum QT depth of the current frame is superior to the maximum multi-tree depth and when the temporal maximum multi-tree depth is equal to the maximum multi-tree depth of the current frame, and when the temporal QT depth is inferior to the QT depth of the current frame, the maximum multi-tree depth of the current block is decreased.

The modifications on the maximum multi-tree depth are enabled only if the palette mode is disabled.

The temporal area is a collocated area from the same reference frame used for the temporal motion predictor. The collocated area of the current block is equal to the CTU size. The collocated area is centered according to the center of the current block. Inside this collocated area, each block position on 16x16 grid is considered to determine the 3 variables used in the proposed method:

-The minimum temporal QT Depth

-The temporal QT Depth

-The temporal maximum multi-tree depth

In addition, the center position of each block is aligned to a 16x16 grid in order to reduce the memory needed to store these variables.

# Results

## BDR results

The proposed modifications are implemented on top of the ECM-9.0 [1] and tested according to the common test conditions (CTC) for ECM-9.0.

All presented tests results have been executed at the same time as the Anchor on our computer farm. Consequently, the reported encoder runtimes are reliable.

### Test 1.1a

The following table gives the results of the Test 1.1a as defined in EE2 description [2].

All classes, with this test have a decrease of encoding time for both RA and LDB configurations.

Table 1 Coding performances of EE2 Test 1.1a.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.14% | -0.09% | 99.7% | 103.7% |
| Class A2 | -0.13% | -0.10% | -0.13% | 98.7% | 103.4% |
| Class B | -0.09% | -0.07% | -0.03% | 97.8% | 101.8% |
| Class C | 0.00% | 0.00% | -0.06% | 94.9% | 102.0% |
| Class E |  |  |  |  |  |
| Overall | -0.08% | -0.07% | -0.07% | 97.6% | 102.5% |
| Class D | 0.02% | 0.03% | 0.05% | 95.4% | 99.8% |
| Class F | 0.01% | 0.00% | 0.00% | 96.3% | 100.1% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | 0.14% | 0.00% | 97.3% | 102.4% |
| Class C | -0.03% | -0.36% | -0.12% | 97.0% | 99.9% |
| Class E | 0.03% | 0.45% | 0.06% | 98.6% | 101.2% |
| Overall | 0.02% | 0.05% | -0.02% | 97.5% | 101.2% |
| Class D | 0.01% | 0.19% | 0.75% | 98.8% | 102.2% |
| Class F |  |  |  |  |  |

### Test 1.1b

The following table gives the results of the Test 1.1b. In this test, compared to Test 1.1a, the incrementation of the maximum multi-tree depth for the current is enabled for all frames compared to Test 1.1a where the incrementation is possible only if the maximum multi-tree depth of current frame is inferior to the maximum multi-tree depth of the temporal frame.

Table 2 Coding performances of EE2 Test 1.1b.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.25% | -0.34% | -0.41% | 105.8% | 100.1% |
| Class A2 | -0.18% | -0.22% | -0.27% | 101.2% | 99.8% |
| Class B | -0.13% | -0.15% | -0.12% | 99.3% | 100.9% |
| Class C | 0.00% | 0.00% | -0.06% | 94.9% | 102.0% |
| Class E |  |  |  |  |  |
| Overall | -0.13% | -0.16% | -0.19% | 99.7% | 100.8% |
| Class D | 0.02% | 0.03% | 0.05% | 95.4% | 99.8% |
| Class F | 0.01% | 0.00% | 0.00% | 96.3% | 100.1% |

For this test, only class A and Class B of the RA configuration gives different results compared to Test 1.1a as for all other classes of RA configuration and all classes of LDB configuration, the maximum multi-tree depth is constant.

For this test, the encoding runtime is higher than the anchor for class A only. But the average encoding time reduction stays inferior to the ECM-9.0. The BDR gain is also higher with -0.13%.

### Test 1.1c

The following table gives the results of the Test 1.1c. This test is an encoder only method compared to test 1.1a. As we can see, this test gives similar encoder runtimes compared to Test 1.1a. but this Test 1.1c gives BDR losses compared to Test 1.1a.

Table 2 Coding performances of EE2 Test 1.1c.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.00% | 0.16% | 98.2% | 100.0% |
| Class A2 | -0.03% | 0.10% | 0.04% | 98.0% | 99.5% |
| Class B | 0.04% | 0.12% | 0.13% | 96.3% | 100.2% |
| Class C | 0.04% | 0.03% | 0.17% | 94.7% | 100.2% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.07% | 0.13% | 96.6% | 100.0% |
| Class D | 0.04% | 0.03% | -0.05% | 96.0% | 96.7% |
| Class F | 0.06% | 0.02% | 0.06% | 96.1% | 99.8% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | 0.10% | -0.04% | -0.15% | 97.9% | 101.2% |
| Class E | 0.12% | 0.33% | 0.38% | 98.4% | 99.9% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.04% | 0.96% | 0.13% | 98.2% | 101.1% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

## Memory requirement

In the proposed method, some variables of the temporal area need to be stored. This increases consequently the data needed to be stored.

As mentioned in the method description, there is 3 variables to be stored:

-The minimum temporal QT Depth (3 bits)

-The temporal QT Depth (3 bits)

-The temporal maximum multi-tree depth (4 bits)

So, for each block center of the 16x16 grid of a reference frame, 10 (3+3+4) bits need to be stored.

The following table gives the number of centers which need to be stored for a reference frame and the related additional memory in KB.

Table 2 Memory needed for each class.

|  | Number of centers | Memory needed in KB |
| --- | --- | --- |
| Class A 3840x2160 | 32400 | 40.5 |
| Class B 1920x1080 | 8100 | 10.1 |
| Class C 832x480 | 1560 | 1.9 |
| Class D 416x240 | 390 | 0.5 |

# Conclusion

In this contribution, the results of EE2 Tests 1.1a/b/c on top of ECM-9.0 are reported. These tests are related to modifications of the QT BT TT partitioning as presented in [3]. The method predicts some allowances of splits and depths according to the parameters obtained from a temporal area in a reference frame.

The average BD-rate results and encoding runtimes are similar as those presented in the initial contribution.

We suggest adopting this proposal in the next version of ECM.

# References

- ECM-9.0 reference software: https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-9.0

- Vadim Seregin, Jie Chen, Roman Chernyak, Karam Naser, Jacob Ström, Fan Wang, Martin Winken, Xiaoyu Xiu, Kai Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-AD2024, April 2023.

- Guillaume Laroche, Patrice Onno, "AhG12: On ECM partitioning prediction", JVET-AD0130, April 2023.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).