| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0139-v1 |
| --- | --- |

| Title: | EE2-5.2: Improved fixed filters for ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz, Nan Hu, Hongtao Wang, Vadim Seregin 5775 Morehouse Drive, San Diego, CA 92121, USA | Tel: Email: | martak@qti.qualcomm.com nanh@qti.qualcomm.com hongtaow@qti.qualcomm.com vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In ECM-9.0, when applying adaptive loop filter (ALF) to a sample, two fixed filters with the corresponding Laplacian-based classifiers are firstly applied to the input of ALF. The outputs of the fixed filters are then used to derive ALF output. In this contribution, fixed filters are firstly extended to be applied to the samples before deblocking filters. Secondly, classifiers of the fixed filters are extended by using the differences between the mean value and the sample values. At last, the outputs of the first fixed filter are used as the inputs to the second fixed filter. On top of ECM-9.0, simulation results of the proposed methods are reported as follows. (Anchor results of sequence BQTerrace with QP 22 under low delay B are used for tests 5.2a and 5.2b)

|  | All intra |  |  | Random access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Y | EncT | DecT | BD-rate Y | EncT | DecT | BD-rate Y | EncT | DecT |
| Test 5.2a | -0.04% | 100% | 101% | -0.07% | 100% | 100% | -0.05% | 101% | 100% |
| Test 5.2b | -0.19% | 100% | 100% | -0.27% | 100% | 100% | -0.26% | 100% | 99% |
| Test 5.2c | -0.30% | 100% | 99% | -0.44% | 100% | 99% | -0.40% | 100% | 99% |

# Introduction

In ECM-9.0, two Laplacian-based classifiers are applied to a 2x2 block. In each classifier, activity and directionality values are derived based on vertical, horizontal, and diagonal gradients using a window surrounding each 2x2 block. Then a class index is determined based on the activity and directionality values. Two 13x13 diamond shaped fixed filters are selected from the two filter sets by using the derived two class indices. Then the two fixed filters (fi with i=0, 1) are applied to the ALF input samples. Finally, a signalled filter is applied to the ALF input samples, samples before the deblocking filter (DBF), outputs of the two fixed filters, output of a gaussian filter and the residual data.

# Proposed method

In this proposal, in test 5.2a, both fixed filters are applied to samples before DBF and ALF input, where additional diamond 9x9 filter is used for the samples before DBF. The shape of the first fixed filter applied to the ALF input samples is reduced from 13x13 to 9x9, and the shape of the second fixed filter, which is 13x13, applied to ALF input is unchanged as shown in the below table.

Table 1. Filter shapes in the proposed methods

|  | ECM-9.0 | Proposed method |  |
| --- | --- | --- | --- |
|  | ALF input | Samples before DBF | ALF input |
| Fixed filer f0 | 13x13 | 9x9 | 9x9 |
| Fixed filer f1 | 13x13 | 9x9 | 13x13 |

On top of test 5.2a, in test 5.2b, the classifiers of the fixed filters are extended. For each 2x2 block, the mean value of a surrounding window is calculated. Then, for each sample of this window, the difference between the sample value and the mean value is calculated. A scaling factor is determined based on the activity value derived from a Laplacian classifier. The square root of the sum of the squared differences is further quantized to C' by a scaling factor. The value of C' is an integer between 0 and 7, inclusively. With i=0, 1, let Ci denote the classifier from the classifier of i-th fixed filter in ECM-9.0. Then the proposed class index Ci' is derived as

Ci'= C'*896+Ci.

In this proposal, the total number of the fixed filters is not changed.

On top of test 5.2b, in test 5.2c, fixed filter f1 is applied to outputs of f0 (instead of ALF input) and samples before DBF.

# Simulation results

The proposed method was implemented on top of the ECM-9.0 and tested following common test conditions described in JVET-AD2017. Anchor results of sequence BQTerrace with QP 22 under low delay B are used for tests 5.2a and 5.2b

Table 2. Results of test 5.2a: applying fixed filters to samples before DBF.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | -0.01% | 100% | 100% |
| Class A2 | 0.01% | 0.00% | 0.00% | 100% | 101% |
| Class B | -0.02% | 0.00% | 0.00% | 100% | 101% |
| Class C | -0.08% | -0.01% | -0.01% | 100% | 98% |
| Class E | -0.08% | -0.03% | -0.03% | 99% | 102% |
| Overall | -0.04% | -0.01% | -0.01% | 100% | 101% |
| Class D | -0.23% | -0.03% | -0.03% | 98% | 98% |
| Class F | -0.03% | 0.00% | 0.00% | 100% | 102% |
|  | Random access Main 10 |  |  |  |  |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.10% | -0.05% | 101% | 99% |
| Class A2 | -0.05% | -0.03% | -0.07% | 101% | 99% |
| Class B | -0.08% | -0.06% | 0.08% | 100% | 100% |
| Class C | -0.08% | -0.22% | 0.01% | 99% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.07% | -0.10% | 0.00% | 100% | 100% |
| Class D | -0.22% | -0.09% | -0.15% | 100% | 101% |
| Class F | -0.09% | -0.12% | -0.07% | 103% | 99% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.34% | -0.19% | 101% | 100% |
| Class C | -0.04% | -0.30% | -0.25% | 100% | 100% |
| Class E | -0.14% | 0.09% | -0.06% | 101% | 99% |
| Overall | -0.05% | -0.22% | -0.18% | 101% | 100% |
| Class D | -0.21% | 0.29% | 0.61% | 98% | 99% |
| Class F | -0.33% | -0.26% | -0.01% | 101% | 99% |

Table 3. Results of test 5.2b: extended classifier for fixed filters and applying fixed filters to samples before DBF.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.03% | -0.03% | 101% | 99% |
| Class A2 | -0.16% | -0.02% | -0.02% | 100% | 102% |
| Class B | -0.15% | -0.02% | -0.02% | 100% | 101% |
| Class C | -0.23% | -0.03% | -0.03% | 99% | 98% |
| Class E | -0.28% | -0.09% | -0.09% | 99% | 100% |
| Overall | -0.19% | -0.03% | -0.04% | 100% | 100% |
| Class D | -0.29% | -0.04% | -0.04% | 98% | 99% |
| Class F | -0.14% | -0.01% | -0.01% | 100% | 100% |
|  | Random access Main 10 |  |  |  |  |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | -0.16% | -0.04% | 100% | 99% |
| Class A2 | -0.29% | -0.15% | -0.08% | 100% | 99% |
| Class B | -0.22% | -0.11% | -0.06% | 100% | 100% |
| Class C | -0.35% | -0.07% | -0.04% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.27% | -0.12% | -0.06% | 100% | 100% |
| Class D | -0.43% | -0.13% | -0.19% | 100% | 100% |
| Class F | -0.25% | -0.09% | -0.12% | 102% | 99% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.20% | -0.18% | -0.13% | 101% | 100% |
| Class C | -0.34% | 0.00% | -0.24% | 99% | 98% |
| Class E | -0.27% | 0.27% | -0.62% | 101% | 98% |
| Overall | -0.26% | -0.01% | -0.29% | 100% | 99% |
| Class D | -0.51% | 0.03% | -0.15% | 99% | 96% |
| Class F | -0.32% | -0.42% | -0.44% | 101% | 101% |

Table 4. Results of test 5.2c: extended classifier for fixed filters, applying fixed filters to samples before DBF and applying f1 to outputs of f0.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.28% | -0.03% | -0.03% | 101% | 99% |
| Class A2 | -0.25% | -0.02% | -0.02% | 100% | 100% |
| Class B | -0.22% | -0.02% | -0.02% | 100% | 100% |
| Class C | -0.41% | -0.03% | -0.03% | 100% | 98% |
| Class E | -0.37% | -0.09% | -0.09% | 100% | 98% |
| Overall | -0.30% | -0.04% | -0.04% | 100% | 99% |
| Class D | -0.37% | -0.10% | -0.09% | 100% | 97% |
| Class F | -0.26% | -0.02% | -0.02% | 101% | 99% |
|  | Random access Main 10 |  |  |  |  |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.42% | -0.14% | -0.08% | 100% | 98% |
| Class A2 | -0.34% | -0.13% | -0.05% | 101% | 98% |
| Class B | -0.38% | -0.01% | -0.07% | 100% | 98% |
| Class C | -0.60% | -0.10% | -0.08% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.44% | -0.08% | -0.07% | 100% | 99% |
| Class D | -0.65% | -0.14% | -0.09% | 101% | 99% |
| Class F | -0.32% | 0.04% | -0.03% | 100% | 99% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.27% | -0.23% | -0.23% | 100% | 99% |
| Class C | -0.52% | -0.51% | -0.15% | 101% | 99% |
| Class E | -0.46% | 0.06% | 0.30% | 100% | 98% |
| Overall | -0.40% | -0.25% | -0.07% | 100% | 99% |
| Class D | -0.72% | 0.25% | -0.22% | 101% | 97% |
| Class F | -0.21% | -0.09% | -0.53% | 100% | 101% |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).