| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0143-v3 |
| --- | --- |

| Title: | AHG 8: A spatial resampling algorithm and an exemplar software implementation |  |  |
| --- | --- | --- | --- |
| Status: | Input document of JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shurun Wang Binzhe Li Jie Chen Yan Ye (Alibaba) Shiqi Wang (City University of Hong Kong) | Email: | shurun.wsr@alibaba-inc.com binzheli2-c@my.cityu.edu.hk jiechen.cj@alibaba-inc.com yan.ye@alibaba-inc.com shiqwang@cityu.edu.hk |
| Source: | Alibaba, City University of Hong Kong |  |  |

_____________________________

- Abstract

Sections on using spatial resampling to improve coding efficiency for video coding for machines have been included in JVET-AD2030 “Optimization of encoders and receiving systems for machine analysis of coded video content (draft 2)” at the conceptual level, but without providing any detail. This contribution proposes a spatial resampling algorithm and a spatial resampling software implementation for inclusion in JVET-AD2030. Following the common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content as defined in JVET-AD2031, the proposed method is only activated for larger resolution sequences, where the gains can reach as high as 59.10%.

Spatial resampling algorithm

Framework

Fig. 1 shows a typical spatial resampling-based compression framework in the JVET-AD2030 draft technical report [1]. The input visual data I is rescaled with a spatial downsampling model at the encoder side and the output, downsampled data ID, is compressed with the VVC encoder. At the decoder side, the decompressed downsampled data ID' is decoded and reconstructed with the VVC decoder. The reconstructed visual data I' is the output of spatial upsampling, based on the resampling ratio transmitted as metadata along with the VVC bitstream.

Fig. 1 Typical spatial resampling-based compression framework.

Structure of the proposed resampling model

The proposed spatial downsampling network targets to eliminate the spatial redundancy of the input video, improving the representation compactness of the compression system at the encoder side. As shown in Fig. 2 (a), the spatial downsampling network consists of three convolutional layers with the Relu activation function for each layer.

Fig. 2 The network structure of the spatial resampling model, including the spatial downsampling network at the encoder side (a), the detailed structure of Bottleneck Resblock (BRes) (b), and the lightweight spatial upsampling model at the decoder side (c). Specifically, the input channel cin, the output channel number cout, the kernel size k and the stride s of every convolutional layer are denoted as k-s-cin-cout.

The proposed method has been studied under the core experiment for spatial resampling (CE3) in video coding for machines (VCM) of WG4. Within the context of JVET AHG8, downsampling and upsampling are studied strictly as non-normative operations, i.e. downsampling is performed as pre-processing and upsampling as post-processing. This means the upsampling network needs to be sent to the decoder, and for this contribution, we propose that the NNPF SEI can achieve this goal. In NNPF SEI, setting syntax element nnpfc_purpose as 0x04 and setting syntax element nnpfc_mode_idc as 0 can be used to indicate the resolution resampling purpose and then the upsampling neural network can be sent using Neural Network Coding / ISO/IEC 15938-17 or MPEG-7 part 17 (NNC).The value of other syntax elements can be set as in Table 1.

Table 1 An example setting of the syntax element values of NNPF for proposed spatial resampling.

| Syntax | Value | Syntax | Value |
| --- | --- | --- | --- |
| nnpfc_purpose | 0x04 | nnpfc_out_format_idc | 1 |
| nnpfc_mode_idc | 0 | nnpfc_out_order_idc | 2 |
| nnpfc_num_input_pics_minus1 | 0 | nnpfc_out_tensor_luma_bitdepth_minus8 | 2 |
| nnpfc_pic_width_num_minus1 | 1 | nnpfc_out_tensor_chroma_bitdepth_minus8 | 2 |
| nnpfc_pic_width_denom_minus1 | 0 | nnpfc_chroma_loc_info_present_flag | 0 |
| nnpfc_pic_height_num_minus1 | 1 | nnpfc_overlap | 0 |
| nnpfc_pic_height_denom_minus1 | 0 | nnpfc_constant_patch_size_flag | 1 |
| nnpfc_component_last_flag | 0 | nnpfc_patch_width_minus1 | CroppedWidth − 1 |
| nnpfc_inp_format_idc | 1 | nnpfc_patch_height_minus1 | CroppedHeight − 1 |
| nnpfc_auxiliary_inp_idc | 0 | nnpfc_padding_type | 0 |
| nnpfc_inp_order_idc | 2 | nnpfc_complexity_info_present_flag | 0 |
| nnpfc_inp_tensor_luma_bitdepth_minus8 | 2 | nnpfc_metadata_extension_num_bits | 0 |
| nnpfc_inp_tensor_chroma_bitdepth_minus8 | 2 |  |  |

Sending the spatial upsampling model results in extra bandwidth expense. To reduce the transmission expense of the spatial upsampling model, compared to CE3 in VCM of WG4, a lightweight spatial upsampling is proposed, which reduces the number of convolutional layers and is based on Bottleneck Resblock (BRes), as shown in Fig. 2 (b). Specifically, the first and third convolutional layers are responsible for data dimension reduction and recovery respectively with 1×1 kernel, improving the computational efficiency. The second convolutional layer is responsible for feature extraction with 3×3 kernel. The outputs of the first and second layers are activated with Relu. The detailed spatial upsampling network structure is shown in Fig. 2 (c).

Encoder decision

Motivated by the diversity of video content, the proposed spatial resampling is adaptively performed based on the resolution of input video and spatial complexity with pre-analysis. Specifically, if the resolution of input video is larger than 2K (2560×1600), spatial resampling is performed, since the spatial redundancy for high resolution video content is much more abundant. If the resolution of input video is smaller than 1920×1080, spatial resampling is not performed. For 1920×1080input video, spatial resampling is adaptively performed based on the spatial complexity with pre-analysis. Object detection is performed on the input video to extract the area of objects. The larger the object area, the better performance improvement can be achieved with spatial resampling.

Exampler software implementation

The exampler spatial resampling software implementation will be attached to this contribution.

The source code directory of the spatial resampling software for OFM is structured as follows:

- README.txt: contains introduction for the spatial resampling part of OFM

- Spatial_resampling: contains source code for the spatial resampling framework of OFM

  - Scripts: contains scripts to encode and decode SFU and TVD video datasets

  - Spatial_resampling: contains the implementation of the spatial resampling algorithm

  - encoder_main.py: source code to encode the input video with spatial downsampling and video encoder

  - decoder_main.py: source code to decode the bitstream with video decoder and spatial upsampling, for the reconstructed output video

For the installation and usage of the exampler software implementation, please refer to the README.txt for detailed information.

Performance

We evaluate the performance of the proposed spatial resampling network following the common test conditions specified in JVET-AD2031 [2].

The rate-accuracy curve performance of the five selected sequences are shown in the figures below.

For AI configuration:

For RA configuration:

The performance on TVD-image dataset is shown as below, for both object detection and instance segmentation:

Table 2 The object detection performance of the proposed technology on SFU-HW dataset.

|  | RA |  | AI |  |
| --- | --- | --- | --- | --- |
| Sequence Name | Pareto BD-rate | BD-mAP | Pareto BD-rate | BD-mAP |
| Traffic_2560x1600_30 | -6.22% | -1.08 | -22.77% | 3.94 |
| ParkScene_1920x1080_24 | -1.47% | 0.25 | -0.25% | 0.24 |
| Cactus_1920x1080_50 | -59.10% | 15.98 | -50.51% | 28.69 |
| BasketballDrive_1920x1080_50 | 13.07% | -2.07 | -1.00% | 0.23 |
| BQTerrace_1920x1080_60 | 12.66% | -1.89 | -13.87% | 2.59 |
| AVG on Class A and B | -8.21% | 2.24 | -17.68% | 7.14 |

It is worth mentioning that the transmission expense of the spatial upsampling model is not included in the performance evaluation. The lightweight spatial upsampling model is 177540 bytes before compression and 42495 bytes after NNC standard is used to compress it losslessly. This is a fixed, one-time overhead for the entire duration of the coded video sequence.

Conclusion

A lightweight spatial resampling algorithm is described in this contribution to improve the coding efficiency for video coding for machines. Simulation shows that spatial resampling is effective and performs superior to VVC under various coding configurations. Specifically, for a large sequence, this proposed spatial resampling algorithm could achieve up to 59.10% Pareto BD-rate savings under RA configurations on SFU-HW dataset for object detection task. It is suggested to include information about the spatial resampling algorithm and the software implementation into the next version of the technical report for optimization of encoders and receiving systems for machine analysis of coded video content (JVET-AE2030).

References

- ISO/IEC JTC 1/SC 29/WG 5, “Optimization of encoders and receiving systems for machine analysis of coded video content (draft 2),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 output document JVET-AD2030, 30th JVET meeting, Antalya, April 2023 (available online at https://jvet-experts.org/).

- ISO/IEC JTC 1/SC 29/WG 5, “Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 output document JVET-AD2031, 30th JVET meeting, Antalya, April 2023 (available online at https://jvet-experts.org/).

Patent rights declaration(s)

Alibaba group and City University of Hong Kong may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

___________________