| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0153-v3 |
| --- | --- |

| Title: | Non-EE2: Enabling template-based inter tools for scaled reference pictures in the RPR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Hong-Jheng Jhu, C.-W. Kuo, Changyue Ma, Ning Yan, Wei Chen, Xianglin Wang | Email: | xiaoyuxiu@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to improve the coding efficiency of the RPR in ECM-9.0 by enabling the template-based inter reordering tools (including ARMC, MMVD, affine MMVD, template-based BCW derivation, reference picture list reordering and MVD sign and magnitude-suffix prediction) and the LIC for scaled reference pictures. Additionally, the contribution identifies one encoder crash issue of ECM-9.0 in RA configuration when the RPR is enabled and provides the fix. The proposed changes were implemented on top of ECM-9.0 with MR437. The simulation results are summarized as follows:

- RPR-1.5x

- LB PSNR1: -0.89% (Y), -1.04% (U), -0.67% (V), 104.3% (Enc), 104.1% (Dec)

- LB PSNR2: -0.86% (Y), -1.07% (U), -0.68% (V), 104.3% (Enc), 104.1% (Dec)

- RPR-2.0x

- LB PSNR1: -0.77% (Y), -0.67% (U), -0.54% (V), 104.4% (Enc), 103.8% (Dec)

- LB PSNR2: -0.65% (Y), -0.72% (U), -0.59% (V), 104.4% (Enc), 103.8% (Dec)

# Introduction

In ECM-9.0 [1], several template-based reordering schemes, including ARMC, MMVD, affine MMVD, template-based BCW derivation, reference list reordering and MVD sign and magnitude-suffix prediction, are applied to reduce the signaling overhead of inter modes. The basic idea of the schemes is to sort the possible combinations of related syntax elements (e.g., merge/MMVD index, BCW index, refIdx and MVD sign and magnitude-suffix) according to template cost and signaling the syntax elements with appropriate context models. Additionally, when the RPR is enabled, the template-based reordering schemes are disabled for all scaled reference pictures. Specifically, if the resolution of one reference picture is different from that of the current picture, the template prediction process is bypassed with the corresponding cost being set to maximum.

At the same time, in ECM-9.0, the LIC is also always skipped at motion compensation stage if any of the reference pictures is in different resolution to the current picture. In such case, the LIC flag is not signaled at CU level and inferred to be 0.

# Proposal

To further enhance the RPR performance, this contribution proposes to enable the template-based inter reordering tools (including ARMC, MMVD, affine MMVD, template-based BCW derivation, reference picture list reordering and MVD sign and magnitude-suffix prediction) and the LIC for scaled reference pictures. Specifically, the main aspects of all the related tools are kept exactly the same as the existing designs in the proposal, except that the motion-compensation filters are replaced by the RPR filters to generate the template prediction samples during the processes.

Additionally, one encoder crash issue is identified for ECM-9.0 RA configuration when the RPR is enabled. The crash is caused by the encoder assertion due to the unfilled AMVP candidate lists at certain corner cases of the SMVD mode when one reference picture’s resolution is different to the current picture. The appropriate fix is provided in the contribution which is also submitted as MR447 for the ECM master branch.

# Simulation results

The proposed changes were implemented on top of ECM-9.0 [1] and tested using the RPR test conditions for the LDB configuration as described in [2] and the RA configuration as described in [3]. Specifically, the two RPR configurations are set as follows.

| RPR-1.5x |  | RPR-2.0x |  |
| --- | --- | --- | --- |
| RA | LDB | RA | LDB |
| ScalingRatioHor: 1.5 | ScalingRatioHor: 1.5 | ScalingRatioHor: 2.0 | ScalingRatioHor: 2.0 |
| ScalingRatioVer: 1.5 | ScalingRatioVer: 1.5 | ScalingRatioVer: 2.0 | ScalingRatioVer: 2.0 |
| FractionNumFrames: 1.0 | FractionNumFrames: 1.0 | FractionNumFrames: 1.0 | FractionNumFrames: 1.0 |
| RPR: 1 | RPR: 1 | RPR: 1 | RPR: 1 |
| UpscaledOutput: 1 | UpscaledOutput: 1 | UpscaledOutput: 1 | UpscaledOutput: 1 |
| SwitchPocPeriod: 16 | SwitchPocPeriod: 0 | SwitchPocPeriod: 16 | SwitchPocPeriod: 0 |

## RPR-1.5x:

|  | Low-delay B PSNR1 |  |  |  |  | Low-delay B PSNR2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class B | -0.84% | -0.74% | -0.66% | 103.5% | 102.7% | -0.83% | -0.77% | -0.66% | 103.5% | 102.8% |
| Class C | -0.99% | -1.12% | -0.37% | 105.3% | 105.5% | -0.91% | -1.13% | -0.37% | 105.3% | 105.5% |
| Class E | -0.86% | -1.42% | -1.09% | 104.5% | 104.5% | -0.84% | -1.50% | -1.11% | 104.5% | 104.5% |
| Overall | -0.89% | -1.04% | -0.67% | 104.3% | 104.1% | -0.86% | -1.07% | -0.68% | 104.3% | 104.1% |
| Class D | -0.64% | -0.41% | -0.78% | 103.8% | 101.2% | -0.57% | -0.39% | -0.86% | 103.8% | 101.2% |
| Class F | -1.26% | -1.32% | -0.92% | 103.7% | 104.2% | -1.06% | -1.13% | -0.77% | 103.7% | 104.2% |

## RPR-2.0x:

|  | Low-delay B PSNR1 |  |  |  |  | Low-delay B PSNR2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class B | -0.78% | -0.70% | -0.62% | 105.0% | 102.9% | -0.68% | -0.69% | -0.67% | 105.0% | 102.9% |
| Class C | -0.83% | -0.83% | -0.35% | 104.7% | 104.2% | -0.68% | -0.88% | -0.40% | 104.7% | 104.2% |
| Class E | -0.69% | -0.40% | -0.68% | 104.5% | 104.8% | -0.59% | -0.55% | -0.73% | 104.5% | 104.8% |
| Overall | -0.77% | -0.67% | -0.54% | 104.4% | 103.8% | -0.65% | -0.72% | -0.59% | 104.4% | 103.8% |
| Class D | -0.58% | -0.63% | -0.41% | 103.9% | 101.3% | -0.33% | -0.84% | -0.60% | 103.9% | 101.3% |
| Class F | -1.30% | -0.79% | -0.68% | 104.6% | 104.8% | -0.87% | -0.91% | -0.52% | 104.6% | 104.8% |

# In the below, complementary results of the proposed changes are provided for class C and D in random-access configuration.

|  | RA |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RPR-1.5x |  |  |  |  | RPR-2.0x |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class C | -1.15% | -1.06% | -0.95% | 103.9% | 104.0% | -1.10% | -1.22% | -0.91% | 103.4% | 104.0% |
| Class D | -1.02% | -1.24% | -0.90% | 104.0% | 103.9% | -0.99% | -0.85% | -0.76% | 102.7% | 104.1% |

# Reference

- ECM-9.0, https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM.

- V. Seregin, J. Chen, L. Li, K. Naser, J. Ström, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)”, JVET-AE2024, April 2023.

- R. Yu, K. Andersson, “AHG12: RPR luma filter modifications and RPR enabling fixes in ECM”, JVET-Z0062, April 2022.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).