| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0155-v1 |
| --- | --- |

| Title: | Editor commentary on the post-filter hint SEI message semantics |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information / errata report |  |  |
| Author(s) or Contact(s): | Gary J. Sullivan Dolby Laboratories United States Ye-Kui Wang Bytedance United States | Email: Email: | gary.sullivan@dolby.com yekui.wang@bytedance.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

The authors have provided this contribution to address issues encountered in the specification of the post-filter hint SEI message during the finalization editing of WG 5 N 179 (FDIS ISO/IEC 23008-2 5th edition) and JVET-AD2006 (Additional SEI messages for VSEI (Draft 4)) after consideration of item 4 of JVET-AD0078 (AHG9: On comments for the draft of VSEI [T. Chujoh, Y. Yasugi, T. Ikai (Sharp)]).

# Observed issues

We report the following observations:

- The semantics do not say that the described post-filtering should be applied.

- It does not seem clear whether the “16-bit precision” description applies only to the cross-correlation matrix case or also to the filter coefficient cases.

- A specified 32-bit range may be excessive for a syntax element that has 16-bit precision.

- Much of the text is written from the encoder perspective. We generally do not specify things from the encoder perspective. The decoder-side processing is not clearly described or is discussed only vaguely. For example, no equations are given for the decoder’s filtering process.

- It does not seem clear whether the decoder should divide the filter tap values (or equivalently, the result of the filtering process) by the sum of the filter tap values or if the encoder is responsible for the filter normalization.

- There are issues relating to how to interpret the filter depending on whether the filter length is even (in which case the output samples may presumably be located spatially between input samples) or odd (in which case the location of an output sample may presumably have a location corresponding to the centre tap).

- The handling of the boundaries of the picture and cropping window are not described and seems unclear.

- It seems unclear why should it matter whether a horizontal filter or a vertical filter is applied first if both are to be applied. Perhaps integer rounding is expected to be applied between the two filtering operations, but this is not stated.

- The discussion of Wiener post-filtering is a bit vague (e.g., what exactly is a Wiener filter, and how would a decoder derive one from the provided data?).

- A decoder could use a subset of a provided cross-correlation matrix to derive a Wiener post-filter with a smaller region of support than the provided statistics would enable. It does not seem clear whether this is intended to be an acceptable or possible practice.

Detailed editors’ notes are provided within the text to highlight and discuss the issues.

# Text with editors’ notes

    - D.3.14	Post-filter hint SEI message semantics

This SEI message provides the coefficients of a post-filter or correlation information for the design of a post-filter for potential use in post-processing of the current picture after it is decoded, cropped, and output, to obtain improved displayed quality. [Ed. Notice that this does not say that such a post-processing operation should be applied. Should it say that?]

filter_hint_size_y specifies the vertical size of the filter coefficient or correlation array. The value of filter_hint_size_y shall be in the range of 1 to 15, inclusive.

filter_hint_size_x specifies the horizontal size of the filter coefficient or correlation array. The value of filter_hint_size_x shall be in the range of 1 to 15, inclusive.

filter_hint_type identifies the type of the filter hints as specified in Table D.6. The value of filter_hint_type shall be in the range of 0 to 2, inclusive. The value of filter_hint_type equal to 3 is reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this document. Decoders shall ignore post-filter hint SEI messages having filter_hint_type equal to 3.

Table D.6 — filter_hint_type values

| Value | Description |
| --- | --- |
| 0 | Coefficients of a 2D-FIR filter |
| 1 | Coefficients of two 1D-FIR filters |
| 2 | Cross-correlation matrix |

filter_hint_value[ cIdx ][ cx ][ cy ] specifies a filter coefficient or an element of a cross-correlation matrix between the original and the output signal with 16-bit precision. [Ed. Does the “16-bit precision” description apply only to the cross-correlation matrix case or also to the filter coefficient cases?] The value of filter_hint_value[ cIdx ][ cx ][ cy ] shall be in the range of −231 + 1 to 231 − 1, inclusive. [Ed. (GJS): Why does this have a 32-bit range if it has “16-bit precision”? (YK): Per the current value range, it seems that it should be 32-bit precision rather than 16-bit precision. However, even assuming the 32-bit precision, according to the current value range of −231 + 1 to 231 − 1, inclusive, the value range after normalization (i.e., division by 232) would be (−0.5, 0.5), not [−0.5, 0.5]. Is that the design intent? (GJS): If “normalization” refers to division by 216, then the allowed value after the division operation would have a range of [−215, 215), so the 32-bit range is way more than the one extra bit necessary to include the upper bound.] cIdx specifies the related colour component, cx represents an index in the horizontal direction, and cy represents an index in the vertical direction.

When filter_hint_type is equal to 1, filter_hint_size_y shall be equal to 2.

Depending on the value of filter_hint_type, the following applies:

—	If filter_hint_type is equal to 0, the coefficients of a two-dimensional FIR filter with the size of filter_hint_size_x * filter_hint_size_y are provided. [Ed. Does the decoder normalize the filter tap values in this case – i.e., should the decoder divide by the sum of the filter tap values? Does the 16-bit precision description apply to this case also? Is it implied that if the width and height of the filter are odd, the middle tap corresponds to the position of an input sample? If the width or height of the filter is even, are the output positions shifted in their location relative to the input positions? Does it make sense for the width or height to be an even number? Overall, what is the decoder expected to do with the provided filter tap values? How is the decoder supposed to apply the filter, including at the boundaries of the output picture (e.g., clipping of the array indices to keep them within the boundaries of the cropped output picture)?]

—	Otherwise, if filter_hint_type is equal to 1, the filter coefficients of two one-dimensional FIR filters are provided. In this case, the index cy equal to 0 specifies the filter coefficients of the horizontal filter and cy equal to 1 specifies the filter coefficients of the vertical filter. In the indicated filtering process, the horizontal filter is applied first and the result is filtered by the vertical filter. [Ed. Again, there are similar issues of clarification as for the case with filter_hint_type equal to 1. Also, why should it matter whether the horizontal filter or the vertical filter is applied first? Is integer rounding expected to be applied between the two filtering operations?]

—	Otherwise (filter_hint_type is equal to 2), the hint values specify a cross-correlation matrix between the original signal s and the output signal prior to the post-filtering operation s′.

When filter-hint_type is equal to 2, the cross-correlation matrix values for a colour component identified by cIdx with the size of filter_hint_size_x * filter_hint_size_y should be derived by the encoder as follows: [Ed. This is from the encoder perspective. We generally do not specify things from the encoder perspective.]

filter_hint_value[ cIdx ][ cx ][ cy ]=Round( 14( bitDepth - 8 ) * w * h *
	n = 0h - 1m = 0w - 1s m [ n ] * s'[ Clip3( 0, w-1,m+cx-midX ) ][ Clip3( 0, h-1, n+cy-midY ) ] ) 

				(D-16)

where s denotes the array of samples of the colour component cIdx of the original picture, s′ denotes the corresponding array of the cropped output picture, h denotes the vertical height of the related colour component, w denotes the horizontal width of the related colour component, bitDepth denotes the bit depth of the colour component, midX is equal to ( filter_hint_size_x >> 1 ), midY is equal to ( filter_hint_size_y >> 1 ), for 0 <= cx < filter_hint_size_x and 0 <= cy < filter_hint_size_y.

NOTE	A decoder can derive a Wiener post-filter from the cross-correlation matrix between the original signal s and the output signal s′ and the auto-correlation matrix of the output signal s′ (which the decoder can compute from the output signal s′). [Ed. This is a bit vague, although true. Notice that a decoder could use a subset of the provided cross-correlation matrix to derive a Wiener post-filter with a smaller region of support.]