| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0156-v2 |
| --- | --- |

| Title: | [AHG9] Message wrapping and importance indication for the SEI processing order SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gary J. Sullivan Sean McCarthy Peng Yin Dolby Laboratories United States | Email: | gary.sullivan@dolby.com |
| Source: | Dolby Labs |  |  |

_____________________________

# Abstract

This contribution proposes four modifications of the draft SEI processing order SEI message for VVC, relative to draft 4 (JVET-AD2027).

- Enabling a “wrapping” capability, controlled by a wrapping flag, so that rather than having the SEI messages always carried in a way that they will be potentially interpreted by decoders that do not understand the SEI processing order SEI message, the encoder can place them where they will only be interpreted in the proper context.

- Adding an importance flag to indicate whether a stage of the sequential processing is needed for proper output of the indicated sequence of SEI message processing operations.

- Adding an explicit indication of the number of SEI messages that are in the indicated sequence, to avoid needing to infer this from the amount of data in the SEI message and to ensure that at least two messages are in the indicated sequence.

- Constraining the order indication values so that SEI messages are not sent in an order that differs from their natural sequential order (and reducing the size of these values to 8 bits).

- Removing the constraint that SEI processing order SEI messages cannot change in a CVS.

Finally, the contribution points out that the SEI processing order message can be useful not only for indicating stages of processing operations, but also for indicating the properties and intended uses of the video content after particular stages of processing (e.g. by sending CLL, ATC, CCV or AVE SEI messages). The consequence of this observation is that the SEI processing order SEI message could be useful for more purposes than may have been previously appreciated.

# Problem statement and proposed approach

## Background

JVET has been considering the development of an “SEI processing order (SPO) SEI message” that would indicate a preferred order of processing, where the output of one stage is indicated to be the input of another stage.

One problem with the current approach is that the encoder may not want legacy decoders to apply some SEI messages with the legacy interpretation – i.e., the interpretation that the input pictures for an SEI message are the cropped decoded output pictures of the decoding process and the output pictures are for display. The current draft provides no way to deal with this possibility.

We further note that SEI messages may not only be for indicating stages of post-processing, but can also be for indicating properties of the video content. It is possible that some indicated property may only apply at some particular point in the sequences processing stages. For example, a post-processing operation may affect the video content light level, so the indicated light level may only be valid before or after some particular post-processing operation is performed (e.g. a colour transform operation).

We also note that the number of messages indicated in the SPO SEI message needs to be computed from the quantity of data in the SEI message, which is not typically done and does not naturally enforce the presence of any particular minimum (or maximum) number of SEI messages. It also complicates the syntax expression.

## Wrapping

For these reasons, it is better to have a way to “hide” SEI message from legacy decoders, in a manner similar to the “nesting” approach applied for particular scalable layers. Herein we refer to this principle as “wrapping”, as it is proposed that an SEI message can be “wrapped” within an SPO SEI message. A wrapped SEI message will be carried within the SPO SEI message rather than being present outside of it in the bitstream, and thus will not be incorrectly interpreted with the legacy meaning.

## Importance indication

In some cases an SEI message may appear in an indicated order in an SEI processing order SEI message that the decoding system does not support – either because it is a new SEI message that was not yet defined when the decoder was implemented or simply because the designer of the decoding system chose not to support that SEI message (e.g., due to insufficient processing power or some other reason).

In some cases, an SEI message may be important for proper use of the video content, while in other cases, it may not be especially important.

It is suggested to associate each SEI message in the SPO SEI message loop with an importance flag, and to indicate that if the decoder does not support an SEI message that has the importance flag set to 1, the decoder should ignore the entire SPO SEI message and all of its indicated processing operations and properties.

## SEI message quantity indication

We propose adding an indication of the number of SEI messages that are in the indicated processing order sequence, to avoid needing to infer this from the amount of data in the SEI message and to ensure that at least two messages are in the indicated sequence.

## Order constraint

In the current draft for the SPO SEI message, there is no constraint on the relationship between the loop variable i and the indicated processing order po_sei_processing_order[ i ]. Thus the indicated processing order may differ arbitrarily from the order in which the SEI messages are listed in the SPO SEI message. We suggest that this could make the SPO SEI message confusing to interpret and could possibly lead to implementation problems due to the need to sort the list to establish ordered sequence being indicated. We thus propose a simple constraint: for i greater than 0, po_sei_processing_order[ i ] shall be equal to po_sei_processing_order[ i − 1 ] or po_sei_processing_order[ i − 1 ] + 1.

With this constraint, we suggest that one byte (rather than two) should be sufficient for coding the value of po_sei_processing_order[ i ].

## Static content constraint

The current draft says “When there are multiple SEI processing order SEI messages present in a CVS, they shall have the same content.”

This constraint can be a problem in two ways:

- This constraint would prohibit sending more than one SEI processing order SEI message with different content, for example as alternatives for delivering video to different display environments (e.g., for displays with different dynamic range capabilities or for viewing in different ambient viewing environments.

- When used with either the already drafted prefix approach or the proposed wrapping approach, this could prevent changing the content of relevant messages from changing from picture-to-picture in a way that would otherwise be reasonable, such as post-filter hint SEI messages, film grain characteristics SEI messages, tone mapping information SEI messages, and colour remapping SEI messages.

# Examples of usage

An example of the use of the proposed scheme (using the syntax elements described below in sections 3 and 4) is as follows:

- A post-filter hint (PFH) SEI message (to improve picture quality), intended as discretionary, with:

po_sei_wrapping_flag[ 0 ] = 0 (i.e., the SEI message is carried outside of the SPO SEI message)

po_sei_importance_flag[ 0 ] = 0 (to indicate that this process is optional for purposes of this SPO)

po_sei_processing_order[ 0 ] = 0 (this is the first stage in the SPO)

NOTE 1: A similar example could be a film grain characteristics (FGC) SEI message.

- A tone mapping information (TMI) SEI message (to adapt to some alternative display), necessary for appropriate interpretation of the following data, with:

po_sei_wrapping_flag[ 1 ] = 0 (i.e., the SEI message is carried outside of the SPO SEI message)

po_sei_importance_flag[ 1 ] = 1 (to indicate that this process is fundamental to the purpose of this SPO)

po_sei_processing_order[ 1 ] = 1 (this is the second stage in the SPO)

NOTE 2: A similar example could be a colour remapping information (CRI) SEI message.

- A content light level information (CLL) SEI message (to indicate the light level after applying the tone mapping information)

po_sei_wrapping_flag[ 2 ] = 1 (i.e., the SEI message is wrapped inside the SPO SEI message)

po_sei_importance_flag[ 2 ] = 0 (to indicate that this indicator is not necessary for interpretation of the output of the second stage)

po_sei_processing_order[ 2 ] = 1 (this is the third stage in the SPO)

NOTE 3: Similar examples could be an alternative transfer characteristics (ATC) SEI message, a content colour volume (CCV) SEI message, or an ambient viewing environment (AVE) SEI message.

In this example, the wrapping functionality is useful, since the indicated CLL information is only valid if the tone mapping process indicated in the TMI SEI message is applied, so it is important for the CLL SEI message should not be used (and thus should be hidden from legacy decoders) unless the tone mapping processing has been applied.

The importance indication functionality is also useful, as it allows the first stage to be indicated as optional (not necessary, not important) to the operation or interpretation of the indicated sequence of operations.

In this example, it might also be reasonable for the second stage to have the wrapping flag equal to 1 (po_sei_wrapping_flag[ 1 ] = 1) so that the tone mapping function would not be applied by legacy decoders.

In the case that the third stage is an ATC SEI message or an AVE SEI message, it might also be reasonable for the third stage to have the importance flag set to 1 (po_sei_importance_flag[ 2 ] = 1) to indicate that the sequence of operations should only be applied if the decoder understands that this will transform the video content in a way that changes the transfer characteristics used for its display or that this will transform the video for use in a different viewing environment.

# Proposed syntax

The proposed syntax is as follows:

| sei_processing_order( payloadSize ) { | Descriptor |
| --- | --- |
| po_num_sei_messages_minus2 | u(8) |
| for( i = 0; i < po_num_sei_messages_minus2 + 2; i++) { |  |
| po_sei_wrapping_flag[ i ] | u(1) |
| po_sei_importance_flag[ i ] | u(1) |
| if( po_sei_wrapping_flag[ i ] ) { |  |
| reserved_alignment_6bits | u(6) |
| sei_message( ) |  |
| } else { |  |
| po_sei_prefix_flag[ i ] | u(1) |
| po_sei_payload_type[ i ] | u(13) |
| if( po_sei_prefix_flag[ i ]) { |  |
| po_num_prefix_bytes[ i ] | b(8) |
| for( j = 0; j < po_num_prefix_bytes[ i ]; j++ ) |  |
| po_prefix_byte[ i ][ j ] | b(8) |
| } |  |
| } |  |
| po_sei_processing_order[ i ] | u(8) |
| } |  |
| } |  |

# Proposed semantics

(Only the aspects relevant to the proposed changes are given here.)

(Delete the statement saying: “When there are multiple SEI processing order SEI messages present in a CVS, they shall have the same content,” as discussed above.)

po_num_sei_messages_minus2 plus 2 indicates the number of SEI messages that have a processing order indicated in the SEI processing order SEI message.

(Delete the statement saying: “It is a requirement of bitstream conformance that, within an SEI processing order SEI message, there shall be at least two pairs of the syntax elements po_sei_payload_type[ i ] and po_sei_processing_order[ i ], and there shall be at least two values of po_sei_processing_order[ i ] that are not equal,” since the constraint is naturally enforced by the “_minus2” encoding.)

…

po_sei_importance_flag[ i ] indicates the degree of importance determined by the encoder for the SEI message with index i.

If the decoding system cannot interpret or does not support any indicated SEI message that has po_sei_importance_flag[ i ] equal to 1, it should ignore the entire SEI processing order SEI message.

reserved_alignment_6bits has no meaning and shall be equal to 0 in bitstreams conforming to this version of this Specification. Decoders shall allow this syntax element to have other values and shall ignore the value.

If po_sei_wrapping_flag[ i ] is equal to 0, an SEI message should be present outside of the SEI processing order SEI message with payloadType equal to po_sei_payload_type[ i ]. However, if po_sei_wrapping_flag[ i ] is equal to 0 and no SEI message is present with payloadType equal to po_sei_payload_type[ i ], the following applies:

- If po_sei_importance_flag[ i ] is equal to 1, the decoder should ignore the entire SEI processing order SEI message.

- Otherwise, the decoder should ignore all data associated with the loop variable value of i.

…

po_sei_processing_order[ 0 ] shall be equal to 0, and for i greater than 0, po_sei_processing_order[ i ] shall be equal to po_sei_processing_order[ i − 1 ] or po_sei_processing_order[ i − 1 ] + 1.

The value of po_sei_processing_order[ po_num_sei_messages_minus2 + 1 ] shall not be equal to 0.

# Closing observation

Finally, we point out that the SEI processing order message can be useful not only for indicating stages of processing operations, but also for indicating the properties and intended uses of the video content after such particular stages of processing (e.g. by sending CLL, ATC, CCV or AVE SEI messages, as discussed in section 2). The consequence of this observation is that the SEI processing order SEI message could be useful for more purposes than may have been previously appreciated.

# Patent rights declaration(s)

Dolby Laboratories may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).