| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0158-v2 |
| --- | --- |

| Title: | AHG12: LUT-based angle calculation for DIMD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alireza Aminlou Jani Lainema | Tel: Email: | first.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

In the current version of DIMD in ECM9.0, for each neighboring sample of the block, a ratio value is calculated based on the gradient in horizontal and vertical directions, and then the intra mode is derived by searching the ratio value inside an angle table. The ratio has high dynamic range between 0 and 65536 (i.e., 16 bits), and searching the ratio value inside the angle table is a sequential process. This contribution proposes to calculate the ratio value in logarithm-like domain and replace the ratio searching with a look-up-table. The following BD-rate and runtime impact observed when it is implemented in ECM-9:

AI { 0.01%, 0.05%, 0.05%, 99%, 97%}

RA { } // RA simulations are still running.

# Introduction

The current version of DIMD in ECM9.0 has the following steps for each block:

- For each sample in the top and left neighboring sides of the current block

  - Calculate gradient in horizontal and vertical direction (Gx and Gy)

  - Determine the region according to sing and magnitude of Gx and Gy

  - Calculate the ratio of Gx and Gy (using LUT-based multiply and shift)

  - Determine the angle index by searching ratio value inside an angle table

  - Determine the intra mode using angle index and region

  - Add magnitude of this mode to a histogram array according to intra mode

- Determine best intra modes and their corresponding weight values based on the histogram array

- Generate intra predictions using the best intra modes, and PLANAR mode

- Generate the final prediction using the weighted average of intra predictions

The complexity of the steps that are performed for each neighboring sample (i.e., 1.a – 1.f) can be critical for the overall complexity of DIMD process. This proposal concerns calculation of ratio (i.e., step 1.c) and angle index (i.e., step 1.d).

In step 1.c, the ratio is calculated by finding the smaller and larger value of Gx and Gy, and then dividing the small value by large value. The division operation is implemented by a LUT-based multiply and shift. As a results of division, the ratio value has high dynamic range, between 0 and 65536.

In step 1.d, the ratio is searched inside angle table, and the best index is selected in a way that the ratio has the closest value to the content of the angle table at index location. This search is simple, however a sequential process, that may introduce large latency to DIMD process. The content of the angle table in ECM9.0 is defined as below and has exponential-like shape:

int angTable[17] = { 0, 2048, 4096, 6144, 8192, 12288, 16384, 20480, 24576, 28672, 32768, 36864, 40960, 47104, 53248, 59392, 65536 };

# Proposed changes to ECM

This contribution proposes to calculate the ratio in logarithmic-like domain (ratioLog) and then calculate the angle index using a look-up-table based on the ratioLog. Calculation of the ratio in logarithmic-like domain reduces dynamic range of ratioLog value and makes angle index calculation easier in the next step. Calculation of the angle index using LUT is less complex than the current search method in current DIMD.

## ratioLog calculation

The value of the ratioLog is calculated in two steps. First, approximated logarithm values of Gx and Gy are calculated. Approximated logarithm is calculated using floorLog2 function for power of two numbers, and linear interpolation is used for the intermediate numbers between two consecutive power of two numbers, with 6bit precision is used for the fractional part. Second, the value of the ratioLog is calculated by subtracting approximated logarithm values of Gx and Gy. RatioLog is stored with 3bits precision for fractional part. Then it is clipped to 63 (i.e., 63/8 = 7.875) which represents almost the infinity in logarithmic domain. This reduces the size of LUT in the next step.

## Angle index calculation

Then ratioLog in mapped to an angle index using a LUT, which is defined as below:

int dimd_lut[dimd_lut_size] = {

16, 15, 14, 14, 13, 12, 11, 11, 10, 9, 9, 8, 8, 7, 7, 6, 6, 6, 5, 5, 5, 5, 4, 4, 4, 4, 3, 3, 3, 3, 2, 2, 2, 2, 2, 2, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0};

## Code changes in ECM9.0

Change of the code for step 1.c

| Current process | int s0 = gtY ? absy : absx; int s1 = gtY ? absx : absy; int x = floorLog2(s1); int norm = (s1 << 4 >> x) & 15; int v = g_gradDivTable[norm] ¦ 8; x += (norm != 0); int shift = 13 - x; int ratio; if (shift < 0) { shift = -shift; int add = (1 << (shift - 1)); ratio = (s0 * v + add) >> shift; } else { ratio = (s0 * v) << shift; } |
| --- | --- |
| Proposed process | int s0 = gtY ? absy : absx; int s1 = gtY ? absx : absy; int bs1 = floorLog2(s1); int bs0 = floorLog2(s0); int offset1 = (bs1 > 0) ? (1 << (bs1 - 1)) : 0; int offset0 = (bs0 > 0) ? (1 << (bs0 - 1)) : 0; int ls1 = (bs1 << bitShift) + (((s1 << bitShift) + offset1) >> bs1); int ls0 = (bs0 << bitShift) + (((s0 << bitShift) + offset0) >> bs0); int ls1 = (bs1 << bitShift) + ((s1 << bitShift) >> bs1); int ls0 = (bs0 << bitShift) + ((s0 << bitShift) >> bs0); int ratioLog = (ls1 - ls0 + offset2) >> bitShift2; if (ratioLog > dimd_lut_size - 1) ratioLog = dimd_lut_size - 1; |

Change of the code for step 1.d

| Current process | int idx = 16; for( int i = 1; i < 17; i++ ) { if( ratio <= angTable[i] ) { idx = ratio - angTable[i - 1] < angTable[i] - ratio ? i - 1 : i; break; } } |
| --- | --- |
| Proposed process | int idx = dimd_lut[ratioLog]; |

# Experiments results

Tables below summarize the coding efficiency of the proposed method in ECM 9.0 over ECM 9.0 using the JVET common test conditions.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 Full |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.04% | 0.04% | 99% | 94% |
| Class A2 | 0.01% | -0.04% | 0.02% | 99% | 97% |
| Class B | 0.00% | 0.06% | 0.07% | 99% | 98% |
| Class C | 0.00% | 0.03% | -0.01% | 99% | 97% |
| Class E | 0.00% | 0.16% | 0.15% | 99% | 98% |
| Overall | 0.01% | 0.05% | 0.05% | 99% | 97% |
| Class D | 0.04% | 0.07% | 0.08% | 99% | 96% |
| Class F | 0.03% | 0.06% | 0.01% | 99% | 97% |
| TGM |  |  |  |  |  |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 Full |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.15% | 0.04% | 97.2% |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.07% | -0.08% | 97.1% |  |
| Class C | 0.01% | -0.25% | 0.07% | 97.2% | 98.8% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.01% | -0.39% | -0.21% | 98.1% | 99.0% |
| Class F | 0.05% | 0.37% | 0.08% | 96.6% | 95.5% |
| TGM |  |  |  |  |  |

# Conclusion

The proposed method simplifies the implementation of DIMD process, with negligible impact on RD performance. It is requested to include this method in the next round of EE test.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).