| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting: Geneva, CH, 11–19 July 2023 | Document: JVET-AE0160-v1 |
| --- | --- |

| Title: | EE1-1.5: Optimization for complexity-performance trade-off of HOP network |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Renjie Chang Liqiang Wang Xiaozhong Xu Shan Liu | Email: | renjiechang@tencent.com liqiangwang@tencent.com xiaozhongxu@tencent.com shanl@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution reports the EE-1.5 result of JVET-AE0160. Based on the unified network architecture of High Operation Point (HOP), this contribution proposes further optimization including new resblock and split network design to improve the complexity-performance trade-off of HOP network. Specifically, based on the depthwise separable convolution and group convolution, two novel resblocks are proposed to reduce the computational complexity without affecting the coding performance too much. Besides, the split network design for luma and chroma components is introduced into HOP network. Based on NNVC-5.1, the test results are shown in order of RA and AI configurations as follows.

Compared with NNVC-5.0 anchor:

Test 1.5.1 (Test designed resblocks with depthwise separable convolution and group convolution):

- Subtest 1: HOP resblock (as base comparison method for Test 1.5.1)

RA: xx.xx%	xx.xx%		xx.xx%		EncT: xxx%	DecT: xxxxx%

AI : xx.xx%	xx.xx%		xx.xx%		EncT: xxx%	DecT: xxxxx%

- Subtest 2: HOP resblock with depthwise separable convolution

RA: xx.xx%	xx.xx%		xx.xx%		EncT: xxx%	DecT: xxxxx%

AI : xx.xx%	xx.xx%		xx.xx%		EncT: xxx%	DecT: xxxxx%

- Subtest 3: HOP resblock with group convolution

RA: xx.xx%	xx.xx%		xx.xx%		EncT: xxx%	DecT: xxxxx%

AI : xx.xx%	xx.xx%		xx.xx%		EncT: xxx%	DecT: xxxxx%

Test 1.5.2 (Test split network design):

- Subtest 1: HOP network (as base comparison method for Test 1.5.2)

RA: xx.xx%	xx.xx%		xx.xx%		EncT: xxx%	DecT: xxxxx%

AI : xx.xx%	xx.xx%		xx.xx%		EncT: xxx%	DecT: xxxxx%

- Subtest 2: HOP network with split architecture for luma and chroma

RA: xx.xx%	xx.xx%		xx.xx%		EncT: xxx%	DecT: xxxxx%

AI : xx.xx%	xx.xx%		xx.xx%		EncT: xxx%	DecT: xxxxx%

Introduction

In this contribution, further optimization including new resblock and split network design is proposed to improve the complexity-performance trade-off of HOP network. In Test 1.5.1, the depthwise separable convolution and group convolution are integrated into HOP resblock [1] to improve the complexity-performance trade-off, separately. In Test 1.5.2, the split network design similar to Low Operation Point (LOP) network is introduced into HOP network. Please note that, considering the time and resource mostly taken up by HOP development, all these proposed networks in this contribution are trained based on the training scripts of filter 0 and tested based on HOP interface to accelerate the development. Therefore, the HOP resblock/network is also trained as the based method based on training scripts of filter 0 for a fair comparison.

Proposed method

The network architecture for new resblock and split network design is firstly presented in Section 2.1 and Section 2.2, respectively. Then, the network information in both training stage and inference stage is provided in Section 2.3 and Section 2.4, respectively.

HOP resblock with depthwise separable convolution and group convolution

For convenience, the HOP resblock is integrated into filter0 network to verify the performance of the HOP resblock and serve as a basic method for comparison. The network architecture of Test 1.5.1-subtest 1 is shown in Fig. 1. To be consistent with the HOP network, the number of resblock N is set to 24 and the channel number of resblock is also designed according to HOP network design.

Fig. 1: Network architecture of Test 1.5.1-subtest 1

In Test 1.5.1, the HOP resblock with depthwise separable convolution and group convolution is also integrated into filter network. The proposed HOP resblock with depthwise separable convolution and group convolution is shown in Fig. 2. g in convolution layers denotes group convolution.

(a)   				 (b) 				    (c)

Fig. 2: Different resblocks. (a) HOP resblock, (b) HOP resblock with depthwise separable convolution, (c) HOP resblock with group convolution.

HOP network with split architecture design

In Test 1.5.2, the split architecture for luma and chroma components is integrated into HOP network. Similar to Test 1.5.1, the HOP network from JVET-AD0380 is trained based on training scripts of filter 0 and also can be regarded as a basic method for the comparison in Test 1.5.2. The HOP network with split architecture is shown is Fig. 3.

Fig. 3: HOP network architecture with split architecture

Compared with HOP network in Test1.5.2-subtest 1, only both the resblock part and tail part are split into luma part and chroma part, and other parts such as head part and channel numbers are identical to HOP network.

Network information in training stage

Since all these models are trained based on train scripts of filter 0, the training datasets and training strategy in these EE tests are different from those in JVET-AD0380. The network training details are shown as follows.

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | Tesla A100 40GB |
|  | Framework: | Pytorch v1.9.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 130 |
|  | Batch size: | 64 |
|  | Loss function: | L1, L2 |
|  | Training time (for 1 model): | ~10 days |
|  | Training data information: | DIV2K, TVD, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): |  |
| Optional |  |  |
|  | Number of iterations | 1200 |
|  | Patch size | 144x144 |
|  | Learning rate: | 1e-4 |
|  | Learning rate update strategy |  |
|  | Optimizer: | ADAM |
|  | Preprocessing: | flipped, rotated |
|  | Mini-batch selection process: | random cropped |
|  | Training data update strategy: |  |
|  | Other information: |  |

Network information in inference stage

In these EE tests, SADL [2] is adopted as the inference library and all the models are quantized into int16 precision. The network inference details are shown as follows.

| - Network Information in Inference Stage |  |  |  |
| --- | --- | --- | --- |
|  | Filters | Test 1.5.1 | Test 1.5.2 |
| Mandatory | HW environment: |  |  |
|  | GPU Type | CPU only |  |
|  | Framework: | SADL |  |
|  | Number of GPUs per Task | 0 |  |
|  | Number of Parameters (Each Model) | Subtest 1: 1.42M Subtest 2: 1.23M Subtest 3: 1.25M | Subtest 1: 1.45M Subtest 2: 0.93M |
|  | Total Number of Parameters (All Models) | Subtest 1: 1.42M Subtest 2: 1.23M Subtest 3: 1.25M | Subtest 1: 1.45M Subtest 2: 0.93M |
|  | Parameter Precision (Bits) | int16 |  |
|  | Memory Parameter (MB) | Subtest 1: 2.8M Subtest 2: 2.4M Subtest 3: 2.6M | Subtest 1: 2.8M Subtest 2: 1.9M |
|  | Multiply Accumulate (kMAC/pixel) | Subtest 1: 461 Subtest 2: 402 Subtest 3: 408 | Subtest 1: 480 Subtest 2: 316 |
|  | Calculation Method | block basis |  |
| Optional |  |  |  |
|  | Total Conv. Layers | 173 | 301 |
|  | Total FC Layers | 0 |  |
|  | Total Memory (MB) |  |  |
|  | Batch size: | 1 |  |
|  | Patch size | 144x144 |  |
|  | Changes to network configuration or weights required to generate rate points |  |  |
|  | Peak Memory Usage (Total) |  |  |
|  | Peak Memory Usage (per Model) |  |  |
|  | Border handling |  |  |
|  | Other information: |  |  |

Results

All the experiments are performed based on the common test condition in [3]. The test results of EE1-1.5.1 and EE1-1.5.2 are provided in Section 3.1 and Section 3.2, respectively. The average results are summarized as follows.

EE1-1.5.1-subtest 1:

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| RA |  |  |  |  |  |
| AI |  |  |  |  |  |

EE1-1.5.1-subtest 2:

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| RA |  |  |  |  |  |
| AI |  |  |  |  |  |

EE1-1.5.1-subtest 3:

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| RA |  |  |  |  |  |
| AI |  |  |  |  |  |

EE1-1.5.2-subtest 1:

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| RA |  |  |  |  |  |
| AI |  |  |  |  |  |

EE1-1.5.2-subtest 2:

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| RA |  |  |  |  |  |
| AI |  |  |  |  |  |

Results of EE1-1.5.1

Table 1. Performance of EE1-1.5.1-subtest 1

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-5.0 VTM anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -9.95% | -22.73% | -22.71% | 139% | 33944% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -11.48% | -21.39% | -23.48% | 134% | 31897% |
| Class F |  |  |  |  |  |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-5.0 VTM anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -7.71% | -16.86% | -19.61% | 114% | 14416% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -7.65% | -15.58% | -19.24% | 110% | 12750% |
| Class F |  |  |  |  |  |

Table 2. Performance of EE1-1.5.1-subtest 2

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-5.0 VTM anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -9.69% | -23.33% | -21.97% | 149% | 42941% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -11.07% | -21.56% | -22.67% | 145% | 40133% |
| Class F |  |  |  |  |  |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-5.0 VTM anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -7.49% | -16.92% | -19.45% | 117% | 18082% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -7.41% | -15.36% | -19.11% | 112% | 16114% |
| Class F |  |  |  |  |  |

Table 3. Performance of EE1-1.5.1-subtest 3

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-5.0 VTM anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-5.0 VTM anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

Results of EE1-1.5.2

Table 4. Performance of EE1-1.5.2-subtest 1

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

Table 5. Performance of EE1-1.5.2-subtest 2

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-5.0 anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-5.0 anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

Conclusions

In this contribution, several optimizations for HOP network are proposed. Specifically, the depthwise separable convolution and group convolution are integrated into HOP network to improve the complexity-performance trade-off. Besides, the split architecture design is also introduced into HOP network to improve the trade-off. Experimental results demonstrate the effectiveness of proposed methods.

References

- E. Alshina, F. Galpin. BoG report on NN-filter design unification. JVET-AD0380, 30th Meeting, Antalya, TR, 21–28 April 2023.

- F. Galpin, T. Dumas, P. Bordes, E. François. AHG11: Small Ad-hoc Deep-Learning Library (SADL) update. JVET-AB0086, 28th Meeting, Mainz, DE, 20–28 October 2022.

- E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AD2016, 30th Meeting, Antalya, TR, 21–28 April 2023.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).