| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0162-v2 |
| --- | --- |

| Title: | AHG11/AHG14: Fix MS-SSIM calculation for SR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Renjie Chang Liqiang Wang Xiaozhong Xu Shan Liu | Email: | renjiechang@tencent.com liqiangwang@tencent.com xiaozhongxu@tencent.com shanl@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In current NNVC-5.1 software, the MS-SSIM calculation for super resolution (SR) coding tools is not supported. An invalid MS-SSIM result is obtained when enabling those coding tools, such as reference picture resampling (RPR) and neural-network based super resolution (NNSR). This contribution proposes a fix for the MS-SSIM calculation both for conventional RPR and NNSR. This fix does not affect the PSNR results.

MS-SSIM results with bugfix for NNSR:

RA: -3.99%	-1.83%		-1.74%		EncT: 106%	DecT: 448%

AI : -3.56%	-3.12%		-1.96%		EncT: 113%	DecT: 420%

MS-SSIM results without bugfix for NNSR:

RA: -16.77%	-10.71%	#VALUE!	EncT: 107%	DecT: 489%

AI : -17.76%	-12.99%	-12.02%	EncT: 113%	DecT: 455%

Introduction

In the current NNVC-5.0 software, the MS-SSIM calculation for SR is not supported. Specifically, when some SR coding tools, such as RPR and NNSR [1], are enabled, the MS-SSIM result is wrongly calculated based on the down-sampled pictures, instead of up-sampled one.

To correct the MS-SSIM calculation for SR, the contribution proposes a fix as follows.

Firstly, the MS-SSIM calculation is corrected to be conducted based on up-sampled pictures.

#if JVET_AC0196_NNSR

if (m_pcEncLib->getUseNnsr() && m_pcEncLib->isResChangeInClvsEnabled())

{

‘’’’’’

upscaledPSNR[comp] = upscaledSSD ? 10.0 * log10( (double)maxval * maxval * upscaledWidth * upscaledHeight / (double)upscaledSSD ) : 999.99;

upscaledMsssim[comp] = xCalculateMSSSIM (upscaledOrgPB.bufAt(0, 0), upscaledOrgPB.stride, upscaledRecPB.bufAt(0, 0), upscaledRecPB.stride, upscaledWidth, upscaledHeight, bitDepth);

}

else

{

upscaledPSNR[comp] = dPSNR[comp];

upscaledMsssim[comp] = msssim[comp];

}

#else

if (m_pcEncLib->isResChangeInClvsEnabled())

{

‘’’’’’

upscaledPSNR[comp] = upscaledSSD ? 10.0 * log10( (double)maxval * maxval * upscaledWidth * upscaledHeight / (double)upscaledSSD ) : 999.99;

upscaledMsssim[comp] = xCalculateMSSSIM (upscaledOrgPB.bufAt(0, 0), upscaledOrgPB.stride, upscaledRecPB.bufAt(0, 0), upscaledRecPB.stride, upscaledWidth, upscaledHeight, bitDepth);

}

#endif

Then, the correct MS-SSIM values are added into results.

#if JVET_AC0196_NNSR

m_gcAnalyzeAll.addResult(upscaledPSNR, (double) uibits, MSEyuvframe, upscaledPSNR, upscaledMsssim, upscaledMsssim, isEncodeLtRef);

#else

m_gcAnalyzeAll.addResult(dPSNR, (double) uibits, MSEyuvframe, upscaledPSNR, msssim, upscaledMsssim, isEncodeLtRef);

#endif

#if JVET_AC0196_NNSR

m_gcAnalyzeI.addResult(upscaledPSNR, (double) uibits, MSEyuvframe, upscaledPSNR, upscaledMsssim, upscaledMsssim, isEncodeLtRef);

#else

m_gcAnalyzeI.addResult(dPSNR, (double)uibits, MSEyuvframe, upscaledPSNR, msssim, upscaledMsssim, isEncodeLtRef);

#endif

#if JVET_AC0196_NNSR

m_gcAnalyzeP.addResult(upscaledPSNR, (double) uibits, MSEyuvframe, upscaledPSNR, upscaledMsssim, upscaledMsssim, isEncodeLtRef);

#else

m_gcAnalyzeP.addResult(dPSNR, (double)uibits, MSEyuvframe, upscaledPSNR, msssim, upscaledMsssim, isEncodeLtRef);

#endif

#if JVET_AC0196_NNSR

m_gcAnalyzeB.addResult(upscaledPSNR, (double)uibits, MSEyuvframe, upscaledPSNR, upscaledMsssim, upscaledMsssim, isEncodeLtRef);

#else

m_gcAnalyzeB.addResult(dPSNR, (double) uibits, MSEyuvframe, upscaledPSNR, msssim, upscaledMsssim, isEncodeLtRef);

#endif

Besides, the MS-SSIM for each down-up picture is also output.

msg( NOTICE, " MS-SSIM2: [Y %6.4lf    U %6.4lf    V %6.4lf ]", upscaledMsssim[COMPONENT_Y], upscaledMsssim[COMPONENT_Cb], upscaledMsssim[COMPONENT_Cr] );

Results

The proposal is tested on top of NNVC-5.0 following NNVC CTC [2] but disabling all NN tools. The fix does not affect the PSNR result. For comparison, the test result of NNSR without the proposed bugfix is also shown below.

Results of NNSR with the proposed fix

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-5.0 VTM anchor |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -7.45% | -8.36% | -8.06% | -11.23% | -12.04% | -11.79% | 118% | 11016% |
| Class A2 | -3.22% | 3.20% | 3.71% | -8.70% | 2.88% | 3.07% | 115% | 1642% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -2.13% | -1.03% | -0.87% | -3.99% | -1.83% | -1.74% | 106% | 448% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-5.0 VTM anchor |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR |  | V-PSNR |  | Y-MSIM |  |  | U-MSIM | V-MSIM |  | EncT | DecT CPU |
| Class A1 | -5.54% | -7.19% |  | -7.10% |  | -7.49% |  |  | -8.12% | -7.01% |  | 170% | 5303% |
| Class A2 | -1.26% | -9.31% |  | -3.10% |  | -13.88% |  |  | -10.59% | -4.74% |  | 123% | 10310% |
| Class B | 0.00% | 0.00% |  | 0.00% |  | 0.00% |  |  | 0.00% | 0.00% |  | 100% | 100% |
| Class C | 0.00% | 0.00% |  | 0.00% |  | 0.00% |  |  | 0.00% | 0.00% |  | 100% | 100% |
| Class E | 0.00% | 0.00% |  | 0.00% |  | 0.00% |  |  | 0.00% | 0.00% |  | 100% | 100% |
| Overall | -1.13% | -2.75% |  | -1.70% |  | -3.56% |  |  | -3.12% | -1.96% |  | 113% | 420% |
| Class D | 0.00% | 0.00% |  | 0.00% |  | 0.00% |  |  | 0.00% | 0.00% |  | 100% | 100% |
| Class F | 0.00% | 0.00% |  | 0.00% |  | 0.00% |  |  | 0.00% | 0.00% |  | 100% | 100% |

Results of NNSR without the proposed fix

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-5.0 VTM anchor |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -7.45% | -8.36% | -8.06% | -42.44% | -29.45% | -28.71% | 121% | 12630% |
| Class A2 | -3.22% | 3.20% | 3.71% | -41.43% | -24.09% | #VALUE! | 117% | 1885% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 106% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 104% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -2.13% | -1.03% | -0.87% | -16.77% | -10.71% | #VALUE! | 107% | 489% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 105% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 108% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-5.0 VTM anchor |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -5.54% | -7.19% | -7.10% | -41.44% | -21.28% | -20.83% | 172% | 6065% |
| Class A2 | -1.26% | -9.31% | -3.10% | -65.10% | -56.66% | -51.26% | 126% | 11900% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 106% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 106% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 104% |
| Overall | -1.13% | -2.75% | -1.70% | -17.76% | -12.99% | -12.02% | 113% | 455% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 103% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 102% |

Conclusions

This contribution proposes a fix for the MS-SSIM calculation of down-up pictures. This fix does affect the PSNR results and bits results. It is recommended that adopt this proposal to NNVC reference software.

References

- R. Chang, L. Wang, X. Xu, S. Liu. EE1-2.2: GOP Level Adaptive Resampling with CNN-based Super Resolution. JVET-AC0196, 29th Meeting, by teleconference, 11–20 January 2023.

- E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AD2016, 30th Meeting, Antalya, TR, 21–28 April 2023.

Patent rights declaration(s)

Tencent does not have any current or pending patent rights relating to the technology described in this contribution.