| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0169-v4 |
| --- | --- |

| Title: | EE2-2.2/EE2-2.3/EE2-2.4: Bi-predictive IBC GPM, IBC BVP-merge, bi-predictive IBC merge, and IBC MBVD list derivation for camera captured and screen contents |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Yoshitaka Kidani, Haruhisa Kato, Kei Kawamura (KDDI) 2Changyue Ma, Xiaoyu Xiu, Wei Chen, Hong-Jheng Jhu, Che-Wei Kuo, Ning Yan, Xianglin Wang (Kwai) 3Gleb Verba, Zhi Zhang, Pavel Nikitin, Han Huang, Vadim Seregin, Marta Karczewicz (Qualcomm) | Tel: Email: | 1yo-kidani@kddi.com 2machangyue@kwai.com 3gverba@qti.qualcomm.com, zhizhang@qti.qualcomm.com |
| Source: | KDDI Corporation. (KDDI Research, Inc.), Kwai Inc., and Qualcomm Incorporated |  |  |

# Abstract

This contribution reports the experimental results of Test 2.2c (bi-predictive IBC GPM), Test 2.3 (bi-predictive IBC), and Test 2.4 (IBC MBVD) in the context of Exploration Experiments 2 (JVET-AD2024).

Test 2.2c proposes bi-predictive IBC GPM for screen content coding. Test 2.3 suggests IBC BVP-merge and bi-predictive IBC merge for camera captured content coding. Test 2.4 presents IBC MBVD list derivation.

The experimental results reported show the best performance in Test 2.4c over ECM-9.0 as follows:

AI:

Overall : {-0.15%, -0.19%, -0.13%, 101.3%, 100.1%}, Class F: {-0.42%, -0.30%, -0.39%, 100.9%, 101.7%}, Class TGM : {-1.06%, -1.00%, -0.99%, 100.6%, 102.3%};

RA:

Overall : {-0.05%, -0.16%, -0.01%, 100.5%, 101.4%}, Class F: {-0.38%, -0.72%, -0.51%, 100.6%, 101.7%}, Class TGM : {-0.86%, -0.86%, -0.92%, 100.4%, 101.8%};

Several missing test points in RA are copied from anchor.

# Introduction

In the 30th JVET meeting, an adaptation of bi-predictive IBC for camera captured and screen contents was investigated in JVET-AD0134 [1] and JVET-AD0215 [2]. The technologies in the two proposals can be summarized in two aspects. The first aspect is an adaptation of bi-predictive IBC GPM only for screen content. The second aspect is that IBC BVP-merge and bi-predictive IBC merge for camera captured content. The two aspects are studied as Test 2.2c and Test 2.3.

In addition, in the 30th JVET meeting, IBC MBVD list derivation was also investigated in JVET-AD0396 [3]. The performance of IBC MBVD list derivation stand-alone and the combination with bi-predictive IBC GPM, IBC BVP-merge, and bi-predictive IBC are studied as Test 2.4.

# Test description

## Summary of testing elements in Test 2.2c, Test 2.3, and Test 2.4

Table 1 describes the summary of testing elements in Test 2.2c, Test 2.3, and Test 2.4 regarding the adaptation for natural content and screen content.

Table 1 Summary of testing elements in Test 2.2c, Test 2.3, and Test 2.4

| # | Camera captured content | Screen content |
| --- | --- | --- |
| 2.2c | N/A | Bi-predictive IBC GPM |
| 2.3a | IBC BVP-merge | N/A |
| 2.3b | 2.3a + bi-predictive IBC merge (with enabled ECM IBC MBVD and IBC regular merge) | N/A |
| 2.4a | IBC MBVD list derivation (with enabled ECM IBC MBVD and IBC regular merge) | IBC MBVD list derivation |
| 2.4c | 2.4a + 2.3b | 2.4a + 2.2c |

## Bi-predictive IBC GPM (Test 2.2c)

Bi-predictive IBC GPM generates the prediction samples for both GPM partitions using the different IBCs. In the bi-predictive IBC GPM, the core IBC GPM design (e.g., 48 GPM modes, IBC merge candidate list) is kept the same as that of the uni-predictive IBC GPM in in ECM-9.0. Here, the uni-predictive IBC GPM generates the prediction samples for each GPM partition using the IBC and intra mode, respectively.

Only the signaling and blending schemes are slightly modified from the existing uni-predictive IBC GPM. For signallling, two flags are signaled to indicate the prediction modes of two partitions. For blending, GPM adaptive blending for inter GPM is also utilized in bi-predictive IBC GPM

## IBC BVP-merge and bi-predictive IBC merge (Test 2.3)

IBC BVP-merge, inspired by AMVP-merge, derives the two required BVs from IBC block vector prediction (BVP) and IBC merge. Two different indices for the IBC BVP and the IBC merge candidates are signalled from the encoder to the decoder, different from the AMVP-merge mode.

Bi-predictive IBC merge derives the two BVs from the existing IBC merge candidate list, utilizing two different indices. The two indices are signaled from the encoder to the decoder. The target of the bi-predictive IBC merge is IBC MBVD and IBC regular merge.

## IBC MBVD list derivation (Test 2.4)

IBC MBVD list derivation allows adaptive BVD offsets along MBVD directions. Specifically, the MBVD candidates search is a two-step algorithm, which starts with checking template SAD costs of offsets added to BVP along each direction with the interval of M-pel. The second step of the search checks template SAD costs for intermediate candidates around the selected candidates. The candidates with the lowest TM cost are included into the final MBVD list.

# Experimental results

The proposed methods were implemented on top of the ECM-9.0 [4] and tested according to the common test condition of AI and RA configuration as specified in the JVET-AD2017 [5] and JVET-AD2024 [6].

Experimental results of Test 2.2c, Test 2.3a, Test 2.3b, Test 2.4a, and Test 2.4c are summarized below.

Table 2 Results of Test 2.2c over ECM-9.0.

|  |  |  | AI |  |  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| A1 |  |  |  |  |  |  |  |  |  |  |
| A2 |  |  |  |  |  |  |  |  |  |  |
| B |  |  |  |  |  |  |  |  |  |  |
| C |  |  |  |  |  |  |  |  |  |  |
| E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| D |  |  |  |  |  |  |  |  |  |  |
| F | -0.11% | -0.26% | -0.23% | 100.2% | 99.6% | -0.20% | -0.31% | -0.35% | 100.1% | 99.5% |
| TGM | -0.42% | -0.39% | -0.35% | 100.1% | 98.7% | -0.29% | -0.23% | -0.33% | 100.2% | 99.4% |

Table 3 Results of Test 2.3a over ECM-9.0.

|  |  |  | AI |  |  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| A1 | 0.01% | -0.15% | 0.01% | 98.9% | 100.7% |  |  |  |  |  |
| A2 | -0.06% | -0.13% | -0.05% | 99.4% | 100.0% |  |  |  |  |  |
| B | -0.08% | -0.10% | 0.03% | 99.5% | 99.7% |  |  |  |  |  |
| C | 0.00% | -0.02% | -0.01% | 100.4% | 100.4% |  |  |  |  |  |
| E | -0.05% | 0.04% | 0.12% | 99.4% | 99.9% |  |  |  |  |  |
| Overall | -0.04% | -0.07% | 0.02% | 99.5% | 100.1% |  |  |  |  |  |
| D | 0.03% | 0.11% | 0.07% | 101.0% | 100.2% |  |  |  |  |  |
| F |  |  |  |  |  |  |  |  |  |  |
| TGM |  |  |  |  |  |  |  |  |  |  |

Table 4 Results of Test 2.3b over ECM-9.0.

|  |  |  | AI |  |  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| A1 | -0.01% | -0.02% | -0.02% | 100.1% | 100.6% | 0.02% | -0.11% | 0.11% | 100.1% | 100.3% |
| A2 | -0.25% | -0.28% | -0.21% | 100.4% | 100.8% | -0.09% | -0.03% | -0.11% | 100.2% | 99.2% |
| B | -0.22% | -0.19% | -0.27% | 100.9% | 100.0% | -0.09% | -0.04% | -0.15% | 100.2% | 99.7% |
| C | -0.08% | -0.06% | -0.13% | 102.3% | 100.3% | 0.01% | -0.03% | 0.06% | 100.3% | 100.1% |
| E | -0.16% | -0.09% | -0.17% | 101.2% | 100.6% |  |  |  |  |  |
| Overall | -0.15% | -0.13% | -0.17% | 101.0% | 100.4% | -0.04% | -0.05% | -0.03% | 100.2% | 99.8% |
| D | -0.02% | -0.04% | -0.05% | 103.1% | 101.2% | -0.02% | -0.19% | 0.14% | 100.2% | 100.1% |
| F |  |  |  |  |  |  |  |  |  |  |
| TGM |  |  |  |  |  |  |  |  |  |  |

Table 5 Results of Test 2.4a over ECM-9.0.

|  |  |  | AI |  |  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| A1 | -0.01% | 0.07% | -0.08% | 100.0% | 99.9% | -0.02% | -0.10% | -0.03% | 100.5% | 101.1% |
| A2 | -0.10% | -0.14% | -0.09% | 100.5% | 100.7% | -0.05% | -0.15% | -0.16% | 99.8% | 100.5% |
| B | -0.10% | -0.01% | -0.05% | 101.2% | 101.0% | -0.07% | 0.05% | -0.13% | 99.7% | 100.1% |
| C | -0.03% | -0.05% | -0.04% | 102.2% | 102.1% | 0.01% | -0.09% | -0.13% | 99.9% | 98.8% |
| E | -0.09% | -0.03% | 0.15% | 102.2% | 101.6% |  |  |  |  |  |
| Overall | -0.07% | -0.03% | -0.02% | 101.3% | 101.1% | -0.04% | -0.06% | -0.12% | 99.9% | 100.1% |
| D | 0.01% | -0.19% | 0.02% | 100.9% | 99.5% | 0.01% | 0.09% | -0.06% | 100.1% | 98.5% |
| F | -0.35% | -0.21% | -0.07% | 100.4% | 101.8% | -0.34% | -0.30% | -0.24% | 100.5% | 101.7% |
| TGM | -0.74% | -0.73% | -0.73% | 100.3% | 101.5% | -0.59% | -0.60% | -0.59% | 100.6% | 100.6% |

Table 6 Results of Test 2.4c over ECM-9.0.

|  |  |  | AI |  |  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| A1 | 0.00% | 0.03% | -0.02% | 100.6% | 100.1% | 0.00% | -0.20% | 0.09% | 100.4% | 103.4% |
| A2 | -0.25% | -0.31% | -0.21% | 100.7% | 99.5% | -0.09% | -0.15% | -0.12% | 100.5% | 102.4% |
| B | -0.23% | -0.28% | -0.24% | 101.1% | 99.7% | -0.11% | -0.14% | -0.12% | 100.4% | 101.5% |
| C | -0.08% | -0.17% | -0.07% | 102.3% | 101.2% | 0.02% | -0.15% | 0.11% | 100.6% | 98.9% |
| E | -0.18% | -0.18% | -0.04% | 101.4% | 99.9% |  |  |  |  |  |
| Overall | -0.15% | -0.19% | -0.13% | 101.3% | 100.1% | -0.05% | -0.16% | -0.01% | 100.5% | 101.4% |
| D | -0.03% | -0.14% | 0.07% | 101.2% | 99.3% | 0.03% | 0.08% | 0.29% | 100.3% | 98.9% |
| F | -0.42% | -0.30% | -0.39% | 100.9% | 101.7% | -0.38% | -0.72% | -0.51% | 100.6% | 101.7% |
| TGM | -1.06% | -1.00% | -0.99% | 100.6% | 102.3% | -0.86% | -0.86% | -0.92% | 100.4% | 101.8% |

Several missing test points in RA are copied from anchor. Those are indicated with italic script.

# References

- Y. Kidani, et al., “Non-EE2: Bi-predictive IBC for natural and screen content,” JVET-AD0134, April 2023.

- C. Ma, et al., “Non-EE2: Extension of IBC-GPM,” JVET-AD0215, April 2023.

- Z. Zhang, et al., “EE2-related: IBC MBVD for coding of camera captured content,” JVET-AD0396, April 2023.

- ECM-9.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-9.0

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AD2017, April. 2023.

- V. Seregin, et al., “Exploration experiment on enhanced compression beyond VVC capability (EE2)”, JVET-AD2024, April 2023.

# Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Kwai Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).