| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0170-v1 |
| --- | --- |

| Title: | Non-EE2: Enhancements on CCP merge |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Zhipin Deng2, Li Zhang1 18910 University Center Lane San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | {zhangkai.video, zhipin.deng, lizhang.idm}@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This proposal presents the results of the enhanced CCP merge mode. The fusion prediction can be generated by the CCP-merge prediction and the MM-CCCM prediction as a weighted sum. Besides, CCP merge mode is harmonized with LB-CCP by including the LB-CCP flag in the CCP merge candidate. Simulation results based on ECM-9.0 are reported as below:

AI {}; RA {}.

# Introduction

Non-local cross-component prediction (CCP) mode a.k.a. CCP merge mode and LB-CCP have been adopted in ECM-9.0. With the CCP merge mode, a CCP model can be inherited from a previously coded block or from a history-based table. With LB-CCP, CCP prediction can be filtered, indicated by a flag.

# Proposed method

In a proposed CCP-merge fusion method, a fusion prediction can be generated by the CCP-merge prediction and the MM-CCCM prediction as a weighted sum. A flag is signalled conditioned on the CCP-merge flag, to indicate whether the CCP-merge prediction should be fused with the MM-CCCM prediction. At encoder, the fused predictions with different CCP-merge indices are pre-selected by SATD costs and two extra RDO tests are applied based on the pre-selection.

Besides, it is proposed to harmonize CCP merge mode and LB-CCP. The LB-CCP flag is included in the CCP merge candidate.

# Experimental results

The proposed method is implemented on top of ECM-9.0 [1] and tested following the common test conditions [2]. The simulation results are summarized in Table 1. In the test, the weighting values on the two predictions are equal.

Table 1: Coding performance over ECM-9.0

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% | -0.51% | -0.47% |  |  |  |  |  |  |  |
| Class C | -0.01% | -0.20% | -0.15% |  |  | 0.02% | -0.21% | 0.00% |  |  |
| Class E | -0.02% | -0.26% | -0.27% |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | 0.02% | -0.13% | -0.01% |  |  | 0.01% | -0.37% | 0.06% |  |  |
| Class F |  |  |  |  |  |  |  |  |  |  |

# Conclusions

The proposed method can achieve some BD-rate gains with minor runtime changes. It is recommended to study the proposed method in the next round of EE2.

# References

- ECM-9.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-9.0.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).