| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0171-v1 |
| --- | --- |

| Title: | AHG11: Neural network-based in-loop filter with layer normalization |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Li, Kai Zhang, Li Zhang | Tel: Email: | yue.li@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution proposes to introduce the layer normalization technique into the NN-based filtering design. The proposed method uses separate models for luma and chroma. BD-rate changes on top of NNNC-5.0 VTM anchor and NNVC-5.0 anchor, as well as model complexities, are reportedly summarized as below:

Compared with NNVC-5.0 VTM anchor: RA: %, %, %, AI: %, %, %

Compared with NNVC-5.0 anchor: RA: %, %, %, AI: %, %, %

kMAC/pixel of frame basis: 396 (luma: 370, chroma: 26), #parameters: 2.5M (luma: 1.9M, chroma: 0.6M)

Introduction

Layer normalization is a technique to normalize the distributions of intermediate layers and has been widely used in the transformer block [1].

Proposed method

Network

This contribution proposes to introduce the layer normalization technique into the NN-based filtering design. Specifically, the basic building block in the proposed model comprises layer normalization, 1×1 and 3×3 convolutional layers. Inference interface is based on the NN-based filter set #1 in NNVC-5.0 [2].

Inference

Libtorch is used for performing the inference of the proposed CNN filters in NNVC. The network information in the inference stage is provided in Table 1 as suggested by [3].

Table 1. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | Libtorch |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | see Section 3.2 |
|  | Parameter Precision (Bits) | 32 (float) |
|  | Memory Parameter (MB) | see Section 3.2 |
|  | Multiply Accumulate (kMAC/pixel) | see Section 3.2 |
| Optional |  |  |
|  | Total Conv. Layers |  |
|  | Total FC Layers |  |
|  | Total Memory (MB) |  |
|  | Batch size: |  |
|  | Patch size |  |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information: |  |

Training

PyTorch is used as the training platform. DIV2K [4] and BVI-DVC [5] are adopted as the training datasets. The network information in the training stage is provided in Table 2 as suggested by [3].

Table 2. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: A100-SXM-80GB |
|  | Framework: | PyTorch |
|  | Number of GPUs per Task | 2 |
|  | Epoch: | 60 |
|  | Batch size: | 64 |
|  | Training time: | 240h |
|  | Training data information: | DIV2K, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0 + new MCTF, QP {17, 22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 128×128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

Experimental results

The proposed method is integrated into NNVC-5.0 (filter set #1) and tested according to the common test conditions defined in [3].

Performance comparison

Compared with NNVC-5.0 VTM anchor, the proposed method brings on average {%, %, %} and {%, %, %} BD-rate changes for {Y, Cb, Cr} under RA and AI configurations.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-5.0 VTM Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -11.06% | -33.51% | -27.93% | 185% | 50095% |
| Class C | -12.33% | -23.84% | -24.96% | 142% | 34164% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -14.07% | -24.92% | -26.76% | 133% | 31305% |
| Class F | -5.98% | -14.13% | -12.39% | 234% | 16938% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over NNVC-5.0 VTM Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -8.19% | -25.57% | -24.30% | 150% | 31427% |
| Class C | -9.32% | -20.19% | -23.18% | 127% | 20620% |
| Class E | -11.95% | -24.86% | -24.65% | 154% | 31035% |
| Overall |  |  |  |  |  |
| Class D | -8.81% | -21.46% | -23.50% | 120% | 16959% |
| Class F |  |  |  |  |  |

Compared with NNVC-5.0 anchor (nn intra + low-complexity filter), the proposed method (with nn intra) brings on average {%, %, %} and {%, %, %} BD-rate changes for {Y, Cb, Cr} under RA and AI configurations.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-5.0 Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -6.56% | -27.89% | -22.72% | 148% | 687% |
| Class C | -7.47% | -15.68% | -17.68% | 124% | 558% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -8.17% | -17.63% | -19.47% | 120% | 451% |
| Class F | -4.02% | -9.47% | -8.48% | 165% | 194% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over NNVC-5.0 Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -4.11% | -20.94% | -19.53% | 127% | 709% |
| Class C | -4.92% | -14.43% | -17.38% | 117% | 606% |
| Class E | -6.03% | -21.31% | -21.36% | 127% | 693% |
| Overall |  |  |  |  |  |
| Class D | -3.97% | -16.39% | -17.71% | 113% | 474% |
| Class F |  |  |  |  |  |

kMAC/pixel and memory

|  | kMAC/pixel (frame basis) | kMAC/pixel (block basis) | Total Parameter Number | Total parameter memory (float) |
| --- | --- | --- | --- | --- |
| Proposed | 396 | 501 | 2.5M | 10M |

Conclusions

This contribution presents a NN filtering method featuring layer normalization operations. The proposed filtering method shows a favorable trade-off in terms of the coding performance and complexity.

References

- J. L. Ba, J. R. Kiros, G. E. Hinton, Layer normalization, arXiv preprint arXiv:1607.06450 (2016).

- https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/

- E. Alshina, R. -L. Liao, S. Liu, A. Segall, Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-AD2016, by teleconference, April 2023.

- https://data.vision.ee.ethz.ch/cvl/DIV2K/ R. Timofte, E. Agustsson, S. Gu, J. Wu, A. Ignatov, L. V. Gool.

- Ma, Di, Fan Zhang, and David R. Bull. "BVI-DVC: A Training Database for Deep Video Compression." arXiv preprint arXiv:2003.13552 (2020).

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).