| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0174-v1 |
| --- | --- |

| Title: | AHG7: On TM control for non-inter tools |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | {zhipin.deng, zhangkai.video, lizhang.idm}@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This proposal aims at solving an identified issue reported by AHG7. It is asserted that template costs are calculated in several IBC and intra techniques, but there is no option to turn them off correctly in ECM. In ECM-9.0, three issues are observed. Firstly, ARMC of serval IBC techniques are erroneously turned off by the inter TM SPS flag, which messed up the tool evaluation results of inter TM tools (group 1) and non-inter TM tools (group 3). Secondly, the same IBC ARMC aspects are erroneously turned on even if they are required to be turned off in group 3 off test. Thirdly, the template aspect of LBCCP is not disabled in group 3 off test. This contribution proposes to introduce an SPS control flag to solve such identified issues in ECM.

# Problem Statement

According to AHG7 mandates [1], the template matching (TM) search for inter tools (group 1) and non-inter tools (group 3) should be tested independently. It was agreed to define the TM search as calculating TM cost more than one time [2]. However, there are several issues identified in ECM-9.0, which don’t align with the AHG7 instructions.

Firstly, Adaptive Reordering based Motion Compensation (ARMC) for inter tools and non-inter tools are mixed up. In ECM-9.0, there is an SPS flag (i.e., inter TM sps flag) to control the enabling/disabling of template based inter tools. Moreover, there is an SPS flag (i.e., ARMC sps flag) to control the enabling/disabling of ARMC. The ARMC is used not only for inter tools, but also for IBC and intra tools. Currently, during the group 1 off test, the inter TM sps flag would force the ARMC sps flag to be equal to false, which would erroneously disable IBC ARMC aspects. It is claimed that such behavior breaks the original design intention and messed up the tool evaluation results of inter TM tools (group 1) and non-inter TM tools (group 3).

Secondly, the ARMC is used to reorder the IBC candidates during the IBC merge list construction and IBC MBVD list construction. As there is no control to turn off the ARMC just for non-inter tools, IBC ARMC aspects are erroneously turned on even if group 3 off requires them to be turned off.

Thirdly, the template aspect of Local-Boosting Cross-Component Prediction (LBCCP) is not disabled in group 3 off test. It requires a comparison between two template costs, and the one with lower cost is finally selected. The template part is just one of two technical aspects of LBCCP but not the whole. Furthermore, it is not appropriate to have a flag for every template related element of each coding tool just for testing purpose.

# Proposed method

To solve the above problems, it is proposed to use an SPS flag (i.e., non-inter TM sps flag) to control the non-inter TM tools. With the proposed SPS flag, the problems can be solved as follows:

- The ARMC of inter tools (group 1) and non-inter tools (group 3) are decoupled.

  - The ARMC is turned off only if both inter TM sps flag and non-inter TM sps flag are equal to false. The inter ARMC is controlled by both inter TM sps flag and ARMC sps flag, and the non-inter ARMC is controlled by both non-inter TM sps flag and ARMC sps flag.

  - Group 1 off test would set inter TM sps flag to be false.

  - Group 3 off test would set non-inter TM sps flag to be false.

- The template aspect of LBCCP is controlled by the non-inter TM sps flag.

- The SPS flags of current tools in group 3 are controlled by the non-inter TM sps flag.

# Experimental results

The proposed method is implemented on top of AHG7 base software (i.e., ECM-9.0 commit #3f6ca8e0) [1]. According to AHG7 mandates, the proposed group 1 and group 3 tool-off test results versus corresponding ECM-9.0 tool off tests are summarized in Table 1-2. Per-sequence results of class F and TGM with the proposed change are summarized in Table 3-4.

Table 1: Coding performance of the proposed group 1 off test (versus ECM-9.0 group 1 off test)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 group 1 off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-9.0 group 1 off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

Table 2: Coding performance of the proposed group 3 off test (versus ECM-9.0 group 3 off test)

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 group 3 off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.14% | 0.09% | 98.9% | 99.47% |
| Class A2 | 0.03% | 0.13% | 0.15% | 99.3% | 100.65% |
| Class B | 0.02% | 0.16% | 0.18% | 99.3% | 98.19% |
| Class C | 0.04% | 0.20% | 0.09% | 100.1% | 98.39% |
| Class E | 0.02% | -0.04% | 0.09% | 99.0% | 104.58% |
| Overall | 0.03% | 0.13% | 0.13% | 99.4% | 99.90% |
| Class D | 0.01% | -0.05% | 0.16% | 100.0% | 103.46% |
| Class F | 1.61% | 1.41% | 1.35% | 92.6% | 79.04% |
| Class TGM | 3.08% | 3.00% | 3.00% | 93.4% | 75.83% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-9.0 group 3 off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.09% | 0.01% | 98.7% | 97.2% |
| Class C | -0.04% | 0.06% | 0.01% | 99.0% | 99.4% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.02% | -0.02% | 0.10% | 99.1% | 102.2% |
| Class F | 0.97% | 0.81% | 0.78% | 96.3% | 96.1% |
| Class TGM | 2.41% | 2.65% | 2.92% | 98.9% | 98.2% |

Table 3: Per-sequence results of the proposed group 1 off test (versus ECM-9.0 group 1 off test)

|  |  | Random Access Main 10 |  |  |  |  | Low Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over ECM-9.0 group 1 off |  |  |  |  | Over ECM-9.0 group 1 off |  |  |  |  |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class F | BasketballDrillText |  |  |  |  |  |  |  |  |  |  |
|  | ArenaOfValor |  |  |  |  |  |  |  |  |  |  |
|  | SlideEditing |  |  |  |  |  |  |  |  |  |  |
|  | SlideShow |  |  |  |  |  |  |  |  |  |  |
| Class TGM | FlyingGraphic |  |  |  |  |  |  |  |  |  |  |
|  | Desktop |  |  |  |  |  |  |  |  |  |  |
|  | Console |  |  |  |  |  |  |  |  |  |  |
|  | ChineseEditing |  |  |  |  |  |  |  |  |  |  |
|  | Class F |  |  |  |  |  |  |  |  |  |  |
|  | ClassTGM |  |  |  |  |  |  |  |  |  |  |

Table 4: Per-sequence results of the proposed group 3 off test (versus ECM-9.0 group 3 off test)

|  |  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over ECM-9.0 group 3 off |  |  |  |  | Over ECM-9.0 group 3 off |  |  |  |  |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class F | BasketballDrillText | 0.81% | 1.01% | 0.70% | 93.8% | 91.9% | 0.03% | -0.11% | 0.12% | 96.6% | 103.9% |
|  | ArenaOfValor | 0.77% | 0.84% | 0.82% | 93.5% | 91.9% | 0.19% | 0.03% | 0.14% | 94.4% | 94.9% |
|  | SlideEditing | 2.65% | 2.67% | 2.45% | 91.2% | 86.5% | 2.35% | 2.14% | 2.03% | 96.2% | 94.9% |
|  | SlideShow | 2.20% | 1.14% | 1.44% | 92.0% | 53.5% | 1.33% | 1.18% | 0.83% | 97.9% | 91.0% |
| Class TGM | FlyingGraphic | 3.94% | 3.76% | 3.70% | 92.4% | 73.8% | 3.00% | 3.82% | 4.89% | 97.6% | 94.0% |
|  | Desktop | 2.88% | 2.86% | 2.92% | 92.0% | 80.6% | 2.18% | 2.17% | 2.23% | 99.4% | 102.2% |
|  | Console | 3.89% | 3.73% | 3.74% | 96.0% | 80.3% | 2.97% | 3.04% | 3.07% | 100.7% | 99.3% |
|  | ChineseEditing | 1.62% | 1.64% | 1.64% | 93.1% | 69.3% | 1.47% | 1.56% | 1.48% | 98.0% | 97.5% |
|  | Class F | 1.61% | 1.41% | 1.35% | 92.6% | 79.0% | 0.97% | 0.81% | 0.78% | 96.3% | 96.1% |
|  | ClassTGM | 3.08% | 3.00% | 3.00% | 93.4% | 75.8% | 2.41% | 2.65% | 2.92% | 98.9% | 98.2% |

# Conclusions

It is asserted that the proposed method solves an identified issue of AHG7. It is proposed to adopt the proposed method to the ECM.

# References

- X. Li, L.-F. Chen, Z. Deng, J. Gan, E. François, H.-J. Jhu, R.-L. Liao, H. Wang, “JVET AHG report: ECM tool assessment (AHG7) ”, JVET-AD0007, Feb. 2023.

- Xiang Li, “AHG7: Report on AHG meeting on ECM tool assessment”, JVET-AD0041, Feb. 2023.

- ECM-9.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-9.0.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).