| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0178-v3 |
| --- | --- |

| Title: | Non-EE2: Cross-component prediction merge mode for chroma inter coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Man-Shu Chiang, Hsin-Yi Tseng, Chia-Ming Tsai, Cheng-Yen Chuang, Chih-Wei Hsu, Ching-Yeh Chen, Tzu-Der Chuang, Olena Chubach, Yi-Wen Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {man-shu.chiang, ivy-hy.tseng, chia-ming.tsai, cheng-yen.chuang, cw.hsu, chingyeh.chen, peter.chuang, olena.chubach, yiwen-yw.chen yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

In ECM-9.0, when the current chroma intra block is coded in cross-component prediction (CCP) merge mode, the cross-component model parameters of this chroma block can be inherited from previously coded blocks, as described in JVET-AD0188. This contribution extends the CCP merge mode to inter coding blocks, where the final chroma inter prediction combines motion-compensation predicted signals and cross-component predicted signals derived using an inherited CCP model.

On top of ECM-9.0, the simulation results of the proposed method are reported below:
RA: {Y BD-rate = 0.xx%, Cb BD-rate = -0.xx%, Cr BD-rate = -0.xx%, EncT = 10x%, DecT = 10x%}

Introduction

Cross-component prediction (CCP) merge mode [1] is adopted in ECM to improve the coding efficiency of chroma intra coding. In CCP merge mode, cross-component model parameters of the current chroma block can be inherited from a selected candidate in the CCP merge list comprising spatial adjacent, spatial non-adjacent, history-based, and default candidates. In EE2 Test 2.7 [2], temporal and shifted temporal candidates are further included in the CCP merge list for non-intra slices. These CCP models are inherited from blocks coded in CCLM, MMLM, CCCM, GLM, chroma fusion, and CCP merge modes in chroma intra coding. In this contribution, methods and results of extending CCP merge mode to inter coding are presented.

Proposed methods

In the proposed CCP merge mode for chroma inter coding, one flag indicates whether the proposed method is used for an inter coding block. If the proposed method is used, a CCP model is selected from the CCP merge list. After generating the cross-component predicted signals using the selected CCP model, the final prediction of the current chroma inter block is formed by combining the cross-component predicted and the motion-compensation predicted signals. Details of the construction of the CCP merge list for chroma inter coding and combined prediction of the current chroma inter block are described in the following sections.

Construction of CCP merge list for chroma inter coding

For the current chroma inter block, in addition to the existing candidates of CCP merge mode in chroma intra coding, as described in [1] and [2], the CCP merge list includes a second type of shifted temporal candidates. While positions of the first type shifted temporal candidates are derived based on a neighboring motion vector, the positions of the second type are derived based on the current motion vector. As depicted in Fig. 1, the position of the collocated block and the positions of C0i and C1i, 1 ≤ i ≤ 10, are shifted by a motion vector from the current block, where C0i and C1i are the positions of the temporal candidates. The second type shifted temporal candidates are inserted after the spatial adjacent candidates. After constructing the CCP merge list, the candidate with the lowest template cost is implicitly selected.

Fig. 1. Positions of shifted temporal candidates derived based on the current motion vector

Combined prediction of current chroma inter block

The final prediction for the current chroma inter block is the weighted average of the cross-component and motion-compensation predicted signals. The cross-component predicted signals are obtained through the regular intra coding process using the selected CCP model. The weights for combining predictions are fixed as (wCCP, winter) = (3/4, 1/4).

Experimental results

The proposed method is implemented on top of ECM-9.0 [3] and tested following the common test conditions [4]. The simulation results over ECM 9.0 are summarized in Table 1. The results with asterisk are short-length simulation results.

Table 1: Simulation results of the proposed method

|  | Over ECM 9.0 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RA |  |  |  |  |  | LB |  |  |  |  |  |
|  |  | Y | Cb | Cr | EncT | DecT |  | Y |  |  | Cb |  | Cr |  | EncT |  | DecT |  |
| Class A1 |  | -0.48% | -1.97% | -2.64% | 107.1% | 103.1% |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  | -0.63%* | -2.32%* | -3.38%* | 105.1%* | 102.2%* |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B |  | 0.04% | -1.22% | -1.07% | 104.7% |  | 100.6% |  | 0.11%* |  |  | -2.46%* |  | -2.62%* |  | 104.8%* |  | 101.7%* |  |
| Class C |  | 0.04% | -0.73% | -0.74% | 105.7% | 99.7% |  | 0.00% |  |  | -1.21% |  | -1.76% |  | 106.7% |  | 101.3% |  |
| Class E |  |  |  |  |  |  |  | 0.08% |  |  | 0.08% |  | -0.01% |  | 105.2% |  | 103.9% |  |
| Overall |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class D |  | 0.13% | -0.88% | -0.72% | 104.5% | 99.8% |  | -0.04% |  |  | -1.38% |  | -1.70% |  | 105.9% |  | 100.4% |  |
| Class F |  | -0.11% | -0.80% | -0.92% | 104.4% |  | 101.1% |  | -0.30%* |  |  | -1.71%* |  | -1.44%* |  | 103.3%* |  | 104.2%* |  |

Conclusions

This contribution presents extending CCP merge mode in chroma intra coding to chroma inter coding. The proposed method results in promising coding gains. It is recommended to study this contribution in EE.

References

- K. Zhang, L. Zhang, Z. Deng, C.-M. Tsai, H.-Y. Tseng, C.-Y. Chuang, C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, O. Chubach, Y.-W. Chen, Y.-W. Huang, S.-M. Lei, “EE2-1.6: Non-local cross-component prediction and cross-component merge mode”, JVET-AD0188, Apr. 2023.

- H.-Y. Tseng, C.-M. Tsai, C.-Y. Chuang, C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, O. Chubach, Y.-W. Chen, Y.-W. Huang, S.-M. Lei, “EE2-2.7: Cross-component merge mode with temporal candidates”, JVET-AE0043, July 2023.

- ECM-9.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-9.0.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).