| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0186 |
| --- | --- |

| Title: | Non-EE2-2.1: Block Vector Guided Chroma Direct Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser Fabrice Le Léannec Philippe Bordes Ya Chen Gagan Rath | Email: | karam.naser@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

Chroma direct mode (DM) is a special intra mode for chroma component where the same prediction mode of the collocated luma block is used for the current chroma block. In ECM-9.0, it is asserted that when the collocated luma block is coded with IBC or IntraTMP, DM is set to planar. This contribution proposes to use a method similar to EE2-2.1, where the collocated luma block vectors are used to obtain its reference block intra mode. The following results are obtained:

AI -0.03%	-0.15%	-0.10% with EncT 100% and DecT 100%

# Introduction

In EE2-2.1, a new mode is added, named block vector guided CCCM, to enhance the prediction when a collocated luma block is coded with IBC or IntraTMP. Specifically, CCCM parameters are obtained from block vector compensated luma block instead of the current collocated frame.

Apart from CCCM, chroma DM mode also considers information from luma to predict the chroma block. Accordingly, when the collocated luma block is coded with IBC or IntraTMP, the block vectors can “guide” the decoder to obtain the intra prediction mode from the block vector compensated luma block. This is more beneficial than using planar mode that does not take any information from the luma component.

# Proposal

When DM mode is used and the collocated luma block is IBC/IntraTMP coded, it is proposed to use the block vector to obtain the reference luma block and use its intra prediction mode.

# Experimental results

The proposal is implemented on ECM-9.0. The following results are obtained in using the ECM common testing conditions:

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.10% | 0.11% | 100% | 99% |
| Class A2 | -0.01% | -0.12% | -0.01% | 100% | 100% |
| Class B | -0.03% | 0.47% | -0.08% | 99% | 100% |
| Class C | -0.02% | -0.37% | 0.10% | 100% | 100% |
| Class E | -0.06% | -0.99% | -0.68% | 101% | 100% |
| Overall | -0.03% | -0.15% | -0.10% | 100% | 100% |
| Class D | 0.13% | -0.30% | 0.63% | 99% | 99% |
| Class F | -0.03% | -1.55% | -0.31% | 99% | 99% |
| Class TGM | -0.14% | -0.54% | -0.38% | 100% | 100% |

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).