| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0189-v1 |
| --- | --- |

| Title: | AHG9: On the design of nnpfa_target_base_flag in NNPFA SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry | Email: | dr.hendry@lge.com |
| Source: | LG electronics |  |  |

_____________________________

# Abstract

This contribution asserted the following problems:

- It is not clear whether base filter and update filter can be activated for the same picture. The current specification text is not clear about this. If base filter and update filter can be activated for the same picture, there is an asserted problem in determine which filter to be cancelled with NNPFA SEI message with the same nnpfa_target_id is present with nnpfa_cancel_flag is equal to 1 is present since nnpfa_target_base_flag is not present in such SEI message.

- When nnpfa_target_base_flag is equal to 0 but there is no NNPFC SEI message with nnpfc_id equal to nnpfa_target_id and nnpfc_base_flag equal to 0 that precedes the NNPFA SEI message in decoding order, it may be beneficial to disallow such case.

- The constraint regarding target pictures for NNPFC and for NNPFA in the semantics of NNPFA SEI message may cause conformance checking use incorrect set of pictures as the target NNPFC SEI message in the constraint definition does not consider the value of nnpfa_target_base_flag.

The following are proposed to overcome the asserted problems above:

- For asserted problem 1, the following options are proposed:

  - Option 1:

- Move the signalling of nnpfa_target_base_flag such that it is present without being conditioned on the value of nnpfa_cancel_flag.

- When the value of nnpfa_cancel_flag is equal to 1, it cancels the persistence of the target NNPF with that was previously activated with the same value of nnpfa_target_id and nnpfa_target_base_flag.

  - Option 2: Add a constraint such that there shall be no two (or more) NNPFA SEI message active (i.e, with nnpfa_cancel_flag equal to 0) for the same picture regardless their value of nnpfa_target_base_flag.

- For asserted problem 2, the following options are proposed:

  - Option 1: Add a constraint such that when the value of nnpfa_target_base_flag is equal to 0 in an NNPFA SEI message, there shall be at least one NNPFC SEI message with nnpfc_id equal to nnpfa_target_id and nnpfc_base_flag equal to 0 that precedes the NNPFA SEI message in decoding order.

  - Option 2: Add a note to describe that when nnpfa_target_base_flag is equal to 0, the target NNPF may be a base NNPF in the case where there is no NNPFC SEI message with nnpfc_id equal to nnpfa_target_id and nnpfc_base_flag is equal to 0 that precedes the NNPFA SEI message in decoding order.

- For asserted problem 3, it is proposed to update the constraint related to nnpfcTargetPictures and nnpfaTargetPictures by also consider the value of nnpfa_target_base_flag.

# On activation of base and update filter

## Problem description

In the 30th JVET meeting, it was agreed to adopt nnpfa_target_base_flag in neural-network post-filter activation (NNPFA) SEI message to allow the choice between activation of base filter or the latest (updated) filter. However, it is not clear whether base filter and update filter can be activated for the same picture. One may argue that base filter and its update are different filter, therefore, both can be activated for the same picture. On the other hand, one may also consider that they are the same filter so they cannot be active at the same time for the picture. This needs to be clarified.

If the intent is treat base and update filters as different filter, then the current signalling of nnpfa_target_base_flag is problematic in identifying which NNPFA activation needs to be cancelled when there are two activation of NNPFs of the same nnpfa_target_id but with different value of nnpfa_target_base_flag. In other words, when both base and update NNPF with the same nnpfc_id are activated for the same picture, it is unclear how to distinguish which persistence of these two will be cancelled when later a new NNPFA SEI message with the same nnpfa_target_id with nnpfa_cancel_flag equal to 1 is present since nnpfa_target_base_flag is not present when nnpfa_cancel_flag is equal to 1.

On the other hand, if the intent is to treat base and update filters as the same filter so that they cannot be active at the same time, the current specification misses constraint for that.

## Proposed solution

To solve the asserted problem above, the following is proposed:

Option 1:

- Move the signalling of nnpfa_target_base_flag such that it is present without being conditioned on the value of nnpfa_cancel_flag.

- When the value of nnpfa_cancel_flag is equal to 1, it cancels the persistence of the target NNPF with that was previously activated with the same value of nnpfa_target_id and nnpfa_target_base_flag.

Option 2:

- Add a constraint such that there shall be no two (or more) NNPFA SEI message active (i.e, with nnpfa_cancel_flag equal to 0) for the same picture regardless their value of nnpfa_target_base_flag.

## Text implementation

Text for option 1:

| nn_post_filter_activation( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfa_target_id | ue(v) |
| nnpfa_target_base_flag | u(1) |
| nnpfa_cancel_flag | u(1) |
| if( !nnpfa_cancel_flag ) { |  |
| nnpfa_target_base_flag | u(1) |
| nnpfa_persistence_flag | u(1) |
| nnpfa_num_output_entries | ue(v) |
| for( i = 0; i < nnpfa_num_output_entries; i++ ) |  |
| nnpfa_output_flag[ i ] | u(1) |
| } |  |
| } |  |

…

The neural-network post-filter activation (NNPFA) SEI message activates or de-activates the possible use of the target neural-network post-processing filter (NNPF), identified by nnpfa_target_id and nnpfa_target_base_flag, for post-processing filtering of a set of pictures. For a particular picture for which the NNPF is activated, the target NNPF is derived as follows:

–	If nnpfa_target_base_flag is equal to 1, the target NNPF is the base NNPF with nnpfc_id equal to nnpfa_target_id.

–	Otherwise (nnpfa_target_base_flag is equal to 0), the target NNPF is the NNPF specified by the last NNPFC SEI message with nnpfc_id equal to nnpfa_target_id that precedes the first VCL NAL unit of the current picture in decoding order and is not a repetition of the NNPFC SEI message that contains the base NNPF.

NOTE 1 – There can be several NNPFA SEI messages present for the same picture, for example, when the NNPFs are meant for different purposes or for filtering of different colour components.

nnpfa_target_id indicates the target NNPF, which is specified by one or more NNPFC SEI messages that pertain to the current picture and have nnpfc_id equal to nnpfa_target_id. The value of nnpfa_target_id shall be in the range of 0 to 232 − 2, inclusive.

An NNPFA SEI message with a particular value of nnpfa_target_id shall not be present in a current PU unless one or both of the following conditions are true:

–	Within the current CLVS there is an NNPFC SEI message with nnpfc_id equal to the particular value of nnpfa_target_id present in a PU preceding the current PU in decoding order.

–	There is an NNPFC SEI message with nnpfc_id equal to the particular value of nnpfa_target_id in the current PU.

When a PU contains both an NNPFC SEI message with a particular value of nnpfc_id and an NNPFA SEI message with nnpfa_target_id equal to the particular value of nnpfc_id, the NNPFC SEI message shall precede the NNPFA SEI message in decoding order.

nnpfa_target_base_flag equal to 1 specifies that the target NNPF is the base NNPF with nnpfc_id equal to nnpfa_target_id. nnpfa_target_base_flag equal to 0 specifies that the target NNPF is the NNPF specified by the last NNPFC SEI message with nnpfc_id equal to nnpfa_target_id that precedes the first VCL NAL unit of the current picture in decoding order and is not a repetition of the NNPFC SEI message that contains the base NNPF.

nnpfa_cancel_flag equal to 1 indicates that the persistence of the target NNPF established by any previous NNPFA SEI message with the same nnpfa_target_id and nnpfa_target_base_flag as the current SEI message is cancelled, i.e., the target NNPF is no longer used unless it is activated by another NNPFA SEI message with the same nnpfa_target_id and nnpfa_target_base_flag as the current SEI message and nnpfa_cancel_flag equal to 0. nnpfa_cancel_flag equal to 0 indicates that the nnpfa_target_base_flag, nnpfa_persistence_flag, and nnpfa_num_output_entries follow.

nnpfa_target_base_flag equal to 1 specifies that the target NNPF is the base NNPF with nnpfc_id equal to nnpfa_target_id. nnpfa_target_base_flag equal to 0 specifies that the target NNPF is the NNPF specified by the last NNPFC SEI message with nnpfc_id equal to nnpfa_target_id that precedes the first VCL NAL unit of the current picture in decoding order and is not a repetition of the NNPFC SEI message that contains the base NNPF.

nnpfa_persistence_flag specifies the persistence of the target NNPF for the current layer.

nnpfa_persistence_flag equal to 0 specifies that the target NNPF may be used for post-processing filtering for the current picture only.

nnpfa_persistence_flag equal to 1 specifies that the target NNPF may be used for post-processing filtering for the current picture and all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer associated with a NNPFA SEI message with the same nnpfa_target_id and nnpfa_target_base_flag as the current SEI message and nnpfa_cancel_flag equal to 1 is output that follows the current picture in output order.

NOTE 2 – The target NNPF is not applied for this subsequent picture in the current layer associated with a NNPFA SEI message with the same nnpfa_target_id and nnpfa_target_base_flag as the current SEI message and nnpfa_cancel_flag equal to 1.

Let the nnpfcTargetPictures be the set of pictures to which the last NNPFC SEI message with nnpfc_id equal to nnpfa_target_id that precedes the current NNPFA SEI message in decoding order pertains. Let nnpfaTargetPictures be the set of pictures for which the target NNPF is activated by the current NNPFA SEI message. It is a requirement of bitstream conformance that any picture included in nnpfaTargetPictures shall also be included in nnpfcTargetPictures.

…

Text for option 2:

…

nnpfa_target_base_flag equal to 1 specifies that the target NNPF is the base NNPF with nnpfc_id equal to nnpfa_target_id. nnpfa_target_base_flag equal to 0 specifies that the target NNPF is the NNPF specified by the last NNPFC SEI message with nnpfc_id equal to nnpfa_target_id that precedes the first VCL NAL unit of the current picture in decoding order and is not a repetition of the NNPFC SEI message that contains the base NNPF.

It is constrained that there shall be no two NNPFA SEI messages active (i.e, with nnpfa_cancel_flag equal to 0) for the same picture regardless their value of nnpfa_target_base_flag.

nnpfa_persistence_flag specifies the persistence of the target NNPF for the current layer.

…

# On the value of nnpfa_target_base_flag

## Problem description

The following is specified regarding the target NNPF associated with an NNPFA SEI message:

For a particular picture for which the NNPF is activated, the target NNPF is derived as follows:

–	If nnpfa_target_base_flag is equal to 1, the target NNPF is the base NNPF with nnpfc_id equal to nnpfa_target_id.

–	Otherwise (nnpfa_target_base_flag is equal to 0), the target NNPF is the NNPF specified by the last NNPFC SEI message with nnpfc_id equal to nnpfa_target_id that precedes the first VCL NAL unit of the current picture in decoding order and is not a repetition of the NNPFC SEI message that contains the base NNPF.

NOTE 1 – There can be several NNPFA SEI messages present for the same picture, for example, when the NNPFs are meant for different purposes or for filtering of different colour components.

When nnpfa_target_base_flag is equal to 0 but there is no NNPFC SEI message with nnpfc_id equal to nnpfa_target_id and nnpfc_base_flag equal to 0 that precedes the NNPFA SEI message in decoding order, the current design will allow fall back mechanism such that the base NNPF is assigned to be the target NNPF in such situation. However, it may be argued that having a stricter design for the value of nnpfa_target_base_flag (i.e., when nnpfa_target_base_flag is equal to 0, there is an update to the base filter present that precedes the NNPFA SEI message) would be easier for implementation since it is straightforward case: if nnpfa_target_base_flag is equal to 1, take the first filter of that id with nnpfc_base_flag equal to 1, if not, that the last one with nnpfc_base_flag equal to 0.

## Proposed solution

To solve the asserted problem above, the following is proposed:

Option 1: Add a constraint such that when the value of nnpfa_target_base_flag is equal to 0 in an NNPFA SEI message, there shall be at least one NNPFC SEI message with nnpfc_id equal to nnpfa_target_id and nnpfc_base_flag equal to 0 that precedes the NNPFA SEI message in decoding order

Option 2: Add a note to describe that when nnpfa_target_base_flag is equal to 0, the target NNPF may be a base NNPF in the case where there is no NNPFC SEI message with nnpfc_id equal to nnpfa_target_id and nnpfc_base_flag is equal to 0 that precedes the NNPFA SEI message in decoding order.

## Text implementation

Text for option 1:

…

nnpfa_target_base_flag equal to 1 specifies that the target NNPF is the base NNPF with nnpfc_id equal to nnpfa_target_id. nnpfa_target_base_flag equal to 0 specifies that the target NNPF is the NNPF specified by the last NNPFC SEI message with nnpfc_id equal to nnpfa_target_id that precedes the first VCL NAL unit of the current picture in decoding order and is not a repetition of the NNPFC SEI message that contains the base NNPF. When the value of nnpfa_target_base_flag is equal to 0, there shall be at least one NNPFC SEI message with nnpfc_id equal to nnpfa_target_id and nnpfc_base_flag equal to 0 that precedes the NNPFA SEI message in decoding order.

…

Text for option 2:

…

nnpfa_target_base_flag equal to 1 specifies that the target NNPF is the base NNPF with nnpfc_id equal to nnpfa_target_id. nnpfa_target_base_flag equal to 0 specifies that the target NNPF is the NNPF specified by the last NNPFC SEI message with nnpfc_id equal to nnpfa_target_id that precedes the first VCL NAL unit of the current picture in decoding order and is not a repetition of the NNPFC SEI message that contains the base NNPF.

Note: When nnpfa_target_base_flag is equal to 0, the target NNPF may be a base NNPF in the case where there is no NNPFC SEI message with nnpfc_id equal to nnpfa_target_id and nnpfc_base_flag is equal to 0 that precedes the NNPFA SEI message in decoding order.

…

# On the value of nnpfa_target_base_flag

## Problem description

It is asserted the following constraint in the semantics of NNPFA SEI message regarding the target pictures for NNPFA and NNPFC SEI messages can be made precise by also considering the value of nnpfa_target_base_flag.

Let the nnpfcTargetPictures be the set of pictures to which the last NNPFC SEI message with nnpfc_id equal to nnpfa_target_id that precedes the current NNPFA SEI message in decoding order pertains. Let nnpfaTargetPictures be the set of pictures for which the target NNPF is activated by the current NNPFA SEI message. It is a requirement of bitstream conformance that any picture included in nnpfaTargetPictures shall also be included in nnpfcTargetPictures.

Note that with the above constraint, nnpfcTargetPictures may be the set of pictures that is not really associated with the target NNPFC SEI message. This may base conformance check uses incorrect set of target pictures.

## Proposed solution and text

It is proposed to update the constraint above as follows:

Let the nnpfcTargetPictures be the set of pictures to which the NNPFC SEI message corresponds to the target NNPF (i.e., if the value of nnpfa_target_base_flag is equal to 1, the NNPFC SEI message with nnpfc_id equal to nnpfa_target_id and nnpfc_base_flag equal to 1, otherwise, the last NNPFC SEI message with nnpfc_id equal to nnpfa_target_id and nnpfc_base_flag equal to 0 that precedes the current NNPFA SEI message in decoding order pertains). Let nnpfaTargetPictures be the set of pictures for which the target NNPF is activated by the current NNPFA SEI message. It is a requirement of bitstream conformance that any picture included in nnpfaTargetPictures shall also be included in nnpfcTargetPictures.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).