| Pareil Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0195-v1 |
| --- | --- |

| Title: | AhG7: ECM stages complexity assessment |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | F. Galpin, F. Leleannec, C. Salmon-Legagneur, E. François | Tel: Email: | franck.galpin@interdigital.com |
| Source: | InterDigital |  |  |

# Abstract

This contribution contains information about the current complexity of ECM-9.0 on some sample sequences in comparison with previously developed reference software. The contribution presents a more detailed view of the complexity evolution by showing the complexity at different stages, namely “parsing” (decoding the bins), “decoding” (computing the values of a CU), “prediction” (computing the prediction), “residuals” (computing the residuals), “loop-filters” (dbf, sao, alf etc.). Results are shown on HM-16.10, JEM-7.0, VTM 19.2 and ECM 9.0.

# Introduction

The work on enhanced compression beyond VVC capability is performed inside the ECM software. Several complexity assessment information are already available, typically the runtime impact compared to the VTM software, as well as the AhG7 report on ECM tool assessment (JVET-AD0007).

In this contribution, it is proposed to show a more detailed view of the complexity evolution by showing the complexity at different stages, namely “parsing”, “decoding”, “prediction”, “residuals”, “loop-filters”.

# Experiments protocol

## Decoding stages

Figure 1 Example of decoding pipeline

The above figure shows an example of pipeline (typically VVC decoding) where:

- Parsing is done serially.

- Decoding of a block is performed after the parsing, with some part being done potentially in parallel.

- After decoding, the prediction and residuals are computed, where all processing of blocks can be done in parallel.

- After a block is reconstructed filtering can take place.

Reference software are not completely following this pipeline, but a rough estimate can be made by splitting the decoding process at some key location in the software.

In details, parsing stage sometimes contains decoding process, and the independence of the reconstruction process cannot be assessed since all blocks are processed one by one.

This is especially true for ECM since decoding and reconstruction are intricated and inter-dependent when using the TM methods.

## Complexity assessment method

In order to assess the complexity of the different stages inside the software, the performance counter monitor, which is a hardware feature available on some CPU, is used. To assess the complexity, several counters are available. We choose to use the instructions counter, with the following properties:

- Pros: invariant to the CPU speed or environment noise

- Cons: dependent on the compiler. To mitigate this drawback, all pieces of software are compiled using the same compiler and options.

- Cons: dependent on the implementation (same as the run-time assessment). The bias is especially larger for SIMD/scalar code.

- Cons: does not take into account the instruction complexity (latency, throughput…) nor the cache misses issues.

- Gives a rough estimate of the algorithmic complexity. However, everything is mixed: memory access, computation, control etc.

Only bitstream decoding is performed and the same sequence and QP are used for all reference software.

Another bias is coming from the level of optimization (typically SIMD code) in each software. Recent software tends to have more SIMD optimization than former ones, hiding a bit the real algorithmic complexity. To mitigate this issue, a normalization factor is applied on the complexity values based on runtime/instruction ratio.

## Assessed stages

The following stages are assessed:

- The whole process. Used to validate the complexity assessment w.r.t. the runtime.

- Parsing stage: e.g., computing context, decoding bins

- Decoding stage: e.g., compute mode values, motion vectors, residual coefficients etc.

- Prediction computation: e.g., create the intra prediction, performs the motion computation etc.

- Residual computation: e.g., perform inverse transform.

- Loop-filtering: deblocking, ALF etc.

Complexity related to I/O, HLS decoding, initialization, deinitialization etc. is thus not counted.

## Assessed software

The following software are assessed:

- HM 16.10 – no SIMD

- JEM 7.0 –few SIMD

- VTM 19.2 – both scalar and AVX2

- ECM 9.0 – only AVX2 (*)

- ECM-9.0 TM off (group 1) – only AVX2

(*) Unfortunately, SCALAR mode is broken in ECM software and fair SIMD comparison cannot be done with other software.

In our tests, we observed a x2-x3 increase in number of instructions mainly for stages “prediction” and “loop filter” between AVX2 and SCALAR mode.

Finally, all tests are performed assuming AVX2 mode (i.e., JEM/HM complexity is slightly overestimated compared to other software).

# Results

## Test 1

This test is on HD BQTerrace sequence, encoded at QP=32 in Random Access configuration.

### Performance

The table below shows the performance of each reference software.

| Software | PSNR Y – U – V (dB) | Rate (kb/s) |
| --- | --- | --- |
| HM | 33.87 – 39.76 – 42.29 | 2000.97 |
| JEM | 34.26 – 40.44 – 43.08 | 1793.02 |
| VTM | 34.41 – 40.88 – 43.48 | 1467.99 |
| ECM | 34.67 – 41.44 – 43.91 | 1318.03 |
| ECM no TM | 34.64 – 41.44 – 43.90 | 1335.05 |

### Overall complexity

| Software | Run time (s) | Nb Ginstr. | Ratio instr./time |
| --- | --- | --- | --- |
| HM | 23.9 | 162.45 | 6.8 |
| JEM | 160.6 | 1557 | 9.7 |
| VTM | 36.2 | 271.58 | 7.5 |
| ECM | 293 | 2432.15 | 8.3 |
| ECM no TM | 255.8 | 2153.34 | 8.4 |

The table above shows the overall complexity (total number of instructions during a decoding) and the decoding runtime. The 2 measures are well correlated except for the HM/JEM. Note that both HM and JEM have no/few SIMD instructions.

### SIMD bias

|  | scalar | avx2 | ratio |
| --- | --- | --- | --- |
| parsing | 9.6 | 9.6 | 1.0 |
| decoding | 5.0 | 5.0 | 1.0 |
| prediction | 255.5 | 104.6 | 2.4 |
| residual | 4.7 | 4.7 | 1.0 |
| LF | 327.7 | 92.0 | 3.6 |
| other | 371.1 | 135.3 | 2.7 |
| dec time | 165.4 | 36.2 | 4.6 |

The table above shows the difference scalar and SIMD runs. Overall, the scalar code gives ~2.5x more instructions to execute, with some stages without impact (e.g. parsing does not contain SIMD code) or larger impact (LF contains more SIMD).

### Complexity per stage

|  | parsing | decoding | prediction | residual | LF | other |
| --- | --- | --- | --- | --- | --- | --- |
| hm | 7.4 | 2.6 | 75.7 | 5.5 | 59.2 | 69.4 |
| jem | 7.2 | 264.8 | 702.1 | 8.5 | 485.6 | 540.7 |
| vtm | 9.6 | 5.0 | 104.6 | 4.7 | 92.0 | 135.3 |
| ecm | 14.2 | 563.4 | 876.6 | 71.9 | 828.5 | 889.1 |
| ecm no tm | 13.8 | 190.9 | 947.8 | 71.5 | 851.4 | 912.1 |

The table above shows the number of instructions (in billion) for each stage.

First, we can see that parsing is quite stable.

The decoding was also stable between HM and VTM but became very complex in ECM (about x100 complexity increase).

Prediction and LF complexity also increase by a smaller factor (about x8) while residual increased more (about x15).

The figures below show the complexity ratio during the decoding. As expected, decoding part increased more for ECM.

The chart above shows the evolution of the complexity across the software.

As a reminder, HM and JEM complexity for decoding/prediction/LF stages are greatly over-estimated (about 2.5 times) because of the lack of SIMD code.

## Test 2

xxx

# References

JVET-AE0007 “JVET AHG report: ECM tool assessment (AHG7) “, X. Li (chair), L.-F. Chen, Z. Deng, J. Gan, E. François, H.-J. Jhu, X. Li, H. Wang (vice-chairs)

# Patent rights declaration(s)

InterDigital does not have any current or pending patent rights relating to the technology described in this contribution.