| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0198 |
| --- | --- |

| Title: | AHG12: Extended Search Region for IntraTMP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser Philippe Bordes Fabrice Le Léannec Franck Galpin | Email: | karam.naser@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes an extension to the search range of IntraTMP tools. The extended search range is the same one as of IBC mode. It is based on adding new search region with specific subsampling factor to control the number of added SAD checks.

On top of ECM-9.0, simulation results are reported as follows:

AI:

CTC: 		-0.03%	0.24%	0.02%	102%	103%

classF: 		-0.40%	-1.66%	-0.28%	102%	103%

ClassTGM:	-0.99%	-1.38%	-1.20%	103%	104%

# Introduction

In IntraTMP mode, four search regions are considered for performing the template matching process to find the best block vectors. The regions are (shown in Figure 1):

R1: current CTU

R2: top-left CTU

R3: above CTU

R4: left CTU

Figure 1: IntraTMP search regions in ECM-9.0

The search range of all search regions is subsampled by a factor of 3. After finding the best match, a refinement process is performed. The refinement is done via a second template matching search around the best match with a reduced range.

IntraTMP is similar to IBC in the sense that both use block vectors referring for reference samples located in the reconstructed area in the current frame. However, it is observed that IBC uses larger search area than IntraTMP. Specifically, it uses up to two lines of CTU’s above the current one. The main reason for considering smaller search range for IntraTMP is its associated decoder complexity due to the excessive computation cost associated with its TM process.

For screen contents (classF and TGM), it is observed that more prediction candidates from far areas are particularly useful. For natural contents, however, they are usually locally correlated and it is not significantly beneficial to increase IntraTMP search range.

Figure 2 IBC search area in ECM-9.0

# Proposal

This contribution proposes increasing the search range of IntraTMP to use the same one as IBC. It is proposed to add further search regions to IntraTMP to cover IBC range. The new regions are shown in the figure below:

Figure 3: New proposed search regions (in orange) for IntraTMP

Each search region is subsampled by the following factors

subSamplingFactorWidth_i = RiWidth/ a*BlkW

subSamplingFactorHeight_i = RiHeight/ a*BlkH

where “a” is the ECM constant that controls the gain/complexity trade-off. Thus, the new subsampling factors take into account each region dimensions. Therefore, the added number of template comparisons remains small for all the added regions.

# Experimental results

The proposal is implemented on ECM-9.0. The following results are obtained in using the ECM common testing conditions:

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.29% | -0.36% | 101% | 103% |
| Class A2 | -0.02% | 0.53% | -0.06% | 102% | 103% |
| Class B | -0.07% | 0.35% | -0.14% | 103% | 103% |
| Class C | -0.01% | -0.11% | -0.22% | 102% | 104% |
| Class E | -0.02% | 0.75% | 1.07% | 102% | 104% |
| Overall | -0.03% | 0.24% | 0.02% | 102% | 103% |
| Class D | 0.18% | -0.91% | 1.16% | 103% | 103% |
| Class F | -0.40% | -1.66% | -0.28% | 102% | 103% |
| Class TGM | -0.99% | -1.38% | -1.20% | 103% | 104% |

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).