| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0199 |
| --- | --- |

| Title: | Crosscheck of JVET-AD0064 (AHG12: Enhanced geometrical prediction mode) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yongjo Ahn 19 Yangjaedaero-11 gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +82-2-6912-6561 yongjo.ahn@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This document reports the crosscheck results for EE2-3.2 (JVET-AE0046) proposal on EE2-3.2: Bi-predictive GPM. The EE test proposes bi-predictive geometric partitioning mode with additional modifications for the current ECM tools.

Introduction

EE2-3.2 (JVET-AE0046) proposes four elements to extend the current geometric partitioning mode (GPM) design in ECM as follows:

- Conditionally invoking the extraction process that extracts uni-predictive motion vectors from the initial list

  - The extraction process is invoked only for small blocks 8x8, 16x8 and 8x16.

  - For other larger blocks, the extraction process is bypassed. It means that bi-predictive motion vectors are available for each GPM partition.

- Modifications GPM-MMVD to support bi-predictive motion vector as the base vector

- Modifications of GPM-TM to also support bi-predictive motion vectors

- Enabling 8x8 BDOF (i.e., as in the existing design of multi-pass DMVR) on top of the associated bi-predictive motion vectors for each GPM partition

Crosscheck results

In this report, simulations of ECM-9.0 [2] and EE2-3.2 test are crosschecked under random access (RA) and low-delay B (LDB) common test configurations [3]. The crosscheck results are as follows:

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.46% | -0.09% | 101.4% | 99.9% |
| Class A2 | -0.27% | -0.30% | -0.26% | 101.6% | 99.3% |
| Class B | -0.20% | -0.23% | -0.32% | 101.3% | 99.3% |
| Class C | -0.19% | -0.33% | -0.14% | 101.3% | 99.6% |
| Class E |  |  |  |  |  |
| Overall | -0.20% | -0.32% | -0.21% | 101.4% | 99.5% |
| Class D | -0.13% | -0.06% | -0.16% | 101.6% | 100.0% |
| Class F | -0.08% | -0.22% | -0.17% | 99.5% | 98.6% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.22%* | -0.58%* | -0.38%* | 101.6%* | 101.3%* |
| Class C | -0.23% | -0.68% | -0.58% | 102.1% | 101.0% |
| Class E | -0.17% | -0.01% | -0.46% | 101.7% | 100.6% |
| Overall | -0.21% | -0.47% | -0.46% | 101.8% | 101.0% |
| Class D | -0.06% | -0.05% | 0.10% | 102.3% | 102.3% |
| Class F | -0.23% | -0.49% | -0.28% | 100.5% | 100.2% |

Note (*): In LDB, the bd-rate results are estimated by copying anchor results for BQTerrace at QP22 (due to simulation still running).

Conclusion

It is reported that the crosscheck results matched what was provided by the proponent of EE2-3.2 (JVET-AE0046).

References

- R. Yu, P. Wennersten, J. Enhorn, K. Andersson, “EE2-3.2: Bi-predictive GPM,” JVET-AE0046, 31st Meeting, Geneva, CH, 11–19 July 2023.

- ECM-9.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-9.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AD2017, 30th Meeting, Antalya, TR, 21–28 April 2023.