| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0219-v1 |
| --- | --- |

| Title: | AHG4: Results of visual checking of SVVC VT streams with and without DMVR fix |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | M. Wien K. Andersson P. de Lagrange | Email: | wien@lfb.rwth-aachen.de kenneth.r.andersson@ericsson.com philippe.delagrange@interdigital.com |
| Source: | RWTH Aachen University, Ericsson, InterDigital |  |  |

_____________________________

# Abstract

# Introduction

This document reports observations made on bitstreams prepared for the verification tests for dual-layer VVC coding comparing single-layer to a spatially scalable representation with 1.5× and 2× scaling ratio between base and enhancement layer.

The bitstreams were encoded using VTM at commit “b6a20aa7d1” (interim status after VTM-20.2) by Ericsson and exchanged via the JVET ftp site. The encoder configuration follows the Verification Test plan document JVET-AD2021 [1] with the DMVR encoder parameter DMVREncMvSelect switched on and switched off, respectively.

The goal of the visual checking is to inspect the reconstructed video for the visual impact and potential issues with the modified encoder configuration in both, the single-layer and the dual-layer configurations.

# Visual checking

Visual checking was performed on the bitstreams with highest QP for each sequence by the test coordinator. This rate point was selected since the DMVREncMvSelect modification reportedly shows the most significant difference at low bitrates. The reconstructed videos for all test sequences using the single layer, 1.5×, and 2× configurations were assessed. The viewing included successive presentation of the two versions under consideration for assessment of the overall impression under regular viewing conditions. Further inspections used side-by-side and difference viewing in the YUView player.

## Observations

In general, the reconstructed video sequences resulting from DMVREncMvSelect turned on or off are different for all inter coded pictures. This is due to different encoder decisions resulting from the encoder configuration with and without this parameter activated. When activated, DMVREncMvSelect prevents the occurrence of isolated block artifacts which may be strongly visible and potentially annoying. It is noted that these artifacts have been observed for single-layer as well as for multi-layer coding. Besides this aspect, both variants with and without activation of DMVREncMvSelect generally show a comparable visual quality with comparable artifacts visible in different areas. In some cases, one version may preserve a detail better than the other, but a general benefit was not observed.

In the following, example snapshots are shown illustrating the visual impact of the change. Most of the figures show areas where the DMVR artifact is visible in the reconstructed VTM stream with DMVREncMvSelect not active.

Figure 1 CorsairJoy 1.5× spatial scalability, QPEL=44, frame 210. Left: VTM, right: VTM with DMVREncMvSelect=1

Figure 2 DrivingPOV3 single layer, QP=46, frame 243. Left: VTM, right: VTM with DMVREncMvSelect=1

Figure 2 DrivingPOV3 1.5× spatial scalability, QPEL=47, frame 163. Left: VTM, right: VTM with DMVREncMvSelect=1

Figure 3 DrivingPOV3 2× spatial scalability, QPEL=47, frame 136. Left: VTM, right: VTM with DMVREncMvSelect=1

Figure 4 Procession 1.5× spatial scalability, QPEL=48, frame 239. Left: VTM, right: VTM with DMVREncMvSelect=1

Figure 5 Procession 2× spatial scalability, QPEL=48, frame 58. Left: VTM, right: VTM with DMVREncMvSelect=1

Figure 6 TallBuildings2 1.5× spatial scalability, QPEL=47, frame 35. Left: VTM, right: VTM with DMVREncMvSelect=1

Figure 7 TallBuildings2 2× spatial scalability, QPEL=47, frame 9. Left: VTM, right: VTM with DMVREncMvSelect=1

# RD results

Bjontegaard delta rate numbers and rate-distortion plots are provided for DMVREncMvSelect switched on and switched off. In general, an increase in performance loss is observed for increasing QP while the loss vanishes for low QPs. E.g., at the lowest QP for MountainBay2, the RD points for both variants are identical.

Table 1: Bjontegaard delta rate numbers for DMVREncMvSelect=1.

| BD rate difference [%] | Single layer | 1.5× | 2× |
| --- | --- | --- | --- |
| CorsaireJoy | +1.09 | +0.57 | +0.31 |
| DrivingPOV3 | +1.41 | +0.57 | +0.22 |
| Fortnite | +0.36 | +0.03 | +0.03 |
| Marathon2 | +0.82 | +0.39 | +0.14 |
| MountainBay2 | +0.61 | +0.27 | +0.18 |
| Procession | +0.99 | +0.56 | +0.32 |
| Tallbuildings2 | +0.08 | +0.58 | +0.34 |
| AVG | +0.77 | +0.42 | +0.22 |

## RD plots for CorsairJoy

## RD plots for DrivingPOV3

## RD plots for Fortnite

## RD plots for Marathon2

## RD plots for MountainBay2

## RD plots for Procession

## RD plots for TallBuildings2

# Conclusions

When activated, DMVREncMvSelect prevents the occurrence of isolated block artifacts which may be strongly visible and potentially annoying. It is noted that these artifacts have been observed for single-layer as well as for multi-layer coding. Besides this aspect, both variants, with and without activation of DMVREncMvSelect, generally show a comparable visual quality with comparable artifacts visible in different areas. In some cases, one version may preserve a detail better than the other, but a general benefit was not observed.

# References

- S. Iwamura, P. de Lagrange, M. Wien, “Verification test plan for VVC multilayer coding,” Doc. JVET-AD2021, Antalya, Apr. 2023.