| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0240-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AE0094 (EE2-2.6c: IBC with non-adjacent spatial candidates) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jie Chen | Tel: Email: | jiechen.cj@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

This contribution is a crosscheck report of CE2-2.6c which is reported in JVET-AE0094[1]. It is confirmed that the results match with the results reported in JVET-AE0094.

Introduction

In EE2-2.6c, it is proposed to insert several new candidates, which are obtained from the BVs of non-adjacent spatial neighboring blocks, into the candidate lists of IBC merge modes and IBC AMVP. As shown in Fig. 1, similar grid pattern as that being used for the non-adjacent spatial candidates of regular inter is used to obtain non-adjacent BVs. The obtained BVs are inserted between the adjacent spatial candidates and the HBVP candidates for both IBC merge and IBC AMVP.

The test was checked in this contribution.

Crosscheck results

The tests are conducted under CTC[2] and results are confirmed to be identical to those reported in JVET-AE0094.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.03% | 0.02% | 106.8% | 100.1% |
| Class A2 | -0.01% | -0.11% | -0.01% | 104.9% | 99.9% |
| Class B | 0.00% | 0.06% | 0.06% | 101.0% | 101.1% |
| Class C | -0.01% | -0.05% | 0.00% | 101.0% | 100.8% |
| Class E | -0.02% | 0.05% | 0.01% | 98.2% | 98.2% |
| Overall | 0.00% | 0.00% | 0.02% | 102.1% | 100.2% |
| Class D | 0.03% | -0.18% | -0.11% | 101.8% | 101.9% |
| Class F | -0.45% | -0.22% | -0.26% | 97.9% | 99.6% |
| Class TGM |  |  |  |  |  |

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |  |  |
|  | Y | U |  | V | EncT |  | DecT |
| Class A1 | 0.00% | -0.06% |  | -0.01% | 100.5% |  | 98.6% |
| Class A2 | -0.03% | -0.16% |  | -0.04% | 101.1% |  | 98.4% |
| Class B | -0.01% | -0.08% |  | 0.07% | 100.0% |  | 100.2% |
| Class C | 0.00% | -0.19% |  | 0.04% | 97.3% |  | 97.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.01% | -0.12% |  | 0.02% | 99.6% |  | 98.7% |
| Class D | 0.03% | -0.14% |  | 0.07% | 99.1% |  | 98.7% |
| Class F | -0.25% | -0.23% |  | -0.27% | 97.8% |  | 98.5% |
| Class TGM |  |  |  |  |  |  |  |

Conclusion

This contribution is a crosscheck report of EE2-2.6c. It is confirmed that the results match with the results reported in JVET-AE0094.

References

C. Ma, X. Xiu, W. Chen, H.-H.Jhu, C.-W. Kuo, N. Yan, X. Wang (Kwai), Y. Wang, K. Zhang, L. Zhang (Bytedance), “EE2-2.6: IBC with non-adjacent spatial candidates ”, JVET-AE0094, July. 2023

M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” JVET-AD2017, April 2023.