| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0255-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AE0160 (EE1-1.5: Optimization for complexity-performance trade-off of HOP network) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Du Liu | Email: | du.liu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution reports the inference crosscheck results for JVET-AE0160, including EE1-1.5.1 subtests 2 and 3 and EE1-1.5.2 subtest2. It is reported that the available crosscheck results can match the results in JVET-AE0160.

# Introduction

JVET-AE0160 EE1-1.5 studies two kinds of modifications of the HOP structure. In Test 1.5.1, the residual block is modified to use depthwise separable convolution and group convolution. In Test 1.5.2, the luma and chroma blocks are split into two branches. The training uses filterset 0 framework and the inference test modifies the HOP interface accordingly.

All the tested models were trained in JVET-AE0160, and this report crosschecked the inference of the models.

# Results

The simulations were run with NnlfOption=4 and NnIntraPred=1. The compiler is GCC 8.4.0. Results are on top of NNVC-5.0 anchor (NN Intra and LOP enabled).

## Test 1.5.1 subtest2 int16

Class C and Class D were crosschecked.

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-5.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -4.68% | -15.08% | -14.78% | 117% | 553% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -4.96% | -13.85% | -14.85% | 118% | 499% |
| Class F |  |  |  |  |  |

|  | All intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-5.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -3.07% | -11.08% | -13.46% | 88% | 430% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -2.49% | -10.96% | -13.49% | 86% | 359% |
| Class F |  |  |  |  |  |

Encoding time and decoding time are not reliable since these were not run on the same machine.

## Test 1.5.1 subtest3 int16

Class C and Class D were crosschecked.

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-5.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -4.90% | -14.42% | -14.61% | 126% | 703% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -5.24% | -13.54% | -15.05% | 127% | 630% |
| Class F |  |  |  |  |  |

|  | All intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-5.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -3.17% | -10.57% | -13.14% | 90% | 533% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -2.60% | -10.50% | -13.04% | 87% | 446% |
| Class F |  |  |  |  |  |

Encoding time and decoding time are not reliable since these were not run on the same machine.

## Test 1.5.2 subtest2 int16

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-5.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -2.74% | -10.98% | -12.96% | 244% | 2107% |
| Class A2 |  |  |  |  |  |
| Class B | -3.02% | -12.45% | -12.85% | 242% | 2222% |
| Class C | -2.77% | -10.19% | -10.40% | 193% | 1915% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -3.32% | -11.38% | -13.06% | 195% | 1631% |
| Class F | -1.37% | -6.23% | -6.70% | 295% | 556% |

|  | All intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-5.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -1.72% | -11.20% | -13.26% | 178% | 1893% |
| Class A2 | -2.29% | -10.59% | -8.25% | 137% | 1953% |
| Class B | -1.73% | -10.47% | -10.60% | 122% | 1700% |
| Class C | -1.73% | -8.40% | -11.02% | 103% | 1282% |
| Class E | -2.58% | -12.89% | -15.04% | 130% | 1604% |
| Overall | -1.96% | -10.56% | -11.48% | 129% | 1648% |
| Class D | -0.92% | -9.25% | -11.90% | 98% | 1068% |
| Class F | -0.79% | -7.62% | -8.30% | 109% | 1168% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-5.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -2.77% | -3.31% | -2.58% | 196% | 1702% |
| Class E | -3.71% | 1.66% | 8.19% | 367% | 717% |
| Overall |  |  |  |  |  |
| Class D | -3.16% | 2.86% | -1.95% | 176% | 1603% |
| Class F |  |  |  |  |  |

Encoding time and decoding time are not reliable since these were not run on the same machine.

# References

[1] EE1-1.5: Optimization for complexity-performance trade-off of HOP network, JVET-AE0160, R. Chang, L. Wang, X. Xu, S. Liu, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 31st Meeting, Geneva, CH, 11–19 July 2023.

# Patent rights declaration(s)

Ericsson does not have any current or pending patent rights relating to the technology described in this contribution.