| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0257_v0 |
| --- | --- |

| Title: | Non-EE2: Peak Memory Reduction for ECM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenbin Yin1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | yinwenbin.hit@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

Abstract

In the current design of adaptive loop filter (ALF) and cross-component adaptive loop filter (CCALF) in ECM-9.0, the filtering unit size is set equal to the CTU size. The calculation of co-variance matrix for each CTU requests is a high memory cost at encoder. In this contribution, it is proposed to allow the filtering unit size larger than the CTU size. By doubling the dimensions of the filtering unit for 4K sequences in RA conditions, the peak required total memory at encoder is reduced from 23.2 GB to 19.6 GB, reportedly. Besides, the encoding time is reportedly reduced by 1~2% since the encoder requires less training rounds and determinations for ALF and CCALF.

On top of ECM-9.0, simulation results of the proposed method are reported as below:

RA: %, %, %, %, %.

Introduction

In ALF and CCALF of ECM-9.0, the filtering unit size is set equal to the CTU size. The encoder needs to build up the co-variance matrix for each CTU in Luma-ALF, Chroma-ALF and CCALF respectively. These co-variance matrixes require a high memory cost at encoder side. The peak required total memory is up to 24 GB for 4K sequences at encoder, under the RA conditions.

Proposed Method

To reduce the memory cost at encoder, it is proposed to allow the filtering unit size larger than the CTU size controlled by a SPS flag. In the proposed implementation, the dimensions of the filtering unit for 4K sequences are doubled in RA conditions, as shown in Figure 1. The samples inside an enlarged filtering unit share the same filtering parameters. The parameters such as on/off flag and APS index only needs to be signaled for each filtering unit rather than each CTU. Besides, the encoding time could also be reduced since the encoder requires less training rounds and determinations for ALF and CCALF.

Observed from the encoder logs, the peak encoder memory cost could be decreased from 23.2G to 19.6G for a 4K sequence in RA configurations.

Figure 1 The Proposed Memory Reduction Method

Simulation Results

The proposed method was implemented on top of the ECM-9.0 [1]. Simulations were performed following the common test conditions [2]. More detailed results could be found in the attached excel sheets. The results compared to ECM-9.0 are shown in TABLE 1.

TABLE 1 Performance of the Proposed Method

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.09% | 0.41% | 99% | 100% |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |

Conclusions

In this contribution, a peak memory reduction for ECM encoder is proposed. The proposed method can reduce the peak memory requirement remarkably with a minor performance loss. It is recommended to adopt the proposed method into ECM and CTC.

Reference

- ECM-9.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-9.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-Y2017, by teleconference, January 2022.

Patent Rights Declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).