| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0262-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AE0123 (AHG10: Lagrange multiplier optimization for chroma ALF and CCALF) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Han Zhang | Email: | zhhanzhang@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution reports the crosscheck results of JVET-AE0123: AhG10: Lagrange multiplier optimization for chroma ALF and CCALF. It is reported that the results match what was provided by the proponent.

Introduction

In JVET-AE0123, the Lagrange multiplier of chroma ALF or  CCALF would be modified by a factor when the luma ALF APS new filter flag is true and the number of history chroma ALF APS or CCALF APS is insufficient. Crosschecked results match the proponent’s results.

Results

The crosschecked results are as follows:

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-20.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.08% | -0.70% | -1.03% | 101% | 101% |
| Class C | -0.02% | -0.59% | -0.80% | 100% | 100% |
| Class E | 0.07% | -1.27% | -1.93% | 100% | 101% |
| Overall | 0.05% | -0.80% | -1.18% | 100% | 101% |
| Class D | 0.04% | -0.52% | -2.03% | 99% | 100% |
| Class F | 0.04% | -1.01% | -1.04% | 99% | 99% |
|  |  |  | Low delay P Main10 |  |  |
|  |  |  | Over VTM-20.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | -1.06% | -0.75% | 99% | 101% |
| Class C | -0.09% | -0.42% | -0.65% | 100% | 100% |
| Class E | 0.13% | -1.24% | -2.00% | 100% | 98% |
| Overall | 0.02% | -0.89% | -1.03% | 100% | 100% |
| Class D | 0.07% | -1.38% | -0.95% | 100% | 101% |
| Class F | 0.00% | -0.82% | -0.95% | 100% | 97% |

Conclusion

It is reported that the results match the results provided by the proponent for JVET-AE0123.

Patent rights declaration(s)

Tencent does not have any current or pending patent rights relating to the technology described in this contribution.