| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0279 |
| --- | --- |

| Title: | [AHG9] New nnpfc_mode_idc for reuse of NNPF definition for new purpose |  |  |
| --- | --- | --- | --- |
| Status: | Input contribution to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Youngkwon Lim 665 Clyde Avenue Mountain View, CA 94043 USA | Tel: Email: | young.L@samsung.com |
| Source: | Samsung Research America |  |  |

_____________________________

# Abstract

An NNPF can be associated with a single purpose and ID within a particular CLVS regardless of its capability. Sometime it might be efficient and useful to reuse NNPF data with new purpose and ID. This contribution propose a new nnpfc_mode_idc for referencing an NNPF already defined with a purpose and ID for new purpose and ID without sending the data again.

# Introduction

The NNPFC SEI message includes the field which can indicate the purpose, the intended usage, of the filter and the combination of a purpose and assigned id of the filter cannot be changed in a particular segment of bitstream, e.g. coded layer video sequence (CLVS) of a certain video codec. In some cases, a filter can have multiple capabilities and its operation should be restricted by the purpose indicated by the purpose field, i.e. nnpfc_purpose, to follow the intention of the content creator, service provider and so on. So, if a NNPF needs to be used for more than one distinctive purpose it should be indicated by two different purpose values and id values, respectively. However, it seems like there are no ways to redefined the purpose of a filter which has been already defined. With the current syntax structure, the data must be sent again to use a filter with new purpose even if the filter data is exactly same. This contribution proposes an way of efficiently reusing filter data already associated with an ID and a certain purpose for a different purpose and ID.

# Proposed update of syntax and semantics of NNPFC SEI message

## Overview

It is proposed to introduce a new nnpfc_mode_idc to reference a filter data already associated with one purpose and ID so that the same data can be associated with new purpose and ID.

## Syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| nnpfc_id | ue(v) |
| nnpfc_base_flag | u(1) |  |
| nnpfc_mode_idc | ue(v) |
| if( nnpfc_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_reserved_zero_bit_a | u(1) |
| nnpfc_tag_uri | st(v) |
| nnpfc_uri | st(v) |
| } |  |
| nnpfc_property_present_flag | u(1) |
| **************************** |  |  |
| no changes |  |  |
| ***************************** |  |
| /* ISO/IEC 15938-17 bitstream */ |  |
| if( nnpfc_mode_idc = = 0 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_reserved_zero_bit_b | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| nnpfc_payload_byte[ i ] | b(8) |
| } |  |
| } |  |
| if( nnpfc_mode_idc = = 2 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_reserved_zero_bit_c | u(1) |
| reference_nnpfc_id | ue(v) |
| } |  |
| } |  |

## Semantics

nnpfc_mode_idc equal to 0 indicates that this SEI message contains an ISO/IEC 15938-17 bitstream that specifies a base NNPF (when nnpfc_base_flag is equal to 1) or is an update relative to the base NNPF with the same nnpfc_id value (when nnpfc_base_flag is equal to 0).

When nnpfc_base_flag is equal to 1, nnpfc_mode_idc equal to 1 specifies that the base NNPF associated with the nnpfc_id value is a neural network identified by the URI indicated by nnpfc_uri with the format identified by the tag URI nnpfc_tag_uri.

When nnpfc_base_flag is equal to 0, nnpfc_mode_idc equal to 1 specifies that an update relative to the base NNPF with the same nnpfc_id value is defined by the URI indicated by nnpfc_uri with the format identified by the tag URI nnpfc_tag_uri.

nnpfc_mode_idc equal to 2 specifies that the NNPF previously associated with the referenced_nnpfc_id is reused with the new nnpfc_purpose and nnfpc_id indicated in this SEI message. Within a particular CLVS, both of the base NNPF and updates of the NNPF whose value of the nnpfc_id equals to the value of the reference_nnpfc_id in this SEI message shall be considered the base NNPF and the updates of the NNPF whose value of the nnpfc_id of this SEI message. The value of nnpfc_purpose of this SEI message and any of the SEI message whose value of nnpfc_id equals to the value of the reference_nnpfc_id in this SEI message shall not be identical. The value of nnpfc_mode_idc of any of the SEI messages whose value of nnpfc_id equals to the value of the reference_nnpfc_id in this SEI message shall not be 2.

The value of nnpfc_mode_idc shall be in the range of 0 to 2, inclusive, in bitstreams conforming to this edition of this document. Values of 3 to 255, inclusive, for nnpfc_mode_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_mode_idc in the range of 3 to 255, inclusive. Values of nnpfc_mode_idc greater than 255 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

# Patent rights declaration(s)

Samsung Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).