| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0285-v1 |
| --- | --- |

| Title: | AHG4: Preparation for content layering functional test |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Shunsuke Iwamura Daichi Arai Shimpei Nemoto Atsuro Ichigaya | Tel: Email: | iwamura.s-gc@nhk.or.jp |
| Source: | NHK |  |  |

_____________________________

- Abstract

This contribution reports preparation of content layering functional test to be conducted in 31th JVET meeting at Geneva.

Test sites

Functional test is planned to be conducted during 31st JVET meeting held in Geneva, at the viewing room “M03b” in Montbrilliant building.

Functional test coordination

The coordinators of the functional tests are the chairs of AHG4/AG5. Block diagram of the equipment for the test is illustrated in Figure 1. Multilayer bitstream capsuled in CMAF over MMT is output from the streamer and VVC decoder supporting multilayer decodes the bitstream in real time. Viewers can select which contents (main or sub) to play out by controlling by Tablet PC. VVC multilayer real-time decoder and CMAF/MMT streamer, signal converter for the functional test are prepared by NHK and 4K Monitor is prepared by ITU-T.

Figure 1. Equipment for the functional test

Table 1. Specifications of multilayer VVC decoder

| Profile | Main 10, Multilayer Main 10 |
| --- | --- |
| Level | 6.3 |
| Number of supported layers | 4 layers |
| Input Format | CMAF (ISO/IEC 23000-19) on MMT (ISO/IEC 23008-1) |
| Output Format | Resolution: 1080p, 2160p, 4320p Frame rate [Hz]: 59.94, 60, 119.88, 120 |

Sequences

Spatial resolution of the test sequences used in the functional test is 4K, 60Hz. In the test, L-shape sub-content, Picture-in-Picture style sub-content are validated. Thumbnail and brief explanation is listed in the table below. All the sequences used for the functional test is copyright limited and not allowed to be redistributed.

|  | Content1 Main: Report from library Sub: Main with sign-language | Content2 Main: Main camera Sub1: Main with Home team view Sub2: Main with Away team view | Content3 Main: Children’s TV show Sub1: Main with dance instruction by a boy Sub2: Main with dance instruction by a girl |
| --- | --- | --- | --- |
| L0 |  |  |  |
| L1 |  |  |  |
| L2 |  |  |  |

Coding algorithm

VTM with content layering functionality

VTM-18.0 with content layering functionality is used for generating the bitstream. The main content is coded as base layer bitstream. For the enhancement layer, the sub content with alpha channel information is overlaid on top of the locally decoded picture of the base layer and used as the input signal of the enhancement layer. Since most of the area of the enhancement layer is exactly same as the base layer, most of the block can be coded as “skipped” without RD optimization and only the region with overlay is coded with RD optimization.

Figure 2 illustrates the overview of encoding procedure of the content layering. Note that since the input signal of the enhancement layer is the main content with overlay, alpha channel information does not have to be transmitted to the decoder side.

Figure 2. Content layering using multilayer coding

Figure 3 shows the alternative solution to realize the same functionality by using system layer composition. Here, the main content, sub-content, and alpha channel information is separately encoded as single layer bitstream and transmitted to the decoder (with position information if needed). At the decoder side, the main and sub-content are decoded separately and composited using the position information and alpha channel information.

Figure 3. Content layering using system layer composition

Content layering with scaling window

For some test sequences (Content1 and Content3), the sub-content is not just overlaid on top of the main content. The main content is down-scaled, and L-shaped sub-content is overlaid in the margin. Inter-layer prediction can be effectively performed with scaling window functionality enabled. Down-scaling filter is the same as the filter used in RPR so that inter-layer prediction is perfectly performed in a similar way to the case of normal overlaying sub-content shown in figure 2.

Figure 4. Content layering with scaling window

Software package

The code changes for the content layering (overlaying sub-content on the locally decoded picture of base layer and omitting RD optimization for non-overlaid region when encoding enhancement layer) is planned to be made public after the meeting.

Encoding parameters

The following test conditions are used for the functional test.

- Rate control

  - The rate control is enabled for base and enhancement layer. The allocated bitrate for BL and EL is 20Mbps and 2Mbps.

- Coding structure

  - Random access, RA

    - Intra refresh at approximately 1 second intervals.

    - GOP size 32

    - Picture reordering allowed.

    - Inter-layer and intra-layer prediction allowed. Dependent layer uses the same coding structure + same POC of base layer as additional ref.

Patent rights declaration(s)

NHK may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).