| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0289-v1 |
| --- | --- |

| Title: | AhG11: HOP training process and models |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | F. Galpin, S. Eadie, Y. Li, L. Wang, Z. Xie D. Rusanovskyy Y.Li R. Chang J. Li E. Alshina | Tel: Email: | franck.galpin@interdigital.com seadie@qti.qualcomm.com yue.li@bytedance.com liqiangwang@tencent.com xiezhihuang@oppo.com dmytror@qti.qualomm.com renjiechang@tencent.com yli30@qti.qualcomm.com lijunru@bytedance.com elena.alshina@huawei.com |
| Source: | InterDigital, Qualcomm, ByteDance, Tencent, Oppo, Huawei |  |  |

_____________________________

# Abstract

This contribution gives information on the HOP training or fine-tuning process. In particular, information on models and results for partial retraining are given.

# Introduction

In JVET-AE0191, a new architecture and training process is described for the High Operation Point loop-filtering. The training process is split into 3 stages of approximatively 15 days each: 5 days to generate the dataset and 10 days for training itself.

In order to demonstrate advantages of small architecture changes or training procedure, a partial retraining is available. This contribution gives information on models location and expected results for each stages in case of partial retraining.

# Training process

## Overview

Figure 2 Training steps for HOP model creation

The figure above describes the 3 stages needed to fully train the HOP model:

- Stage I:

  - Extract a dataset of intra coded frames using VTM

  - Train HOP on the dataset: resulting model is HOP.I

- Stage II:

  - Extract a dataset of frames using VTM and HOP model from stage I

  - Train HOP on the dataset: resulting model is HOP.II – Integerized the model.

- Stage III:

  - Extract a dataset of frames using VTM and HOP model from stage II

  - Train HOP on the dataset: resulting model is HOP.III (final model)

Details for training can be found in JVET-AE0191.

All models can be found in the NNVC repository at  https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM.

## Full retraining – training from stage I

In case of full retraining, process described in JVET-AE0191 should be followed.

On CTC described in JVET-AD2016, using NNVC-5.0 VTM as anchor and NNVC-5.0 VTM+HOP.I as test, the following results are found using the model after stage I training:

|  | NNVC 4.0 vs NNVC-5.1 VTM+HOP.II |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | Random Access |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -6.49% | -16.71% | -20.36% | -4.54% | -14.67% | -15.26% |
| Class A2 | -7.13% | -20.33% | -19.13% | -4.88% | -17.83% | -15.76% |
| Class B | -6.96% | -19.57% | -18.77% | -4.02% | -18.94% | -15.84% |
| Class C | -8.16% | -18.73% | -21.03% | -3.74% | -17.31% | -17.52% |
| Class E | -10.44% | -20.69% | -22.16% |  |  |  |
| Overall | -7.76% | -19.22% | -20.16% | -4.22% | -17.43% | -16.15% |
| Class D | -8.00% | -17.72% | -20.87% | -3.86% | -17.96% | -18.27% |
| Class F | -4.30% | -11.29% | -10.89% | -1.76% | -7.95% | -6.95% |

## Training from stage II

In case of training from stage II, a dataset using VTM and HOP.I model should be used.

The model is available in the repository in the directory: training/training_scripts/NN_Filtering_HOP/models/HOP_float.I.sadl

The float model is used to generate the dataset of stage II.

The final model for stage II is the one at epoch 18 after integerization.

On CTC described in JVET-AD2016, using NNVC-5.0 VTM as anchor and NNVC-5.0 VTM+HOP.II as test, the following results are found using the model after stage II training:

|  | NNVC 4.0 vs NNVC-5.1 VTM+HOP.II |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | Random Access |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -7.05% | -17.89% | -20.06% | -9.65% | -18.14% | -22.49% |
| Class A2 | -6.97% | -19.25% | -16.77% | -10.08% | -22.08% | -17.55% |
| Class B | -7.00% | -18.24% | -19.89% | -8.92% | -23.34% | -22.97% |
| Class C | -7.95% | -18.65% | -21.27% | -9.65% | -24.98% | -24.39% |
| Class E | -10.39% | -20.44% | -21.57% |  |  |  |
| Overall | -7.78% | -18.81% | -19.98% | -9.49% | -22.48% | -22.17% |
| Class D | -7.83% | -17.73% | -21.23% | -10.89% | -24.02% | -24.54% |
| Class F | -5.53% | -16.45% | -16.17% | -5.30% | -16.51% | -14.82% |

Using a naïve quantization with quantizer 11 for all weights, the following differences between float and integerized models was found.

|  | St2.ep18.int vs St2.ep18.float |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | Random Access |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | 0.02% | -1.15% | -0.36% | 0.10% | 0.53% | 0.30% |
| Class C | 0.00% | -0.23% | -0.35% | 0.13% | 0.76% | 0.59% |
| Class E | #VALUE! | #VALUE! | #VALUE! | 0.00% | 0.00% | 0.00% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.00% | -0.05% | -0.18% | -0.02% | -0.21% | -0.16% |

## Training from stage III

In case of training from stage III, a dataset using VTM and HOP.II model should be used.

The model is available in the repository in the directory:

training/training_scripts/NN_Filtering_HOP/models/HOP_int16.II.sadl

The int16 model is used to generate the dataset of stage III.

MD5sums of stage 3 datasets can be found in https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc (TBA)

# Patent rights declaration(s)

InterDigital, Qualcomm, ByteDance, Tencent, Oppo, Huawei may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).