| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE1004-v1 |
| --- | --- |

| Title: | Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Errata report |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Benjamin Bross Iole Moccagatta Chris Rosewarne Gary J. Sullivan | Tel: Email: | yekui.wang@bytedance.com benjamin.bross@hhi.fraunhofer.de iole.moccagatta@intel.com chris.rosewarne@canon.com.au gary.sullivan@dolby.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document contains a list of reported errata items for VVC, VSEI, HEVC, AVC, Video CICP, and the TR on usage of video signal type code points, for tracking purposes. Some of the items have been confirmed by the JVET and have been agreed to require fixing. This document also provides publication status backgrounds of these standards.

Incorporated items at the JVET-AE meeting:

- Updated the general descriptions for VVC, VSEI, HEVC, and AVC.

- Removed obsolete/resolved errata items.

- Added 16 items for the ISO text only for HEVC, identified when working on the ITU-T consent text for H.265 version 9 (edition 5).

- Added 9 items on the semantics of post-filter hint SEI message from JVET-AE0155 for both HEVC and AVC.

- Added 4 items for AVC, on semantics of the omnidirectional video SEI messages. These items were among the items for HEVC and have been resolved for HEVC.

- Added an item for AVC on use of the "should" wording for the use of the film grain characteristics SEI messages when present.

- Removed the section for codepoint usage TR as the TR part has been dropped from the title of the document a while ago.

- Added an item for HEVC regarding replacing of "Max slice segments per picture MaxSliceSegmentsPerPicture" in Table A.8 with "Max # of slice segments per picture MaxSliceSegmentsPerPicture" (reported by Cliff Reader – thanks!).

Incorporated items at the JVET-AD meeting:

- For AVC, incorporated the following items:

  - JVET-AD0078 item 4 and 5 on fixing the equation to derive the filter hint cross correlation matrix.

Incorporated items at the JVET-AC meeting:

- For VVC, all errata items have been incorporated in JVET-AC2005: New level and systems-related supplemental enhancement information for VVC (Draft 4).

- For HEVC, incorporated the following items:

  - Confirmed redudnant call to clause 8.6.1 (reported by Cliff Reader – thanks!).

  - Confirmed JVET-AB0120: Addition of interpolation for "initial CPB removal delay offset".

  - JVET-AC0311: On number of conformance tests (option 1.)

  - Updated issues 1 to 4 in Subsection 4.4 per inputs from Miska M. Hannuksela (thanks!).

  - Change the font size from 8 pt to 10 pt for "Table 9-4" in a paragraph in clause 9.3.2.2 (reported by Cliff Reader – thanks!).

- For AVC, incorporated the following items:

  - Confirmed JVET-AB0120: Addition of interpolation for "initial CPB removal delay offset".

  - An issue of talking about VUI in subclause 0.6.1 in ISO/IEC 14496-10:2022 (Edition 10) while the sentence should only talk about SEI messages.

Incorporated items at the JVET-AB meeting:

- For VVC, incorporated the following items:

  - JVET-AB0223: Text improvement for Timing / DU information SEI message in HEVC and VVC

  - Added a NOTE in the HRD text to mention that "leading pictures" associated with a DRAP picture may not be correctly decodable, per the discussion of JVET-AB0055 (On leading pictures design in DRAP SEI Message).

  - Added a NOTE to mention that the definition of IRAP in VSEI covers the case of GDR with ph_recovery_poc_cnt equal to 0, per the discussion of JVET-AB0057 (On the associated IRAP for DRAP and EDRAP pictures).

  - Tickets #1564, #1568, #1569, and #1572

  - JVET-AB0120: Addition of interpolation for "initial CPB removal delay offset"

- For HEVC, incorporated the following items:

  - JVET-AB0223: Text improvement for Timing / DU information SEI message in HEVC and VVC

  - On conformance indication of the Multiview Main, Scalable Main, Scalable Main 10, and 3D Main profiles (resulted from a discussion between Alexis Tourapis and Gary Sullivan – thanks!)

  - On inference of high_precision_offsets_enabled_flag (resulted from a discussion between Andy Fu and Gary Sullivan – thanks!)

  - On a redundant sentence in the definitionn of next_bits( n ) from Gary Sullivan

  - On addition the parenthetic abbreviation TB to the text in clause 9.3.3.1 (suggested by Cliff Reader– thanks!)

  - On indention of item 3 in the paragraph just below Equation 8-14 in clause 8.4.1 (reported and suggested by Cliff Reader – thanks!)

  - On redudnant call to clause 8.6.1 (reported by Cliff Reader – thanks!)

  - JVET-AB0120: Addition of interpolation for "initial CPB removal delay offset"

- For AVC, incorporated the following items:

  - JVET-AB0120: Addition of interpolation for "initial CPB removal delay offset"

Incorporated items at the JVET-AA meeting:

- For VVC: Added an errata item for VVC conformance.

- For VSEI: Added a pointer of some HEVC errata items that also apply for VSEI in Subsection 4.2.

- For HEVC: Removed various items that have been addressed during the editing of the DIS text of HEVC 5th edition in WG05 N00128, and added some editor's notes for the remaining items.

- For AVC:

  - Added changes on motion vector value ranges, based on JVET meeting minutes noted under JVET-AA0222.

  - Added a pointer of some HEVC errata items that also apply for AVC in Subsection 4.2.

Incorporated items at the JVET-Z meeting:

- For AVC, corrections to texts related to alpha blending corresponding to the changes adopted to VSEI from JVET-Z0119.

- Updated the summary and publication status for VVC.

- Updated the summary and publication status for VSEI.

- Updated the summary for HEVC.

Incorporated items at the JVET-Y meeting:

- Removed all errata items for VVC and VSEI, due to that for both VVC and VSEI the second edition texts are being prepared by both ITU-T and ISO/IEC, and all the errata items have been incorporated therein, including fixes for bug tickets #1533 and #1534 for VVC (fixes for these two tickets have not been included in previous versions of the errata report).

- Removed errata items for AVC that have been resolved in both the latest ITU-T text (08/2021) and the new submission (MDS21132_WG05_N00098) of the ISO/IEC 10th edition text.

Incorporated items at the JVET-X meeting:

- Updated the publication status of the standards

- For VVC (the changes are included in an attachment to this document):

  - Some changes resulted from reports/suggestions by Yue Yu and Frank Bossen (thanks!)

  - Various changes resulted from reports/suggestions by Peter de Rivaz (huge thanks!)

- For HEVC (the changes are included both below and in an attachment to this document):

  - Some changes resulted from reports/suggestions by Yue Yu, Frank Bossen, and Cliff Reader (thanks!)

  - Some changes to the semantics of the entry point offset syntax element, resulted from the discussion of JVET-X0050

  - Changes to the semantics of the alpha channel information SEI message resulted from the discussion of JVET-X0059

- For AVC (the changes are included in an attachment to this document):

  - Some changes to the semantics of the alpha blendnig related syntax elements in clause 7.4.2.1.2, message resulted from the discussion of JVET-X0059

Incorporated items at the JVET-W meeting:

- For VVC (the changes are included in an attachment to this document):

  - Fix for tickets #1488, #1511, #1491, #1493, #1494

- For HEVC:

  - Added the attachment for HEVC from JVET-W0187, which includes the following fixes:

    - Correcting an “off by one” error in the loop counter for ar_num_object_updates, which indicates the number of updated annotated objects for the annotated regions SEI message.

    - Adding a description of the purpose of some variables that are derived in the semantics of the annotated regions SEI message

    - Adding a missing descriptor to specify the type of coding for the depth_nonlinear_representation_model[ i ] syntax element (the same as used in another specification, so clearly what was intended) for the depth representation information SEI message

    - Adding missing range specifications for a few syntax elements of the depth representation information SEI message

    - Miscellaneous very small cleanups of consistency, grammar and clarity, and correction of typographical errors.

- For AVC:

  - Added the attachment for HEVC from JVET-W0187, which includes the same list of fixes as above for HEVC.

Incorporated items at the JVET-V meeting:

- For VVC (the changes are included in an attachment to this document):

  - Fix for ticket #1486

- For HEVC

  - Removed the item on the absence of “persistence flag” for the annotated regions SEI message (this was agreed to be removed at the JVET-T meeting in October 2020, but the decision was overlooked)

Incorporated items at the JVET-U meeting:

- For VVC (the changes are included in an attachment to this document):

  - Some minor editorial corrections and improvements (JVET-U0073, JVET-U0085, and one item from Hendry)

  - Fixes for tickets #1416, #1428, #1432, #1454, and #1469

- For AVC: Added a reference to JVET-T1006-v1 for the errata changes, and removed the actual errata text changes from this document. (JVET-U0049)

# General aspects and issues affecting multiple standards

## General and minor matters

Please note that the alignment between the text published by ITU-T and that published by ISO/IEC should also be checked. Minor editorial issues and aspects that are highlighted for potential further checking include the following:

- (for Video CICP only, in the ISO/IEC version only) Logical indentation nesting in the ISO/IEC version of the Video CICP colour interpretation was inadvertently removed in the publication process.

- (for Video CICP only) The range of values for the extended aspect ratio indicator in Video CICP is not clearly specified and may implicitly be interpreted as inadequate to cover the range of values expressed in the video coding standards.

# Reported errata items for VVC

## General

Both the ITU-T second edition text and the ISO/IEC second edition text have been published. The third edition for both ITU-T and ISO/IEC are in the approval and/or publication process. Besides those errata items directly included below in this Section, JVET-AE2005 may include some VVC errata changes (in addition to some amendments).

## Publication status background

Rec. ITU-T H.266 (https://www.itu.int/rec/T-REC-H.266/en)

- (04/2022, Edition 2) Approved 2022-04-29, published 2022-07-12

ISO/IEC 23090-3 (https://www.iso.org/standard/83531.html)

- ISO/IEC 23090-3:2022 (Edition 2); published 2022-09

Rec. ITU-T H.266 (https://www.itu.int/rec/T-REC-H.266/en)

- (08/2020, Edition 1) Approved 2020-08-29, published 2020-11-10

ISO/IEC 23090-3 (https://www.iso.org/standard/73022.html)

- ISO/IEC 23090-3:2021 (Edition 1); published 2021-02

## Errata items for VVC

None

## Errata items for VVC conformance

None.

# Reported errata items for VSEI

## General

Both the ITU-T second edition text and the ISO/IEC second edition text have been published. The third edition for both ITU-T and ISO/IEC are in the approval and/or publication process. JVET-AE2006 may include some VSEI errata changes (in addition to some amendments).

## Publication status background

Rec. ITU-T H.274 (https://www.itu.int/rec/T-REC-H.274/en)

- (05/2022, Edition 2) Approved 2022-05-22, published 2022-07-25

ISO/IEC 23002-7 (https://www.iso.org/standard/83530.html)

- ISO/IEC 23002-7:2022 (Edition 2); published 2022-10

Rec. ITU-T H.274 (https://www.itu.int/rec/T-REC-H.274/en)

- (08/2020, Edition 1) Approved 2020-08-29, published 2020-11-10

ISO/IEC 23002-7 (https://www.iso.org/standard/79112.html)

- ISO/IEC 23002-7:2021 (Edition 1); published 2021-01

# Reported errata items for HEVC

## General

Besides those errata items directly included below in this Section, JVET-AE1006 may include some HEVC errata changes (in addition to some amendments).

## Publication status background

Rec. ITU-T H.265

- (08/2021, Edition 8) Approved 2021-08-22, published 2021-10-13

- (10/2019, Edition 7) Approved 2019-11-29, published 2020-01-10

- (06/2019, Edition 6) Approved 2019-06-29, published 2019-09-23

- (02/2018, Edition 5) Approved 2018-02-13, published 2018-05-11

ISO/IEC 23008-2

- ISO/IEC 23008-2:2020 (Edition 4); published 2020-08

- ISO/IEC 23008-2:2017 (Edition 3); published 2017-10

- ISO/IEC 23008-2:2017/Amd 1:2018 (Additional colour representation code point); published 2018-03

- ISO/IEC 23008-2:2017/Amd 2:2018 (Main 10 still picture profile); published 2018-03

- ISO/IEC 23008-2:2017/Amd 3:2018 (Additional supplemental enhancement information); published 2018-07

- Work items merged for FDIS ballot as publication office action; Stage 50.00 Final text received or FDIS registered for formal approval as of 2020-03-19

  - ISO/IEC DIS 23008-2:201x (Edition 4); DIS ballot closed 2018-10-06; stage 40.99, Full report circulated: DIS approved for registration as FDIS 2019-02-18; Pending FDIS ballot based on WG 11 N 18277

  - ISO/IEC 23008-2:201x (Edition 4)/DAmd 1:201x (Additional supplemental enhancement information); DAM ballot started 2019-07-10, closed 2019-10-02

## Errata items for HEVC

### Items for both the ITU-T text and the ISO text

- In Table A.8, replace "Max slice segments per picture MaxSliceSegmentsPerPicture" with "Max # of slice segments per picture MaxSliceSegmentsPerPicture".

### Items for the ISO text only

- There are two instances “At most, one”, with the comma ‘,’ added (by the ISO/CS editor, I guess). This should not have been added.

- In a couple of places (semantics of vps_extension_data_flag, slice_segment_header_extension_data‌_byte, vps_extension_data_flag), the three words “of the profiles” or “for the profiles” (maybe the latter is better and should be consistently used)” in the phrase below were removed (by the ISO/CS editor, I guess). These words should not have been removed.

… not affect the decoding process of the profiles specified in…

… not affect the decoding process for the profiles specified in…

- In the NOTE in 7.4.3.7, there is one remaining “may be”.

- Just before formulae 8-185 and 8-186, “the following assignments are made” should be changed to “the following assignment is made”.

- Just before formular 8-382, the word “is” is missing.

- In Formula C-24, change “InitCpbRemovalDelayOffset [ SchedSelIdx ]” to “InitCpbRemovalDelay‌Offset[ SchedSelIdx ]” (remove a space just before ‘[’), and change “BitRate [ SchedSelIdx − 1 ]” to “BitRate[ SchedSelIdx − 1 ]” (again (remove a space just before ‘[’)].

- There is a “should” in NOTE 5 in subclause D.3.9, which can be replaced with “is expected to”.

- In D.3.13, NOTE 1: The second sentence is an incomplete sentence: “Synthesis of simulated film grain on the input images, which could be the decoded pictures or converted from the decoded pictures, as the display process is optional and does not necessarily exactly follow the specified semantics of the film grain characteristics SEI message.” Changing “as the display process” to “for the display process” would fix the issue.

- In Formula D-7, change “Î[ xB * 8 + I, yB * 8 + j, c ]” to “Î[ c ][ xB * 8 + i ][ yB * 8 + j ]”.

- In D.3.13, NOTE 8, change “size16x16” to “size 16x16”.

- In D.3.13, just below formula D-15, change “for each value of x, y, and c” to “for each value of c, x, and y”.

- In D.3.14 (Post-filter hint SEI message semantics), all the editor’s notes are still there and should be removed. Maybe in the submitted FDIS text these have already been removed.

- In D.3.14, change “the hint values specify a cross-correlation matrix” to “filter hint values are provided, and the provided filter hint values specify a cross-correlation matrix”.

- In D.3.48, it looks like that the addition of “and display” in “The shutter interval information SEI message indicates the shutter interval for the associated video source pictures prior to encoding and display” should not have been done.

- In F.14.3.8, NOTE 3, change “(see 7.3)” to “(see E.3.1)”.

- In G.11.2.2, just before Formula G-11, change “3-dimensional” to “three-dimensional”.

### Items on the semantics of post-filter hint SEI message from JVET-AE0155

- It does not seem clear whether the “16-bit precision” description applies only to the cross-correlation matrix case or also to the filter coefficient cases.

- A specified 32-bit range may be excessive for a syntax element that has 16-bit precision.

- Much of the text is written from the encoder perspective. We generally do not specify things from the encoder perspective. The decoder-side processing is not clearly described or is discussed only vaguely. For example, no equations are given for the decoder’s filtering process.

- It does not seem clear whether the decoder should divide the filter tap values (or equivalently, the result of the filtering process) by the sum of the filter tap values or if the encoder is responsible for the filter normalization.

- There are issues relating to how to interpret the filter depending on whether the filter length is even (in which case the output samples may presumably be located spatially between input samples) or odd (in which case the location of an output sample may presumably have a location corresponding to the centre tap).

- The handling of the boundaries of the picture and cropping window are not described and seems unclear.

- It seems unclear why should it matter whether a horizontal filter or a vertical filter is applied first if both are to be applied. Perhaps integer rounding is expected to be applied between the two filtering operations, but this is not stated.

- The discussion of Wiener post-filtering is a bit vague (e.g., what exactly is a Wiener filter, and how would a decoder derive one from the provided data?).

- A decoder could use a subset of a provided cross-correlation matrix to derive a Wiener post-filter with a smaller region of support than the provided statistics would enable. It does not seem clear whether this is intended to be an acceptable or possible practice.

-

-

-

-

-

-

-

# Reported errata items for AVC

## General

See Subsection 5.2 below.

## Publication status background

Rec. ITU-T H.264

- (08/2021, Edition 14) Approved 2021-08-22, published 2021-10-13

- (06/2019, Edition 13) Approved 2019-06-13, published 2019-09-06

ISO/IEC 14496-10

- ISO/IEC 14496-10:2022 (Edition 10), published 2022-11 (https://www.iso.org/standard/83529.html)

- ISO/IEC 14496-10:2020 (Edition 9), published 2020-12

- ISO/IEC 14496-10:2014 (Edition 8), published 2014-09

- ISO/IEC 14496-10:2014/Amd 1:2015 (Multi-resolution frame compatible stereoscopic video with depth maps, additional supplemental enhancement information and video usability information), published 2015-11

- ISO/IEC 14496-10:2014/FDAMD 2 (Additional Levels and Supplemental Enhancement Information); stage 50.98, deleted in preparation for Edition 9

- ISO/IEC 14496-10:2014/Amd 3:2016 (Additional supplemental enhancement information); published 2016-12, published 2016-12

- ISO/IEC DIS 14496-10:201x (Edition 9); stage 40.99 Full report circulated: DIS approved for registration as FDIS as of 2018-01-31

## Errata items

- (ISO/IEC 14496-10:2022 (Edition 10)) Subclause 0.6.1 says this: "Rec. ITU-T H.274 | ISO/IEC 23002-7 specifies the syntax and semantics of some video usability information (VUI) parameters and supplemental enhancement information (SEI) messages that do not affect the conformance specifications in Annex C and subclauses F.8, G.8, H.8, and I.8. Among these SEI messages, those for which the syntax and semantics …". However, the VUI that is specified in H.274 is a different VUI than what is specified in AVC, thus it should not talk about the VUI in that sentence, it should only talk about SEI messages.

- Remove the second sentence from the following:

next_bits( n ) provides the next bits in the bitstream for comparison purposes, without advancing the bitstream pointer. It provides a look at the next n bits in the bitstream with n being its argument. When used within the byte stream format as specified in Annex B and fewer than n bits remain within the byte stream, next_bits( n ) returns a value of 0.

- The pictureWidth variable is also modified internally in clause D.2.35.6.2, which seems potentially unwise. Replace

–	If RegionWisePackingFlag is equal to 0 and erp_padding_flag is equal to 1, the following applies:

hPos′ = hPos − left_gb_erp_width		(D-45)
pictureWidth = pictureWidth − left_gb_erp_width − right_gb_erp_width

–	Otherwise, the following applies:

hPos′ = hPos		(D-46)

–	The following applies:

ϕ = 180 − hPos′ * ( 360 ÷ pictureWidth )		(D-47)
θ = 90 − vPos * ( 180 ÷ pictureHeight )

with

–	If RegionWisePackingFlag is equal to 0 and erp_padding_flag is equal to 1, the following applies:

hPos′ = hPos − left_gb_erp_width		(D-45)
tmpPicWidth = pictureWidth − left_gb_erp_width − right_gb_erp_width

–	Otherwise, the following applies:

hPos′ = hPos		(D-46)
tmpPicWidth = pictureWidth

–	The following applies:

ϕ = 180 − hPos′ * ( 360 ÷ tmpPicWidth )		(D-47)
θ = 90 − vPos * ( 180 ÷ pictureHeight )

- In clauses D.2.35.6.4 and D.2.35.6.5, remove the sentences that set the values of offsetX and offsetY, remove offsetX and offsetY as inputs to a process, and replace both "offsetX" and "offsetY" with "0.5".

- The first sentence of D.2.35.6.5 (Mapping of luma sample locations within a cropped decoded picture to sphere coordinates relative to the global coordinate axes) says “This clause specifies the semantics of luma sample locations within a cropped decoded picture to sphere coordinates relative to the global coordinate axes.” Here, “semantics” should be “mapping”.

- In clause E.2.1, make the following changes:

log2_max_mv_length_horizontal and log2_max_mv_length_vertical indicate the maximum absolute value range of a decoded horizontal and vertical motion vector component, respectively, in ¼ luma sample units, for all pictures in the coded video sequence. A value of n asserts that no value of a motion vector component shall exceed the range from −2n to 2n − 1, inclusive, in units of ¼ luma sample displacement. The value of log2_max_mv_length_horizontal shall be in the range of 0 to 15, inclusive. The value of log2_max_mv_length_vertical shall be in the range of 0 to 15, inclusive. When log2_max_mv_length_horizontal is not present, the values of log2_max_mv_length_horizontal and log2_max_mv_length_vertical shall be inferred to be equal to 15.

NOTE 15 – The maximum absolute value range of a decoded vertical or horizontal motion vector component is also constrained by profile and level limits as specified in Annex ‎A and clauses ‎G.10 and ‎H.10.

For bitstreams of levels 6, 6.1, and 6.2 (see Annex A), it has been reported that some decoder implementations do not properly decode bitstreams that have horizontal motion vector component values that exceed the range from −2048 to 2047, inclusive, or that have vertical motion vector component values that exceed the range from −512 to 511, inclusive, in units of ¼ luma sample displacement. It is therefore suggested that bitstreams should not contain motion vector component values that exceed these ranges. This constraint can be indicated by using values of log2_max_mv_length_horizontal less than or equal to 11 and values of log2_max_mv_length_vertical less than or equal to 9.

- In clause G.13.2.1, make the following changes:

log2_max_mv_length_horizontal[ i ] and log2_max_mv_length_vertical[ i ] indicate the values of log2_max_mv_length_horizontal and log2_max_mv_length_vertical, as specified in clause ‎E.2.1, that apply to the current scalable layer representation.

NOTE 17 – The maximum absolute value range of a decoded vertical or horizontal motion vector component is also constrained by profile and level limits as specified in Annex ‎A and clause ‎G.10.

- In clause H.13.2.3, make the following changes:

log2_max_mv_length_horizontal[ i ] and log2_max_mv_length_vertical[ i ] specify the values of log2_max_mv_length_horizontal and log2_max_mv_length_vertical, as specified in clause ‎E.2.1, for the current operation point representation. When log2_max_mv_length_horizontal[ i ] is not present, the values of log2_max_mv_length_horizontal and log2_max_mv_length_vertical for the current operation point representation shall be inferred to be equal to 16. The value of log2_max_mv_length_horizontal[ i ] shall be in the range of 0 to 16, inclusive. The value of log2_max_mv_length_vertical[ i ] shall be in the range of 0 to 16, inclusive.

NOTE 3 – The maximum absolute value range of a decoded vertical or horizontal motion vector component is also constrained by profile and level limits as specified in Annex ‎A or clause ‎H.10.2.

- Add interpolation for "initial CPB removal delay offset". Modify C.4 as follows:

For output timing decoder conformance, the HSS operates as described above, with delivery schedules selected only from the subset of values of SchedSelIdx for which the bit rate and CPB size are restricted as specified in Annex ‎A, Annex ‎G, Annex ‎H, and Annex ‎I for the specified profile and level, or with "interpolated" delivery schedules as specified below for which the bit rate and CPB size are restricted as specified in Annex ‎A, Annex ‎G, Annex ‎H, and Annex ‎I. The same delivery schedule is used for both the HRD and DUT.

When the HRD parameters and the buffering period SEI messages are present with cpb_cnt_minus1 greater than 0, the decoder shall be capable of decoding the bitstream as delivered from the HSS operating using an "interpolated" delivery schedule specified as having peak bit rate r, CPB size c( r ), and initial CPB removal delay ( f( r ) ÷ r ) and initial CPB removal delay offset ( o( r ) ÷ r ) as follows:

 = ( r − BitRate[ SchedSelIdx − 1 ] ) ÷ ( BitRate[ SchedSelIdx ] − BitRate[ SchedSelIdx − 1 ] ),	(C-17)

c( r ) =  * CpbSize[ SchedSelIdx ] + (1 −  * CpbSize[ SchedSelIdx−1 ],	(C-18)

f( r ) = initial_cpb_removal_delay[ SchedSelIdx ] * BitRate[ SchedSelIdx ] + 
		( 1 −  initial_cpb_removal_delay[ SchedSelIdx − 1 ] * BitRate[ SchedSelIdx − 1 ]	(C-19)

o( r ) = α * initial_cpb_removal_delay_offset[ SchedSelIdx] * BitRate[ SchedSelIdx ] +
		( 1 − α ) * initial_cpb_removal_delay_offset[ SchedSelIdx − 1 ] * BitRate[ SchedSelIdx − 1 ]		(C-XX)

- In clause D.2.24 (Post-filter hint SEI message semantics), modify equation D-27 as follows (check and align with the syntax and semantics in the latest HEVC text):

filter_hint(cy,cx)=14bitDepth-8∙h∙wm=0h-1n=0w-1sm,n∙s'(clip3(0,h-1,m+cy-offset_y),clip3(0,w-1,n+cx-offset_x))	(D-27)

- All items in Subsection 4.3.2 (Items on the semantics of post-filter hint SEI message from JVET-AE0155).

- In the film grain characteristics (FGC) SEI message, use the "should" wording for the use of FGC SEI messages when present (check and align with the wording in the latest VSEI and HEVC texts).

# Reported errata items for Video CICP

## General

See also Section 1.

## Publication status background

Rec. ITU-T H.273

- (07/21, Edition 2) Approved 2021-07-14, published 2021-09-24

- (12/16, Edition 1) Approved 2016-12-22, published 2017-04-27

ISO/IEC 23091-2 (previously part of ISO/IEC 23001-8)

- ISO/IEC 23091-2:2019, published 2019-07

## Errata items

- There is an error in the range of values for the sample aspect ratio width and height.

- The CICP revision should account for the various errata issues previously noted in AL1004. It was confirmed that the BT.2100 reference needs updating (to -2).

- The equations for ICtCp with HLG have not been updated and need be corrected.