| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13-20 October 2023 | Document: JVET-AF0047-v1 |
| --- | --- |

| Title: | [AHG4] ECM10.0 evaluation on V3C test content |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sebastian Schwarz, Miska M. Hannuksela Nokia Technologies Munich, Germany | Email: | {sebastian.schwarz, miska.hannuksela}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This document provides performance evaluations of the latest ECM version on volumetric video-based coding (V3C) video test sequences. When compared to ECM8.0, ECM10.0 shows improvements for MPEG immersive video (MIV) content, whereas there is a moderate loss in RA conditions and a moderate gain in AI conditions for video-based point cloud coding (V-PCC) content. Comparing the BD-rate of ECM against VTM, a smaller bitrate reduction is obtained with V3C content than with the content classes of the common test conditions.

# Introduction

Visual volumetric video-based coding (V3C) video test sequences for JVET were introduced in [1] alongside ECM8.0 performance evaluation. A specific metric focused at V3C V-PCC content was introduced in [2]. This metric uses occupancy map information to reliably reflect end-to-end distortion in V3C V-PCC use cases and was adopted by WG7 for video coding evaluation reporting on V-PCC content. In this contribution we provide an update on the performance evolution for V3C test content, from ECM8.0 to ECM10.0. While ECM10.0 brings good gains for V3C MIV content, V3C V-PCC content suffers small losses in RA conditions.

# ECM10.0 coding performance for V3C content

The table below summarises coding performance of ECM10.0 for V3C test content against VTM 22.0. The content used for this evaluation is available at [3]. Full results are attached to this contribution.

| ECM10.0 over VTM22.0 |  |  |  |  |
| --- | --- | --- | --- | --- |
| RA10 | BD-Rate [%] |  |  |  |
|  | Y | U | V | Occ Y |
| V-PCC texture | -12.90% | -27.63% | -7.76% | -7.76% |
| V-PCC geometry | -13.94% |  |  | -6.66% |
| MIV texture | -16.08% | -32.52% | -32.52% |  |
| MIV geometry | -14.47% |  |  |  |
| Average | -14.53% | -30.56% | -30.56% | -7.21% |
| AI10 | BD-Rate [%] |  |  |  |
|  | Y | U | V | Occ Y |
| V-PCC texture | -7.66% | -22.98% | -22.98% | -12.80% |
| V-PCC geometry | -4.20% |  |  | -2.68% |
| MIV texture | -11.57% | -31.28% | -31.28% |  |
| MIV geometry | -16.50% |  |  |  |
| Average | -10.79% | -27.96% | -27.96% | -7.74% |

As a comparison, the table below summarises ECM10.0 coding performance for the CTC content classes against VTM22.0.

| ECM10.0 over VTM22.0 |  |  |  |
| --- | --- | --- | --- |
| RA10 | BD-Rate [%] |  |  |
|  | Y | U | V |
| Class A1 | -22.04% | -26.81% | -35.79% |
| Class A2 | -25.58% | -34.80% | -38.59% |
| Class B | -20.52% | -34.32% | -31.98% |
| Class C | -21.62% | -25.93% | -25.70% |
| Overall | -22.13% | -30.68% | -32.39% |
| Class D | -22.46% | -27.26% | -27.44% |
| Class F | -29.31% | -37.03% | -37.38% |
| Class TGM | -39.16% | -44.92% | -44.87% |
| AI10 | BD-Rate [%] |  |  |
|  | Y | U | V |
| Class A1 | -10.55% | -21.84% | -29.22% |
| Class A2 | -16.68% | -28.64% | -31.54% |
| Class B | -11.03% | -26.59% | -24.84% |
| Class C | -11.07% | -16.55% | -17.07% |
| Class E | -14.59% | -24.38% | -23.05% |
| Overall | -12.49% | -23.54% | -24.66% |
| Class D | -9.17% | -14.34% | -14.52% |
| Class F | -27.16% | -35.63% | -35.47% |
| Class TGM | -43.22% | -50.44% | -49.60% |

# ECM coding performance evolution for V3C content

The tables below summarise the evolution of ECM coding performance from version 8.0 to 10.0 for V3C content.

| ECM10.0 over ECM8.0 |  |  |  |  |
| --- | --- | --- | --- | --- |
| RA10 | BD-Rate [%] |  |  |  |
|  | Y | U | V | Occ Y |
| V-PCC texture | 0.85% | -3.72% | -3.72% | 1.87% |
| V-PCC geometry | 0.97% |  |  | 3.09% |
| MIV texture | -1.93% | -5.23% | -5.23% |  |
| MIV geometry | -5.73% |  |  |  |
| Average | -1.94% | -4.63% | -4.63% | 2.48% |
| AI10 | BD-Rate [%] |  |  |  |
|  | Y | U | V | Occ Y |
| V-PCC texture | -0.92% | -4.71% | -4.71% | -3.60% |
| V-PCC geometry | -0.57% |  |  | -0.97% |
| MIV texture | -2.07% | -5.08% | -5.08% |  |
| MIV geometry | -6.93% |  |  |  |
| Average | -3.00% | -4.93% | -4.93% | -2.28% |

As a comparison, the tables below summarises ECM10.0 coding performance over ECM8.0 for other types of content.

| ECM10.0 over ECM8.0 |  |  |  |
| --- | --- | --- | --- |
| RA10 | BD-Rate [%] |  |  |
|  | Y | U | V |
| Class A1 | -2.94% | -5.59% | -9.73% |
| Class A2 | -4.01% | -8.00% | -8.91% |
| Class B | -2.72% | -7.58% | -7.00% |
| Class C | -2.43% | -3.41% | -3.29% |
| Overall | -2.95% | -6.15% | -6.94% |
| Class D | -2.65% | -3.60% | -3.76% |
| Class F | -4.62% | -6.73% | -6.83% |
| Class TGM | -5.07% | -7.12% | -6.98% |
| AI10 | Y | U | V |
| Class A1 | -1.65% | -3.92% | -5.62% |
| Class A2 | -3.50% | -7.80% | -7.88% |
| Class B | -2.96% | -6.40% | -6.10% |
| Class C | -2.05% | -3.00% | -3.25% |
| Class E | -4.55% | -6.95% | -6.31% |
| Overall | -2.89% | -5.56% | -5.72% |
| Class D | -1.80% | -3.31% | -3.23% |
| Class F | -5.27% | -7.15% | -7.53% |
| Class TGM | -5.83% | -7.53% | -7.42% |

# Summary & Conclusion

This contribution provides coding performance evaluations of ECM10.0 for V3C video content against VTM22.0 and ECM8.0.

Comparing to ECM8.0, ECM10.0 shows improvements in especially for V3C MIV content. We expect the introduction of IBC tools to natural content to have the biggest impact here, as this relates well to the MIV content type. V-PCC content suffers slightly between ECM10.0 and ECM8.0 in RA conditions. We don’t have an explanation for this yet and will explore further.

Comparing the performance of ECM10.0 for V3C content to the numbers achieved in other content classes, there seems to be room for improvement for V3C type of content.

# References

[1]	S. Schwarz, M.M. Hannuksela, „Volumetric video-based coding (V3C) test content”, JVET-AD0044, https://jvet-experts.org/doc_end_user/current_document.php?id=12591

[2]	S. Schwarz, M.M. Hannuksela, „Occupancy-only PSNR calculations for V3C V-PCC coding evaluation”, JVET-AE0092, https://jvet-experts.org/doc_end_user/current_document.php?id=13040

[3] https://mpegfs.int-evry.fr/mpegcontent/ws-mpegcontent/Explorations/V3C

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).