| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0057-v1 |
| --- | --- |

| Title: | EE2-3.5: DMVR with robust MV derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson, R. Yu | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

Multi-pass DMVR can sometimes produce dislocated subblocks due to use of unreliable motion vectors which leads to subjective artifacts. In Test 3.5a a normative approach that include boundary distortion to the selection criteria when there is a risk that a subblock otherwise will derive unreliable motion vector is tested. Test 3.5b is an encoder only solution that punish selection of DMVR when there is a risk that one subblock will have unreliable motion vectors.

Objective performance (BDR) versus ECM-10.0 is reported as:

For CTC at normal QP range:

Test 3.5a RA: Y: -0.01%, U -0.01%, V 0.01%	 Enc:97%* Dec:100.3% VmPeak: 99.0%

Test 3.5b RA: Y:  0.12%, U  0.11%, V 0.11%	 Enc:100%* Dec:101.7% VmPeak: 99.7%

For CTC at higher QP range QP= 40, 43, 47, 50:

Test 3.5a RA: Y -0.08%, U -0.16%, V -0.05%	 Enc: 95%* Dec:101.6% VmPeak: 99.0%

Test 3.5a RA: Y  1.64%, U  1.77%, V  1.82%	 Enc: 97%* Dec:103.6% VmPeak: 99.1%

Encoding time may not be accurate but since decoder performs selection of motion vector for DMVR decoding time is a good estimate of complexity.

It is asserted that both Test 3.5a and Test 3.5b can improve subjective quality but since ECM is work in progress and not a frozen standard it is preferred to have a solution that not penalize the usage of DMVR.

# Introduction

In JVET-AE0108 [1] it was asserted that multi-pass DMVR can produce visual artifacts for block sizes from 32x32 or greater and a normative solution was proposed to be tested in EE2 [2]. This corresponds to Test 3.5a. It was also asked for how it would compare with an encoder only solution like the one in VTM (config param DMVREncSelect). Test 3.5b corresponds to the encoder only solution.

# Proposal

## Test 3.5a

Select motion vector that minimize distortion based on both subblock boundary distortion and subblock matching distortion when there is a risk to have unreliable motion vectors based on checks on spatial activity, subblock motion vector differences and boundary differences. The method consists of following steps:

1.	Check spatial activity for the reference subblocks centered at the block motion vector. If it is determined that the spatial activity is lower than a threshold go to step 2.

2.	Each candidate motion vector for the subblock is compared with a neighboring subblock’s motion vectors across a subblock boundary (if there exists). If the absolute difference between the candidate motion vector and the neighboring subblock’s motion vector for one component is greater than a threshold go to step 3.

3.	A boundary check which is based on neighboring samples is made for the top respectively the left subblock boundary. If the boundary check indicates that there not is a true edge along the top subblock boundary or the left subblock boundary continue to step 4.

4.	Determine subblock boundary distortion across left and/or top subblock boundary. For any subblock boundary distortion which is greater than a threshold, add the subblock boundary distortion to the block matching distortion and go to step 5.

5.	Select motion vectors that minimize the total distortion.

## Test 3.5b

In the encoder only solution the RD cost is set to a maximum value when at least one subblock of a coding unit has unreliable motion vector based on steps 1 to 4 as in Test 3.5a following steps without adding the boundary distortion to the selection criteria. Furthermore, the RD cost is not punished for the highest temporal layer and the method is only used for QP equal to or above 33 to reduce BDR impact and still maintain subjective quality.

# Results

Decoder timings are accurate, average over several runs without MD5 and without writing reconstruction to disk. Encoder timings are not accurate but expected to be similar as decoder timings since DMVR is used on the decoder side.

## CTC [3]

Test 3.5a:

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | 0,00% | -0,02% | 0,00% | 97,3% | 99,9% | 98,7% |
| Class A2 | -0,02% | -0,05% | -0,05% | 97,8% | 100,1% | 98,2% |
| Class B | -0,01% | 0,05% | 0,04% | 97,6% | 100,7% | 99,1% |
| Class C | 0,00% | -0,06% | 0,01% | 94,3% | 100,3% | 99,8% |
| Class E |  |  |  |  |  |  |
| Overall | -0,01% | -0,01% | 0,01% | 96,7% | 100,3% | 99,0% |
| Class D | -0,02% | 0,09% | 0,05% | 98,3% | 100,1% | 100,1% |
| Class F | 0,01% | -0,01% | -0,05% | 99,5% | 102,9% | 99,1% |

Test 3.5b:

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | 0,17% | 0,12% | 0,14% | 99,9% | 101,3% | 99,8% |
| Class A2 | 0,17% | 0,16% | 0,17% | 101,1% | 103,0% | 99,5% |
| Class B | 0,10% | 0,09% | 0,08% | 99,1% | 101,9% | 99,7% |
| Class C | 0,07% | 0,08% | 0,07% | 99,8% | 100,7% | 100,0% |
| Class E |  |  |  |  |  |  |
| Overall | 0,12% | 0,11% | 0,11% | 99,8% | 101,7% | 99,7% |
| Class D | 0,06% | 0,10% | 0,10% | 99,9% | 99,9% | 100,0% |
| Class F | 0,04% | 0,07% | 0,08% | 99,7% | 100,1% | 99,7% |

## QP 40, 43, 47, 50

Test 3.5a:

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 QP 40, 43, 47, 50 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0,07% | -0,14% | -0,04% | 96,2% | 101,1% | 98,6% |
| Class A2 | -0,10% | -0,16% | -0,07% | 97,8% | 101,7% | 98,7% |
| Class B | -0,06% | -0,13% | 0,07% | 93,1% | 101,9% | 98,7% |
| Class C | -0,08% | -0,23% | -0,17% | 96,1% | 101,5% | 99,8% |
| Class E |  |  |  |  |  |  |
| Overall | -0,08% | -0,16% | -0,05% | 95,4% | 101,6% | 99,0% |
| Class D | -0,11% | 0,03% | -0,03% | 92,2% | 103,2% | 100,2% |
| Class F | 0,06% | -0,01% | -0,04% | 96,1% | 102,4% | 98,9% |

Test 3.5b:

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 QP 40, 43, 47, 50 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | 1,68% | 1,68% | 1,83% | 98,5% | 102,9% | 98,8% |
| Class A2 | 2,13% | 1,93% | 2,12% | 103,1% | 114,7% | 98,8% |
| Class B | 1,59% | 1,89% | 1,84% | 94,6% | 102,0% | 98,7% |
| Class C | 1,32% | 1,58% | 1,57% | 96,4% | 98,6% | 99,8% |
| Class E |  |  |  |  |  |  |
| Overall | 1,64% | 1,77% | 1,82% | 97,5% | 103,6% | 99,1% |
| Class D | 1,34% | 1,59% | 1,33% | 98,1% | 99,9% | 100,2% |
| Class F | 0,72% | 0,72% | 0,57% | 97,5% | 98,8% | 98,9% |

## Additional sequences QP 40, 43, 47, 50

Test 3.5a:

|  | BDR Y | BDR U | BDR V |
| --- | --- | --- | --- |
| DrivingPOV3_3840x2160_60fps_10b | -0,02% | -0,07% | -0,15% |
| MountainBay2_3840x2160_30fps_42 | -0,02% | -0,02% | 0,05% |
| TallBuildings2_3840x2160_30fps_ | -0,01% | -0,04% | 0,06% |
| CorsairJoy_3840x2160_25p_420_10 | -0,04% | 0,29% | -0,02% |
| Procession_3840x2160_60p_420_10 | -0,15% | -0,13% | -0,24% |
| Marathon2_3840x2160_30fps_10bit | -0,10% | -0,14% | -0,33% |
| Fortnite_Part1_3840x2160_60fps_ | 0,00% | -0,04% | 0,14% |
| Average | -0,05% | -0,02% | -0,07% |

Test 3.5b:

|  | BDR Y | BDR U | BDR V |
| --- | --- | --- | --- |
| DrivingPOV3_3840x2160_60fps_10b | 2,76% | 2,83% | 3,16% |
| MountainBay2_3840x2160_30fps_42 | 2,63% | 2,45% | 2,37% |
| TallBuildings2_3840x2160_30fps_ | 0,79% | 0,68% | 0,67% |
| CorsairJoy_3840x2160_25p_420_10 | 2,20% | 2,72% | 2,21% |
| Procession_3840x2160_60p_420_10 | 1,54% | 1,69% | 1,57% |
| Marathon2_3840x2160_30fps_10bit | 2,14% | 2,32% | 2,01% |
| Fortnite_Part1_3840x2160_60fps_ | 0,85% | 0,94% | 1,06% |
| Average | 1,85% | 1,95% | 1,86% |

## Visual example

Figure 1: CatRobot at QP 47, POC 228, Left: ECM-10.0, Right: Test 3.5a

Figure 2: CatRobot at QP 47, POC 228, Left: ECM-10.0, Right: Test 3.5b

# Conclusion

It has been shown that the impact of the normative solution Test 3.5a is minor on objective performance but that the benefit increases for higher QPs where DMVR is used more often. The encoder only solution, Test 3.5b, has a significant drop in BDR, especially for higher QP. It is asserted that both Test 3.5a and Test 3.5b can improve subjective quality.

It is suggested to adopt the normative solution Test 3.5a to next version of ECM.

# References

[1] K. Andersson, R. Yu, “AHG12: DMVR with robust MV derivation”, JVET-AE0108, Geneva, Switzerland, July 2023.

[2] V. Seregin, et.al, “Exploration experiment on enhanced compression beyond VVC capability (EE2)”, JVET-AE2024, Geneva, Switzerland, July 2023.

[3] M. Karczewice, Y. Yan, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AE2017, Geneva, Switzerland, July 2023.

[4] ECM-10.0, available at https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-10.0

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).