| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting: Hannover, Germany, 13–20 October 2023 | Document: JVET-AF0067-v1 |
| --- | --- |

| Title: | AHG9: On the SEI processing order SEI message (Part 2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lulin Chen Olena Chubach Yu-Wen Huang Shawmin Lei | Email: | {lulin.chen, olena.chubach, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to restructure the SEI processing order SEI message in JVET-AE2027 into SEI processing order group characteristic SEI message and SEI processing order group activation SEI message with the following brief description.

- The SEI processing order group characteristic SEI message carries information indicating the preferred initial processing order type for different types of SEI messages that may be present in a CVS. The use of SEI processing order groups combined with any other types of SEI messages that may be present in a CVS for specific pictures is indicated with SEI processing order group activation SEI messages indicating the preferred final processing order types.

- The SEI processing order group activation SEI message activates or de-activates the use of the combination of target SEI messages and/or target SEI processing order groups with the preferred final processing order type for processing of a set of pictures.

- When no SEI processing order groups specified by SEI processing order group characteristic SEI messages rather than SEI messages indicated in the activation message are present for activation, the SEI processing order group activation SEI message may be used without a preceding SPOGC SEI message.

# Introduction

In a separate contribution JVET-AF0065, the scope of persistence, processing order type, processing purpose, and support of neural network post filter (NNPF) SEI messages are, as proposed, added to JVET-AE2027 - SEI processing order SEI message in VVC (draft 5).

The proposed additions in JVET-AF0065 with Option 2 are enclosed (briefly highlighted) for discussion convenience.

| sei_processing_order( payloadSize ) { |  | Descriptor |  |
| --- | --- | --- | --- |
| po_cancel_flag |  | u(1) |  |
| if( !po_cancel_flag ) { |  |  |  |
| po_persistence_flag |  | u(1) |  |
| po_purpose |  | u(8) |  |
| po_type |  | u(4) |  |
| reserved_alignment_2bits |  | u(2) |  |
| po_num_sei_messages_minus2 |  | u(8) |  |
| for( i = 0; i < po_num_sei_messages_minus2 + 2; i++) { |  |  |  |
| po_sei_wrapping_flag[ i ] |  | u(1) |  |
| po_sei_importance_flag[ i ] |  | u(1) |  |
| if( po_sei_wrapping_flag[ i ] ) { |  |  |  |
| reserved_alignment_6bits |  | u(6) |  |
| sei_message( ) |  |  |  |
| sei_activation_present_flag[ i ] |  | u(1) |  |
| if( sei_activation_present_flag[ i ] ) { |  |  |  |
| reserved_alignment_7bit |  | u(7) |  |
| sei_message( ) //The corresponding activation SEI message, if applicable. |  |  |  |
| } |  |  |  |
| } else { |  |  |  |
| po_sei_prefix_flag[ i ] |  | u(1) |  |
| … |  |  |  |
| sei_activation_present_flag[ i ] |  | u(1) |  |
| if( sei_activation_present_flag[ i ] ) { |  |  |  |
| reserved_alignment_7bit |  | u(7) |  |
| sei_message( ) //The corresponding activation SEI message, if applicable. |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table X.1 – po_purpose values

| Value | Description |
| --- | --- |
| 0 | Unspecified or specified by external means |
| 1 | General visual quality improvement |
| 2 | Processing for human viewing |
| 3 | Processing for machine |

Table X.2 – Definition of po_type

| Value | Description |
| --- | --- |
| 0 | Unspecified or Option – Application is optional |
| 1 | Cascade - The indicated SEI messages in an SPO SEI message with po_type is applied in the sequential order with the index i, which means in a way that the indicated SEI messages are ordered with increasing values of the index i for processing. |
| 2 | Alternate – The indicated SEI messages in an SPO SEI message with po_type is applied alternatively (only one is chosen). |
| 3 | Parallel – The indicated SEI messages in an SPO SEI message withs po_type is applied in parallel. |

sei_activation_present_flag[ i ] equal to 1 indicates the activation SEI message following the i-th SEI message is present. sei_activation_present_flag[ i ] equal to 0 indicates the activation SEI message following the i-th SEI message is not present. The activation SEI message, when present, is applicable for the i-th SEI message as the activation for use.

…

# Proposal

Like the NNPF SEI messages and the NNPFG SEI messages in JVET-AE2032 (TuC), it is proposed to restructure the SEI processing order SEI message by specifying the SEI processing order group characteristic SEI message and the SEI processing order group activation SEI message accordingly. And can be described as:

- The SEI processing order group characteristic SEI message carries information indicating the preferred initial processing order type for different types of SEI messages that may be present in  a CVS. The use of SEI processing order groups combined with any other types of SEI messages that may be present in a CVS for specific pictures is indicated with SEI processing order group activation SEI messages indicating the preferred final processing order types.

- The SEI processing order group activation SEI message activates or de-activates the use of the combination of target SEI messages and/or target SEI processing order groups with the preferred final processing order type for processing of a set of pictures.

- When no SEI processing order groups specified by SEI processing order group characteristic SEI messages rather than SEI messages indicated in the activation message are present for activation, the SEI processing order group activation SEI message may be used without a preceding SPOGC SEI message. This is equivalent to the SEI processing order SEI message in JVET-AE2027 with the proposed changes in JVET-AF0065 (also briefed in the above introduction).

As described above, it allows to indicate a combined processing order type, as additional functionality as commented in the last meeting, for processing a set of pictures, e.g., a Cascade processing order type with an Alternate processing order type.

The following sections are mainly for the changes over the revised SEI processing order SEI message in JVET-AF0065 for specifying the SEI processing order group characteristic SEI message and the SEI processing order group activation SEI message.

## SEI processing order group characteristic SEI message

| sei_processing_order_group_characteristic( payloadSize ) { |  | Descriptor |  |
| --- | --- | --- | --- |
| spogc_id |  | u(8) |  |
| spogc_purpose |  | u(8) |  |
| spogc_type |  | u(4) |  |
| spogc_num_sei_messages_minus2 |  | u(8) |  |
| for( i = 0; i < spogc_num_sei_messages_minus2 + 2; i++) { |  |  |  |
| po_sei_wrapping_flag[ i ] |  | u(1) |  |
| po_sei_importance_flag[ i ] |  | u(1) |  |
| if( po_sei_wrapping_flag[ i ] ) { |  |  |  |
| reserved_alignment_6bits |  | u(6) |  |
| sei_message( ) |  |  |  |
| sei_activation_present_flag[ i ] |  | u(1) |  |
| if( sei_activation_present_flag[ i ] ) { |  |  |  |
| reserved_alignment_7bit |  | u(7) |  |
| sei_message( ) //The corresponding activation SEI message, if applicable. |  |  |  |
| } |  |  |  |
| } else { |  |  |  |
| po_sei_prefix_flag[ i ] |  | u(1) |  |
| … |  |  |  |
| sei_activation_present_flag[ i ] |  | u(1) |  |
| if( sei_activation_present_flag[ i ] ) { |  |  |  |
| reserved_alignment_7bit |  | u(7) |  |
| sei_message( ) //The corresponding activation SEI message, if applicable. |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

The SEI processing order group (SPOG) characteristic (SPOGC) SEI message carries information indicating the preferred initial processing order type, as determined by the encoder (i.e., the content producer), for different types of SEI messages that may be present in a CVS. The use of combination of SPOGs, identified by spogc_id, and/or other SEI messages for specific pictures is indicated with SEI processing order group activation SEI messages, indicating the preferred final processing order types.

When an activation SEI message is indicated in an SPOGC SEI message, the activation SEI message is not associated with any picture in a CVS and targeted SEI message inside or outside the SPOGC SEI message for activation shall precede the activation SEI message in decoding order.

NOTE: an example of activation SEI message and targeted SEI message is the neural network post filter (NNPF) activation SEI message and the NNPF characteristic SEI message.

It is a requirement of bitstream conformance that, within an SPOGC SEI message, there shall be at least two of the syntax elements po_sei_payload_type[ i ].

The SPOGC SEI message can carry one or more SEI prefix indications of a particular payloadType. Each SEI prefix indication is a byte string that follows the SEI payload syntax of that value of payloadType and contains a number of complete syntax elements starting from the first syntax element in the SEI payload, and may be followed by bits that do not represent any complete syntax element of the SEI payload.

These SEI prefix indications should provide sufficient information to determine the specific processing order for SEI messages having the same value of payloadType but different preferred processing order.

spogc_id contains an identifying number that may be used to identify an SPOG. The value of spogc_id shall be in the range of 0 to 255. The value of spogc_id shall be unique and different from other values of spogc_id in the bitstream.

spogc_purpose indicates the purpose of the SPOG and has the semantics of spogc_purpose as specified in Table X1. The value of spogc_purpose shall be in the range of 0 to 3, inclusive, in bitstreams conforming to this edition of this document. Values in the range of 4 to 255, inclusive, for spogc_purpose are reserved for future use by ITU T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders shall ignore SEI processing order SEI messages with the value of spogc_purpose in the range of 4 to 255, inclusive.

Table X1 – Definition of spogc_purpose and spoga_purpose

| Value | Description |
| --- | --- |
| 0 | Unspecified or specified by external means |
| 1 | General visual quality improvement |
| 2 | Processing for human viewing |
| 3 | Processing for machine |

Table X2 – Definition of spogc_type and spoga_type

| Values of spogc_type and spoga_type | Description |
| --- | --- |
| 0 | Unspecified |
| 1 | Cascade - The SEI messages in an SPOGC SEI message with spogc_type, or the SPOGs and/or the SEI messages in an SPOGA SEI message with spoga_type is applied in the sequential order with the index i, which means in a way that the indicated SEI messages and/or SPOGs are ordered with increasing values of the index i. |
| 2 | Alternate – The SEI messages in an SPOGC SEI message with spogc_type, or the SPOGs and/or the SEI messages in an SPOGA SEI message with spoga_type is applied alternatively (only one is chosen). |
| 3 | Parallel – The SEI messages in an SPOGC SEI message withs pogc_type, or the SPOGs and/or the SEI messages in an SPOGA SEI message with spoga_type is applied in parallel. |
| 4 -7 | Reserved |

spogc_type indicates the processing order type for the SPOG and has the semantics of spogc_type as specified in Table X2. The value of spogc_type shall be in the range of 0 to 7, inclusive, in bitstreams conforming to this edition of this document. Values of 4 to 7, inclusive, for spogc_type are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore SPOGC SEI messages with spogc_type in the range of 4 to 7, inclusive.

spogc_num_sei_messages_minus2 plus 2 indicates the number of SEI messages that have a processing order type spogc_type as specified in Table X2 indicated in the SPOGC SEI message.

po_sei_importance_flag[ i ] indicates the degree of importance determined by the encoder for the SEI message with index i.

If the decoding system cannot interpret or does not support any indicated SEI message that has po_sei_importance_flag[ i ] equal to 1, it should ignore the entire SEI processing order SEI message.

reserved_alignment_6bits has no meaning and shall be equal to 0 in bitstreams conforming to this version of this Specification. Decoders shall allow this syntax element to have other values and shall ignore the value.

sei_activation_present_flag[ i ] equal to 1 indicates the activation SEI message following the i-th SEI message is present. sei_activation_present_flag[ i ] equal to 0 indicates the activation SEI message following the i-th SEI message is not present. The activation SEI message, when present, is applicable for the i-th SEI message as the activation for use.

If po_sei_wrapping_flag[ i ] is equal to 0, an SEI message should be present outside…

…

## SEI processing order group activation SEI message

| sei_processing_order_group_activation( payloadSize ) { |  | Descriptor |  |
| --- | --- | --- | --- |
| spoga_cancel_flag |  | u(1) |  |
| if( !spoga_cancel_flag ) { |  |  |  |
| spoga_persistence_flag |  | u(1) |  |
| spoga_purpose |  | u(8) |  |
| spoga_type |  | u(4) |  |
| spoga_num_sei_messages_minus1 |  | u(8) |  |
| for( i = 0; i < spoga_num_sei_messages_minus1 + 1; i++) { |  |  |  |
| spoga_member_id_present_flag[ i ] |  | u(1) |  |
| if( spoga_member_id_present_flag[ i ] ) |  |  |  |
| spoga_member_id[ i ] |  | u(8) |  |
| else { |  |  |  |
| po_sei_wrapping_flag[ i ] |  | u(1) |  |
| po_sei_importance_flag[ i ] |  | u(1) |  |
| if( po_sei_wrapping_flag[ i ] ) { |  |  |  |
| reserved_alignment_6bits |  | u(6) |  |
| sei_message( ) |  |  |  |
| sei_activation_present_flag[ i ] |  | u(1) |  |
| if( sei_activation_present_flag[ i ] ) { |  |  |  |
| reserved_alignment_7bit |  | u(7) |  |
| sei_message( ) //The corresponding activation SEI message, if applicable. |  |  |  |
| } |  |  |  |
| } else { |  |  |  |
| po_sei_prefix_flag[ i ] |  | u(1) |  |
| … |  |  |  |
| sei_activation_present_flag[ i ] |  | u(1) |  |
| if( sei_activation_present_flag[ i ] ) { |  |  |  |
| reserved_alignment_7bit |  | u(7) |  |
| sei_message( ) //The corresponding activation SEI message, if applicable. |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

The SEI processing order group activation (SPOGA) SEI message activates or de-activates the use of the combination of target SEI messages, identified with the index i, and/or target SEI processing order groups (SPOGs), identified by spoga_member_id[ i ], for i in the range of 0 to spoga_num_sei_messages_minus1, inclusive, with the preferred processing order type, as determined by the encoder (i.e., the content producer), for processing of a set of pictures.

For a particular picture for which the SPOGs are activated, the target SPOGs are the SPOGs, consisting of the applied SEI messages, specified by the SPOG characteristic (SPOGC) SEI messages, identified by spoga_member_id[ i ], for i in the range of 0 to spoga_num_sei_messages_minus1, inclusive, that precedes the first VCL NAL unit of the current picture in decoding order and the SPOGs of the target SPOGs are defined by the SPOGC SEI messages that have spogc_id equal to any spoga_member_id[ i ] value of the target SPOGs and are present in the current picture unit or precede the current picture in decoding order.

For a particular picture for which the SEI messages are activated, the target SEI messages are the SEI messages indicated in this SEI message with the index i, for i in the range of 0 to spoga_num_sei_messages_minus1, inclusive.

It is a requirement of bitstream conformance that, within an SPOGA SEI message, when no SPOGs are specified by SPOGC SEI messages, there shall be at least two of the syntax elements po_sei_payload_type[ i ].

The SPOGA SEI message can carry one or more SEI prefix indications of a particular payloadType. Each SEI prefix indication is a byte string that follows the SEI payload syntax of that value of payloadType and contains a number of complete syntax elements starting from the first syntax element in the SEI payload, and may be followed by bits that do not represent any complete syntax element of the SEI payload.

These SEI prefix indications should provide sufficient information to determine the specific processing order for SEI messages having the same value of payloadType but different preferred processing order.

spoga_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of the combination of target SEI messages and/or target SPOGs specified by any previous SPOGA SEI message in output order that applies to the current layer. spoga_cancel_flag equal to 0 indicates that the combination of target SEI messages and/or target SPOGs are activated for use and the information follows.

spoga_persistence_flag specifies the persistence of the combination of target SEI messages and/or target SPOGs for the current layer. The persistence flag value in any of the indicated SEI messages and/or SPOGC SEI messages shall have the same value as the value of spoga_persistence_flag.

NOTE – Some SEI messages may not have the persistence flag syntax element, thus the scope of persistence of those SEI messages is indicated by spoga_persistence_flag in the SPOGA SEI message.

spoga_persistence_flag equal to 0 specifies that the combination of target SEI messages and/or target SPOGs applies to the current decoded picture only

spoga_persistence_flag equal to 1 specifies that the combination of target SEI messages and/or target SPOGs applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a SPOGA SEI message that follows the current picture in output order.

spoga_purpose indicates the purpose of the SPOGs and/or SEI messages and has the semantics of spoga_purpose as specified in Table X1. The value of spoga_type shall be in the range of 0 to 255, inclusive, in bitstreams conforming to this edition of this document. Values in the range of 4 to 255, inclusive, for spoga_purpose are reserved for future use by ITU T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders shall ignore SEI processing order SEI messages with the value of spoga_purpose in the range of 4 to 255, inclusive.

spoga_type indicates the SEI processing order type for the SPOGs and/or SEI messages and has the semantics of spoga_type as specified in Table X2. The value of spoga_type shall be in the range of 0 to 7, inclusive, in bitstreams conforming to this edition of this document. Values of 4 to 7, inclusive, for spoga_type are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore SPOGA SEI messages with spoga_type in the range of 4 to 7, inclusive.

spoga_num_sei_messages_minus1 plus 1 indicates the combined number of SEI messages and/or SPOGs that have a processing order type spoga_type as specified in Table X2 indicated in the SPOGA SEI message.

spoga_member_id_present_flag[ i ] equal to 1 specifies that spoga_member_id[ i ] is present. spoga_member_id_present_flag[ i ] equal to 0 specifies spoga_member_id[ i ] is not present.

NOTE – when spoga_member_id_present_flag[ i ] equal to 0 for i in the range of 0 to spoga_num_sei_messages_minus1, incluisive, it indicates that no SPOGs specified by SPOGC SEI messages rather than the SEI messages indicated in this message are present for activation. In this case, the SPOGA SEI message may be used without a preceding SPOGC SEI message and spoga_num_sei_messages_minus1 should be greater than 0.

spoga_member_id[ i ] when present, indicates that the i-th SPOG indicated in this SPOGA SEI message has spogc_id equal to spoga_member_id[ i ]. Each SPOG with spogc_id equal to spoga_member_id[ i ] is defined by an SPOGC SEI message.

po_sei_importance_flag[ i ] indicates the degree of importance determined by the encoder for the SEI message with index i.

If the decoding system cannot interpret or does not support any indicated SEI message that has po_sei_importance_flag[ i ] equal to 1, it should ignore the entire SEI processing order SEI message.

reserved_alignment_6bits has no meaning and shall be equal to 0 in bitstreams conforming to this version of this Specification. Decoders shall allow this syntax element to have other values and shall ignore the value.

sei_activation_present_flag[ i ] equal to 1 indicates the activation SEI message following the i-th SEI message is present. sei_activation_present_flag[ i ] equal to 0 indicates the activation SEI message following the i-th SEI message is not present. The activation SEI message, when present, is applicable for the i-th SEI message as the activation for use.

If po_sei_wrapping_flag[ i ] is equal to 0, an SEI message should be present outside…

…

When, as in the above note, spoga_member_id_present_flag[ i ] equal to 0 for i in the range of 0 to spoga_num_sei_messages_minus1, inclusive, it indicates that no SPOGs specified by SPOGC SEI messages rather than the SEI messages indicated in the SPOGA message are present for activation. In this case, the SPOGA SEI message may be used without a preceding SPOGC SEI message and spoga_num_sei_messages_minus1 should be greater than 0. This is equivalent to the SEI processing order SEI message in JVET-AF0065. In another case that no SEI messages are indicated in the SPOGA message rather than SPOGs specified by SPOGC SEI messages, the SPOGA SEI message groups only the activated SPOGs or a single SPOG with a final processing order type for processing of a set of pictures.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).