| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0079-v2 |
| --- | --- |

| Title: | EE2-2.6: IntraTMP block vector storing |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Yu, Lai Zhang, Luhang Xu, Haoping Yu, Jonathan Gan, Fan Wang Zhihuang Xie, Dong Wang (OPPO) Po-Han Lin, Jian-Liang Lin, Vadim Seregin, Marta Karczewicz (Qualcomm) | Email: | yue.yu@oppo.com pohanlin@qti.qualcomm.com |
| Source: | OPPO, Qualcomm |  |  |

_____________________________

# Abstract

This contribution reports the EE2-2.6 test results for IntraTMP block vector (BV) storing. In ECM-10.0, an intraTMP BV may have a quarter-pel resolution but it is always saved in a full-pel resolution after coding an intraTMP block. JVET-AE0075 and JVET-AE0124 proposed to store IntraTMP BV in a quarter-pel resolution. In addition, it was also proposed that intraTMP BV is stored for HMVP. On top of ECM-10.0, the simulation results are reported as follows:

Test A: storing quarter-pel resolution BVs

AI: 	-0.00% (Y), -0.04% (U), -0.02% (V), 99.6% (EncT), 99.8% (DecT) for natural content

-0.06% (Y), -0.05% (U), 0.03% (V), 98.1% (EncT), 99.8% (DecT) for class F

-0.06% (Y), -0.10% (U), -0.11% (V), 98.8% (EncT), 99.3% (DecT) for class TGM

RA: 	0.00% (Y),  0.01% (U), -0.01% (V), 99.9% (EncT), 100.4% (DecT) for natural content

-0.03% (Y), 0.11% (U), 0.04% (V), 99.6% (EncT), 100.8% (DecT) for class F

-0.07% (Y), -0.03% (U), -0.06% (V), 99.5% (EncT), 100.6% (DecT) for class TGM

Test B: full-pel resolution BV stored for HMVP

AI: 	-0.02% (Y), 0.01% (U), -0.07% (V), 100.1% (EncT), 99.8% (DecT) for natural content

-0.03% (Y), 0.04% (U), 0.04% (V), 99.2% (EncT), 100.6% (DecT) for class F

0.01% (Y), -0.02% (U), -0.01% (V), 99.4% (EncT), 99.3% (DecT) for class TGM

RA: 	0.00% (Y), 0.06% (U), -0.02% (V), 100.3% (EncT), 99.9% (DecT) for natural content

0.00% (Y), -0.01% (U), 0.04% (V), 99.7% (EncT), 100.0% (DecT) for class F

-0.09% (Y), -0.05% (U), -0.02% (V), 99.6% (EncT), 101.1% (DecT)for class TGM

Test C: Test A + Test B with quarter-pel resolution

AI: 	-0.02% (Y), -0.01% (U), -0.05% (V), 100.5% (EncT), 100.1% (DecT) for natural content

-0.07% (Y), -0.02% (U), 0.02% (V), 99.5% (EncT), 99.8% (DecT) for class F

-0.07% (Y), -0.13% (U), -0.06% (V), 99.0% (EncT), 99.4% (DecT) for class TGM

RA: 	0.00% (Y), 0.00% (U), -0.14% (V), 100.4% (EncT), 100.4% (DecT) for natural content

0.01% (Y), 0.11% (U), 0.07% (V), 100.8% (EncT), 100.0% (DecT) for class F

-0.07% (Y), -0.01% (U), 0.00% (V), 99.9% (EncT), 100.9% (DecT) for class TGM

# Proposed method

IntraTMP employs a pre-defined template of the current block to search for a candidate template of the same shape that best matches the template of the current block within a pre-determined search area. In order to search for the best template, a cost function, e.g., SAD is defined and both the encoder and decoder use the same search strategy to obtain the best matched block. Once the best matched block is found, the corresponding reference block indicated by a block vector (BV) will be selected as the prediction block for coding the current block. In ECM-10.0, IntraTMP BVs are always stored for the purpose of coding future IBC blocks, after coding the block in the IntraTMP mode. However, IntraTMP BV is only saved in a full pel resolution, even though some of the IntraTMP BVs may have a quarter-pel resolution. In addition, intraTMP BV is not stored for HMVP.

In ECM-10.0, if the current block is coded as IBC mode, the IBC BV is always stored in a quarter-pel resolution and IBC BV is also stored for HMVP. In order to have a consistent design, this contribution proposes to always save IntraTMP BV in a quarter-pel resolution, instead of a full-pel resolution, after coding an intraTMP block. In addition, quarter-pel IntraTMP BV is also stored for HMVP for coding future blocks.

# Simulation results

The proposed method is implemented on top of the ECM-10.0 software [1] and tested based on the common test condition [2]. Three tests are conducted. Test A is to store quarter-pel resolution IntraTMP BV. Test B is to store full-pel IntraTMP BV for HMVP. Test C is a combination test of Test A and Test B with quarter-pel resolution. The test results are summarized in the following Tables.

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | 0.00% | -0.06% | 0.02% | 99.6% | 100.0% | 100.0% |
| Class A2 | -0.01% | -0.05% | -0.04% | 99.6% | 99.9% | 100.0% |
| Class B | 0.00% | -0.04% | -0.02% | 99.7% | 99.9% | 100.0% |
| Class C | 0.00% | -0.04% | 0.07% | 99.6% | 99.2% | 100.0% |
| Class E | 0.00% | 0.00% | -0.15% | 99.6% | 100.3% | 100.0% |
| Overall | 0.00% | -0.04% | -0.02% | 99.6% | 99.8% | 100.0% |
| Class D | 0.01% | -0.06% | -0.05% | 98.7% | 100.7% | 100.0% |
| Class F | -0.06% | -0.05% | 0.03% | 98.1% | 99.8% | 100.0% |
| Class TGM | -0.06% | -0.10% | -0.11% | 98.8% | 99.3% | 100.0% |

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.02% | -0.10% | -0.09% | 99.6% | 100.8% | 100.3% |
| Class A2 | 0.05% | -0.02% | -0.10% | 100.3% | 100.7% | 99.8% |
| Class B | -0.01% | 0.02% | 0.14% | 99.8% | 100.2% | 99.8% |
| Class C | 0.00% | 0.09% | -0.07% | 99.9% | 100.2% | 100.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.00% | 0.01% | -0.01% | 99.9% | 100.4% | 100.0% |
| Class D | -0.05% | 0.02% | 0.14% | 99.9% | 100.8% | 100.0% |
| Class F | -0.03% | 0.11% | 0.04% | 99.6% | 100.8% | 100.2% |
| Class TGM | -0.07% | -0.03% | -0.06% | 99.5% | 100.6% | 100.0% |

Table 1. Test A: storing quarter-pel resolution IntraTMP BVs

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | 0.01% | 0.06% | -0.15% | 100.2% | 100.8% | 100.0% |
| Class A2 | -0.02% | 0.01% | -0.08% | 100.0% | 99.7% | 100.2% |
| Class B | -0.03% | -0.02% | -0.09% | 100.0% | 100.2% | 100.1% |
| Class C | -0.04% | -0.03% | 0.08% | 100.3% | 99.0% | 100.0% |
| Class E | -0.01% | 0.06% | -0.14% | 100.0% | 99.4% | 100.0% |
| Overall | -0.02% | 0.01% | -0.07% | 100.1% | 99.8% | 100.1% |
| Class D | 0.00% | 0.04% | -0.07% | 100.6% | 101.2% | 100.0% |
| Class F | -0.03% | 0.04% | 0.04% | 99.2% | 100.6% | 100.0% |
| Class TGM | 0.01% | -0.02% | -0.01% | 99.4% | 99.3% | 100.0% |

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | 0.03% | 0.04% | 0.16% | 100.5% | 99.5% | 99.8% |
| Class A2 | 0.04% | -0.01% | 0.04% | 100.4% | 100.4% | 99.8% |
| Class B | -0.02% | 0.18% | 0.01% | 100.3% | 100.1% | 99.9% |
| Class C | -0.01% | -0.01% | -0.25% | 100.1% | 99.6% | 100.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.00% | 0.06% | -0.02% | 100.3% | 99.9% | 99.9% |
| Class D | -0.01% | 0.10% | -0.03% | 100.7% | 100.1% | 100.0% |
| Class F | 0.00% | -0.01% | 0.04% | 99.7% | 100.0% | 99.9% |
| Class TGM | -0.09% | -0.05% | -0.02% | 99.6% | 101.1% | 100.0% |

Table 2. Test B: IntraTMP BV storing for HMVP

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | 0.00% | 0.06% | -0.02% | 99.9% | 100.2% | 100.1% |
| Class A2 | -0.02% | -0.06% | -0.09% | 100.6% | 99.8% | 100.1% |
| Class B | -0.03% | -0.01% | -0.11% | 100.6% | 100.3% | 100.1% |
| Class C | -0.04% | 0.01% | -0.01% | 100.6% | 100.1% | 100.0% |
| Class E | -0.02% | -0.08% | -0.02% | 100.7% | 100.2% | 100.0% |
| Overall | -0.02% | -0.01% | -0.05% | 100.5% | 100.1% | 100.1% |
| Class D | 0.00% | -0.12% | -0.03% | 100.7% | 101.3% | 100.0% |
| Class F | -0.07% | -0.02% | 0.02% | 99.5% | 99.8% | 100.0% |
| Class TGM | -0.07% | -0.13% | -0.06% | 99.0% | 99.4% | 100.0% |

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | 0.02% | -0.04% | -0.02% | 100.4% | 100.0% | 100.2% |
| Class A2 | 0.03% | -0.20% | -0.02% | 100.5% | 100.3% | 100.0% |
| Class B | -0.04% | 0.16% | -0.17% | 100.4% | 100.6% | 100.3% |
| Class C | 0.01% | -0.03% | -0.29% | 100.2% | 100.7% | 100.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.00% | 0.00% | -0.14% | 100.4% | 100.4% | 100.1% |
| Class D | -0.02% | -0.29% | -0.05% | 99.9% | 99.3% | 100.0% |
| Class F | 0.01% | 0.11% | 0.07% | 100.8% | 100.0% | 100.0% |
| Class TGM | -0.07% | -0.01% | 0.00% | 99.9% | 100.9% | 100.0% |

Table 3. Test C: Test A + Test B with quarter-pel resolution

# Conclusions

This contribution reports the results of EE2-2.6. EE2-2.6 proposes a consistent design with IBC and brings 0.02% coding gain for natural content and 0.07% coding gain for both class F and TGM without bringing any additional complexity for AI. It is suggested to adopt the proposed method to ECM.

# References

- ECM-10.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-10.0

- V. Seregin, J. Chen, R. Chernyak, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)”, JVET-AE2024, July 2023.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AE2017, April, 2023.

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).