| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0091-v1 |
| --- | --- |

| Title: | [AHG9] On design for signalling purpose information for NNPFGC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Junghak Nam Seung hwan Kim Jaehyun Lim | Email: | dr.hendry@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

It is asserted that the design for signalling purpose information in neural-network post-filter group characteristic (NNPFGC) SEI message may benefit from some modifications for improvement. The proposed modifications are as follows:

- Add the following definition:

  - Direct member of an NNPFGC: An NNPFC with nnpfc_id equal to nnpfgc_member_id[ i ] for any i in the range of 0 to nnpfgc_num_members_minus2 + 1, inclusive.

  - Indirect member of an NNPFGC: An NNPFC that is a direct or indirect member of another NNPFGC nnpfgcB, in which the value of nnpfgc_id of nnpfgcB is equal to nnpfgc_member_id[ i ] for any i in the range of 0 to nnpfgc_num_members_minus2 + 1, inclusive.

- Clarify that the value of nnpfgc_purpose in NNPFGC, when present, includes all purposes of NNPFC or NNPFGC included in the NNPFGC.

- When nnpfgc_grouping_type is equal to 1 (i.e., alternative grouping) or 3 (i.e., parallel grouping), purpose information is signalled for each member of the NNPFGC. This purpose information may be signalled as nnpfgc_member_purpose[ i ] and the following applies:

  - If nnpfgc_member_id[ i ] is equal to nnpfc_id of a particular NNPFC, the value of nnpfgc_member_purpose[ i ] shall be equal to the value of nnpfc_purpose of the NNPFC with nnpfc_id is equal to nnpfgc_member_id[ i ].

  - Otherwise (i.e., nnpfgc_member_id[ i ] is equal to nnpfgc_id of a particular NNPFGC), the value of nnpfgc_member_purpose[ i ] shall be the union of the values of nnpfgc_member_purpose[ i ] in the NNPFGC.

# Problem Statement

In the 31st JVET meeting, the first design of neural-network post-filter group characteristic (NNPFGC) and neural-network post-filter group activation (NNPFGA) SEI messages were agreed to be included in the Technologies under consideration for future extensions of VSEI (JVET-AE2032). In this first design, nnpfgc_purpose is signalled only when the nnpfgc_grouping_type is equal to 0 (i.e., cascade) or 2 (i.e., parallel).

It is asserted that the signaling design of purpose information in NNPFGC has problems as follow:

- It is not clear if nnpfgc_purpose is constrained to include purposes of all its member NNPFC / NNPFGC. While authors assume this is the intent but not sure if the text clearly describes it.

- Having one aggregate purposes (i.e., nnpfgc_purpose which combines purposes of all member NNPFC / NNPFGC) does not provide much value since it is desirable if we know the purpose(s) of each NNPFC / NNPFGC before making decision whether or not to apply it.

- Having no purpose signaling at all when nnpfgc_grouping_type is not equal to 0 or 2 (e.g., when it is equal to 1 which is alternative case) may not be a good design. Decoder may still want to know the purpose of each NNPFC / NNPFGC member when they are alternative to each other which may help in making decision before using the filters.

It would be desirable if the design for signaling purpose information in NNPFGC is improved to address the asserted problems above.

# Proposal

We propose the following modification to address the asserted problems above:

- Add the following definition:

  - Direct member of an NNPFGC: An NNPFC with nnpfc_id equal to nnpfgc_member_id[ i ] for any i in the range of 0 to nnpfgc_num_members_minus2 + 1, inclusive.

  - Indirect member of an NNPFGC: An NNPFC that is a direct or indirect member of another NNPFGC nnpfgcB, in which the value of nnpfgc_id of nnpfgcB is equal to nnpfgc_member_id[ i ] for any i in the range of 0 to nnpfgc_num_members_minus2 + 1, inclusive.

- Clarify that the value of nnpfgc_purpose in NNPFGC, when present, includes all purposes of NNPFC or NNPFGC included in the NNPFGC.

- When nnpfgc_grouping_type is equal to 1 (i.e., alternative grouping) or 3 (i.e., parallel grouping), purpose information is signalled for each member of the NNPFGC. This purpose information may be signalled as nnpfgc_member_purpose[ i ] and the following applies:

  - If nnpfgc_member_id[ i ] is equal to nnpfc_id of a particular NNPFC, the value of nnpfgc_member_purpose[ i ] shall be equal to the value of nnpfc_purpose of the NNPFC with nnpfc_id is equal to nnpfgc_member_id[ i ].

  - Otherwise (i.e., nnpfgc_member_id[ i ] is equal to nnpfgc_id of a particular NNPFGC), the value of nnpfgc_member_purpose[ i ] shall be the union of the values of nnpfgc_member_purpose[ i ] in the NNPFGC.

# Text implementation

Add the following definition:

Direct member of an NNPFGC: An NNPFC with nnpfc_id equal to nnpfgc_member_id[ i ] for i ranges from 0 to nnpfgc_num_members_minus2 + 1, inclusive.

Indirect member of an NNPFGC: An NNPFC that is a direct or indirect member of another NNPFGC nnpfgcB, in which the value of nnpfgc_id of nnpfgcB is equal to nnpfgc_member_id[ i ] for i ranges from 0 to nnpfgc_num_members_minus2 + 1, inclusive.

Modification to the syntax table:

| nn_post_filter_group_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfgc_id | ue(v) |
| nnpfgc_grouping_type | ue(v) |
| if( nnpfgc_grouping_type = = 0 ¦ ¦ nnpfgc_grouping_type = = 2 ) |  |
| nnpfgc_purpose | u(16) |
| nnpfgc_num_members_minus2 | ue(v) |
| for( i = 0; i <= nnpfgc_num_members_minus2 + 1; i++ ) { |  |
| nnpfgc_member_id[ i ] | ue(v) |
| if( nnpfgc_grouping_type = = 1 ¦ ¦ nnpfgc_grouping_type = = 3 ) |  |
| nnpfgc_member_purpose[ i ] | u(16) |
| } |  |
| … |  |
| } |  |

Modification to the semantics:

…

nnpfgc_purpose has the semantics of nnpfc_purpose but with the exception that the semantics are specified for the NNPF group defined by this SEI message rather than the NNPF defined by an NNPFC SEI message. When present, the value of nnpfgc_purpose shall be the union of the values of nnpfc_purpose of direct and indirect member of the NNPFGC.

…

nnpfgc_member_purpose[ i ] has the semantics of nnpfc_purpose but with the exception that the semantics are specified for the NNPFC or NNPFGC associated with nnpfgc_member[ i ]. When present, the value of nnpfgc_member_purpose[ i ] shall be as follows:

- If nnpfgc_member_id[ i ] is equal to the value of nnpfc_id of an NNPFC, the value of nnpfgc_member_purpose[ i ] shall be equal to the value of nnpfc_purpose of the NNPFC.

- Else (i.e., nnpfgc_member_id[ i ] is equal to the value of nnpfgc_id of an NNPFGC nnpfgcB), the value of nnpfgc_member_purpose[ i ] shall be equal to nnpfgc_purpose of nnpfgcB.

…

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).