| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0092-v1 |
| --- | --- |

| Title: | [AHG9] On the output pictures from NNPFGC with parallel grouping type |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Junghak Nam Seung hwan Kim Jaehyun Lim | Email: | dr.hendry@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

It is asserted that the current text for neural-network post-filter group characteristic (NNPFGC) SEI message when the grouping type is parallel grouping is not clear on the following two cases:

- When two or more NNPFs in a parallel group produce interpolated pictures in between two same consecutive input pictures. Without clear direction what to do, different decoder may order these interpolated pictures in different ways in between those two input pictures.

- When two or more NNPFs in a parallel group produce output pictures associated with the same input picture. Without clear direction what to do, different decoder may override the output of other parallel filter(s).

To address the asserted problems above, the following modifications are proposed:

- When two interpolated pictures, intPicA and intPicB, are outputted by members of an NNPFGC such that intPicA is an interpolated picture outputted by nnpfgc_member_id[ m ] and intPicB is an interpolated picture outputted by nnpfgc_member_id[ n ] and all the following applies:

  - Both intPicA and intPicB are in between the same two consecutive input pictures

  - The value of nnpfgc_grouping_type is equal to 3

  - m is less than n

Then the order of intPicA and intPicB shall be such that intPicA precedes intPicB in output order.

- When the value of nnpfgc_grouping_type of an NNPFGC is equal to 3, it is constrained that there shall be no two or more filters have output pictures that are associated with the same input picture(s).

# Problem Statement

In the 31st JVET meeting, the first design of neural-network post-filter group characteristic (NNPFGC) and neural-network post-filter group activation (NNPFGA) SEI messages were agreed to be included in the Technologies under consideration for future extensions of VSEI (JVET-AE2032).

In the current design, when an NNPFGC SEI message has nnpfgc_grouping_type equal to 3 (i.e., parallel grouping), it is not clear how to handle the output the invocation of NNPFs. We asserted that there are two cases in which the current design does not provide clear direction / description of how to handle the output pictures from this filter.

The first case is when two or more NNPFs in a parallel group produce interpolated pictures in between two same consecutive input pictures. Without clear direction what to do, different decoder may order these interpolated pictures in different ways in between the two input pictures.

The second case is when two or more NNPFs in a parallel group produce output pictures associated with the same input picture. Without clear direction what to do, different decoder may override the output of other parallel filter(s).

# Proposal

We propose the following modification to address the asserted problematic cases above:

- When two interpolated pictures, intPicA and intPicB, are outputted by members of an NNPFGC such that intPicA is an interpolated picture outputted by nnpfgc_member_id[ m ] and intPicB is an interpolated picture outputted by nnpfgc_member_id[ n ] and all the following applies:

  - Both intPicA and intPicB are in between the same two consecutive input pictures

  - The value of nnpfgc_grouping_type is equal to 3

  - m is less than n

Then the order of intPicA and intPicB shall be such that intPicA precedes intPicB in output order.

- When the value of nnpfgc_grouping_type of an NNPFGC is equal to 3, it is constrained that there shall be no two or more filters have output pictures that are associated with the same input picture(s).

# Text implementation

…

nnpfgc_member_id[ i ] indicates the i-th member in the NNPF group defined by this SEI message as follows:

–	If there is an NNPF with nnpfc_id equal to nnpfgc_member_id[ i ] defined in the CLVS, the i-th member in the NNPF group defined by this SEI message is an NNPF that has nnpfc_id equal to nnpfgc_member_id[ i ].

–	Otherwise (there is no NNPF with nnpfc_id equal to nnpfgc_member_id[ i ] defined in the CLVS), the i-th member in the NNPF group defined by this SEI message is an NNPF group with nnpfgc_id equal to nnpfgc_member_id[ i ].

When an nnpfgc_member_id[ i ] value references an nnpfgc_id value of an NNPFGC SEI message nnpfgcSei, it is a requirement of bitstream conformance that the NNPFGC SEI message nnpfgcSei shall have nnpfgc_grouping_type equal to 0. When nnpfgc_grouping_type is equal to 0 or 2, it is a requirement of bitstream conformance that there is an NNPF with nnpfc_id value equal to nnpfgc_member_id[ i ] defined in the CLVS. When nnpfgc_grouping_type is equal to 1, 3, or 4, it is a requirement of bitstream conformance that there is an NNPF with nnpfc_id value equal to nnpfgc_member_id[ i ] or an NNPF group with nnpfgc_id value equal to nnpfgc_member_id[ i ] defined in the CLVS.

When nnpfgc_grouping_type is equal to 0, the NNPFs with nnpfc_id equal to nnpfgc_member_id[ i ] are performed in cascade in increasing order of i, as activated by an NNPFGA SEI message with nnpfga_target_id equal to nnpfgc_id.

When two interpolated pictures, intPicA and intPicB, are outputted by members of an NNPFGC such that intPicA is an interpolated picture outputted by nnpfgc_member_id[ m ] and intPicB is an interpolated picture outputted by nnpfgc_member_id[ n ] and all the following applies:

- Both intPicA and intPicB are in between the same two consecutive input pictures

- The value of nnpfgc_grouping_type is equal to 3

- m is less than n

Then the order of intPicA and intPicB shall be such that intPicA precedes intPicB in output order.

When nnpfgc_grouping_type is equal to 3, there shall be no two pictures, outPicA which is an output picture from nnpfgc_member_id[ m ] and outPicB which is an output picture from nnpfgc_member_id[ n ] where n is not equal to m, are associated with the same picture that is an input picture for both nnpfgc_member_id[ m ] and nnpfgc_member_id[ n ].

…

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).