| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0095-v1 |
| --- | --- |

| Title: | [AHG9] On order of output pictures when skipped candidate input pictures are present in NNPFGA |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Junghak Nam Seung hwan Kim Jaehyun Lim | Email: | dr.hendry@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

It is asserted that the current text for neural-network post-filter group activation (NNPFGA) SEI message has problem when skipped input pictures are used and one or more filters in the target NNPFGC are filters that produce interpolated pictures. When one or more pictures are skipped from being used as input pictures, the skipped pictures may cause problem for decoder to determine where to put the interpolated output pictures from the filter that are supposed to be in between the two input pictures.

This contribution proposes the following modification to overcome the asserted problem. The proposed modifications are as follows:

- For NNPFGA design, signal a flag for the i-th NNPF to specify whether or not the NNPF has interpolated picture(s) output. This flag is constrained to be equal to 1 when the NNPF includes picture rate upsampling purpose and be equal to 0 when the NNPF does not include picture rate upsampling purpose.

- When the value of nnpfga_input_pic_skip_count[ i ][ j ] is greater than 0, signal a new syntax element called nnpfga_num_interpolated_pic[ k ][ j ][ k ] for k ranges from 0 to nnpfga_input_pic_skip_count[ i ][ j ], inclusive, to specify the number of interpolated pictures generated by the i-th NNPF in the target NNPFG between the k-th and the ( k + 1 )-th pictures that are skipped in the list of candidate input pictures candInputPicList[ i ] after the j-th input picture of the NNPF

- It is constrained that when nnpfga_num_interpolated_pic[ k ][ j ][ k ] is present, the sum of nnpfga_num_interpolated_pic[ i ][ j ][ k ] for k ranges from 0 to nnpfga_input_pic_skip_count[ i ][ j ], inclusive, shall be equal to the value of nnpfc_interpolated_pics[ j ] of the NNPF.

# Problem Statement

In the 31st JVET meeting, the first design of neural-network post-filter group characteristic (NNPFGC) and neural-network post-filter group activation (NNPFGA) SEI messages were agreed to be included in the Technologies under consideration for future extensions of VSEI (JVET-AE2032).

In NNPFC design, it has syntax elements that specify how many input pictures the filter needs, which one of these input pictures will have corresponding output pictures, and if interpolated pictures are produced, there is a clear mechanism / description regarding the position of these interpolated pictures (i.e., in between which input pictures). On the other hand, in the current design of NNPFGA SEI message, in the input and output signalling in NNPFGA, there is a feature such that it is possible to skip some pictures to be used as input pictures. When one or more pictures are skipped from being used as input pictures, it is asserted that in between two input pictures, the skipped pictures may cause problem for decoder to determine where to put the interpolated output pictures from the filter that are supposed to be in between the two input pictures as currently there is no clear specification about it.

Figure below illustrate example of this issue. Supposed that there is an NNPFGA that activates an NNPFGC with cascade grouping type containing NNPF A and NNPF B. NNPF A takes 2 input pictures and output 3 pictures (associated with input pictures and 1 interpolated picture). NNPF B takes 2 input pictures but it skip one picture in between these two input pictures (i.e., the interpolated picture outputted by the previous filter) and produces two new interpolated pictures in between the two input pictures. In such situation, it is not clear how to order the interpolated picture from NNFP A and the two interpolated pictures from NNPF B.

Figure 1 – Example of unclear order of output pictures in between two input pictures.

# Proposal

We propose the following modification to overcome the problem described above:

- For NNPFGA design, signal a flag for the i-th NNPF to specify whether or not the NNPF has interpolated picture(s) output. This flag is constrained to be equal to 1 when the NNPF includes picture rate upsampling purpose and be equal to 0 when the NNPF does not include picture rate upsampling purpose.

- When the value of nnpfga_input_pic_skip_count[ i ][ j ] is greater than 0, signal a new syntax element called nnpfga_num_interpolated_pic[ k ][ j ][ k ] for k ranges from 0 to nnpfga_input_pic_skip_count[ i ][ j ], inclusive, to specify the number of interpolated pictures generated by the i-th NNPF in the target NNPFG between the k-th and the ( k + 1 )-th pictures that are skipped in the list of candidate input pictures candInputPicList[ i ] after the j-th input picture of the NNPF

- It is constrained that when nnpfga_num_interpolated_pic[ k ][ j ][ k ] is present, the sum of nnpfga_num_interpolated_pic[ i ][ j ][ k ] for k ranges from 0 to nnpfga_input_pic_skip_count[ i ][ j ], inclusive, shall be equal to the value of nnpfc_interpolated_pics[ j ] of the NNPF.

# Text implementation

…

| nn_post_filter_group_activation( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfga_target_id | ue(v) |
| nnpfga_cancel_flag | u(1) |
| if( !nnpfga_cancel_flag ) { |  |
| nnpfga_persistence_flag | u(1) |
| nnpfga_num_filters_minus2 | ue(v) |
| for( i = 0; i <= nnpfga_num_filters_minus2 + 1; i++ ) { |  |
| nnpfga_target_base_flag[ i ] | u(1) |
| nnpfga_input_all_pics_flag[ i ] | u(1) |
| if( !nnpfga_input_all_pics_flag[ i ] ) { |  |
| nnpfga_num_input_pics_minus1[ i ] | ue(v) |
| nnpfga_interpolated_pic_present_flag[ i ] | u(1) |
| for( j = 0; j <= nnpfga_num_input_pics_minus1[ i ]; j++ ) { |  |
| nnpfga_input_pic_skip_count[ i ][ j ] | ue(v) |
| if( nnpfga_input_pic_skip_count[ i ][ j ] && nnpfga_interpolated_pic_present_flag[ i ] ) |  |
| for( k = 0; k <= nnpfga_input_pic_skip_count[ i ][ j ]; k++ ) |  |
| nnpfga_num_interpolated_pic[ i ][ j ][ k ] | ue(v) |
| } |  |
| } |  |
| nnpfga_num_output_entries[ i ] | ue(v) |
| for( j = 0; j < nnpfga_num_output_entries[ i ]; j++ ) |  |
| nnpfga_output_flag[ i ][ j ] | u(1) |
| } |  |
| } |  |
| } |  |

…

nnpfga_num_input_pics_minus1[ i ] specifies the number of input pictures for the i-th NNPF in the target NNPFG. When present, nnpfga_num_input_pics_minus1[ i ] shall be equal to nnpfc_num_input_pics_minus1 for an NNPF with nnpfc_id equal to nnpfgc_member_id[ i ] of an NNPFGC SEI message with nnpfgc_id equal to nnpfga_target_id. When not present, nnpfga_num_input_pics_minus1[ i ] is inferred to be equal to nnpfc_num_input_pics_minus1 for an NNPF with nnpfc_id equal to nnpfgc_member_id[ i ] in an NNPFGC SEI message with nnpfgc_id equal to nnpfga_target_id.

nnpfga_interpolated_pic_present_flag equal 1 specifies that the i-th NNPF in the target NNPFG outputs one or more interpolated picture(s). nnpfga_interpolated_pic_present_flag equal 0 specifies that the i-th NNPF in the target NNPFG does not output interpolated picture. The value of nnpfga_interpolated_pic_present_flag shall be equal to the value of PictureRateUpsamplingFlag of the i-th NNPF in the target NNPFG.

nnpfga_input_pic_skip_count[ i ][ j ] specifies a j-th picture count that is skipped in the list of candidate input pictures candInputPicList[ i ] when selecting input pictures for the NNPF activated by the i-th loop entry. When nnpfga_input_pic_skip_count[ i ][ j ] is not present, it is inferred to be equal to 0 for all values of j in the range of 0 to nnpfga_num_input_pics_minus1[ i ], inclusive. The variable numCandInputPics, which indicates the number of candidate input pictures to the NNPFG, is derived as follows:

numCandInputPics = 0
for( j = 0; j <= nnpfga_num_input_pics_minus1[ 0 ]; j++ )
	numCandInputPics += 1 + nnpfga_input_pic_skip_count[ 0 ][ j ]	(xx)

Let candInputPicList[ m ] for m in the range of 1 to nnpfga_num_filters_minus2 + 1, inclusive, be a list of pictures in inverse output order that is initially empty and formed in decreasing order of n in the range of 0 to m − 1, inclusive, by including each picture that is output by the NNPF process of the n-th loop entry that has no corresponding picture already present in candInputPicList[ m ], and lastly including each picture present in candInputPicList[ 0 ] that has no corresponding picture already present in candInputPicList[ m ].

When a candidate input picture candInputPicList[ m ][ idx ] for any value of m in the range of 1 to nnpfga_num_filters_minus2 + 1, inclusive, is an NNPF output picture of the n-th NNPF process with the value of n being less than the value of m, the width and height of the candidate input picture are respectively equal to nnpfcOutputPicWidth and nnpfcOutputPicHeight of the NNPF output picture.

The list of input pictures inputPicList[ m ] to the NNPF of the m-th loop entry is derived as follows:

for( k = 0, candIdx = 0; k <= nnpfga_num_input_pics_minus1[ m ]; k++, candIdx++ ) {
	candIdx += nnpfga_input_pic_skip_count[ m ][ k ]
	inputPicList[ m ][ k ] = candInputPicList[ m ][ candIdx ]	(xx)
}

It is a requirement of bitstream conformance that candIdx shall not exceed the number of pictures in candInputPicList[ m ].

It is a requirement of bitstream conformance that the pictures present in inputPicList[ m ], for any value of m in the range of 1 to nnpfga_num_filters_minus2 + 1, inclusive, shall have the same width, height, bit depth, and chroma format.

For purposes of interpretation of the NNPFC SEI message with nnpfc_id equal to nnpfgc_member_id[ i ] in an NNPFGC SEI message with nnpfgc_id equal to nnpfga_target_id, the following variables are specified for the i-th loop entry:

–	The variables BitDepthY, BitDepthC, and ChromaFormatIdc are used as provided for the interpretation of this SEI message.

–	CroppedWidth and CroppedHeight are set equal to the width and height of the pictures in inputPicList[ i ], respectively, in units of luma samples.

–	For each input picture k in the range of 0 to nnpfga_num_input_pics_minus1[ i ], inclusive, the following applies:

–		CroppedYPic[ k ], CroppedCbPic[ k ], and CroppedCrPic[ k ], when present, are set equal to respective sample array of inputPicList[ i ][ k ]

–	When nnpfc_auxiliary_inp_idc is equal to 1 for the NNPF with nnpfc_id equal to nnpfgc_member_id[ i ] in an NNPFGC SEI message with nnpfgc_id equal to nnpfga_target_id, the following applies:

–	It is a requirement of bitstream conformance that inputPicList[ i ][ k ] is the same as candInputPicList[ 0 ][ idx ] for any value of idx in the range of 0 to numCandInputPics − 1, inclusive.

–	StrengthControlVal[ k ] is set equal to InitStrengthControlVal[ idx ].

nnpfga_num_interpolated_pic[ i ][ j ][ k ] specifies the number of interpolated pictures generated by the i-th NNPF in the target NNPFG between the k-th and the ( k + 1 )-th pictures that are skipped in the list of candidate input pictures candInputPicList[ i ] after the j-th input picture of the NNPF. When present, it is constrained that the sum of nnpfga_num_interpolated_pic[ i ][ j ][ k ] for k ranges from 0 to nnpfga_input_pic_skip_count[ i ][ j ], inclusive, shall be equal to the value of nnpfc_interpolated_pics[ j ] of the NNPF.

…

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).