| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0104-v2 |
| --- | --- |

| Title: | Non-EE2: Direct Decoder-side Intra Mode Derivation for IntraTMP blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junyan Huo, Nengfu Qiu, Yanzhuo Ma, Fuzheng Yang, Ming Li | Email: | jyhuo@mail.xidian.edu.cn nfqiu@stu.xidian.edu.cn yzma@mail.xidian.edu.cn fzhyang@mail.xidian.edu.cn myron.li@oppo.com |
| Source: | Xidian University,OPPO |  |  |

_____________________________

- Abstract

In ECM10.0, DIMD is used to derive an intra prediction mode for Intra TMP blocks. It is conducted to the prediction signal after filtering, sub-pixel interpolation, or fusion. This contribution is proposed that using the initial predictions to derive intraTmpDimdMode. On top of ECM-10.0, simulation results are reported as follows:

AI(Y/U/V): -0.00%/-0.01%/-0.06%, EncT 100.1%, DecT 100.2%

Introduction

The process of InraTMP in ECM10.0 can be described as follows. The final prediction are obtained according to BV directly or refined by filtering, sub-pixel interpolation or fusion. With the final predictions, DIMD is then employed to derive the intraTmpDimdMode for the transform procedure.

Fig.1. IntraTMP in ECM10.0

Proposed Methods

In this contribution, it is proposed to use the initial predictions to derive intraTmpDimdMode. The benefit of the proposed scheme is to derive the final prediction and derive intraTmpDimdMode for transform in parallel.

Fig.2 The proposed scheme

Experimental results

The proposed method is implemented on ECM-10.0, and tests are performed following test conditions specified in JVET-Y2017[2]. Table 1 shows the test results in AI configurations. More detailed results can be found in the accompanying Excel tables.

Table 1. Simulation results of the proposed method

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over anchor |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | 0.01% | 0.06% | -0.11% | 100.1% | 100.5% | 99.9% |
| Class A2 | 0.00% | -0.04% | -0.02% | 99.8% | 100.3% | 100.0% |
| Class B | 0.01% | -0.06% | -0.07% | 99.9% | 99.6% | 100.0% |
| Class C | -0.01% | 0.01% | 0.06% | 100.8% | 100.7% | 100.0% |
| Class E | 0.01% | 0.02% | -0.20% | 99.7% | 99.8% | 100.0% |
| Overall | 0.00% | -0.01% | -0.06% | 100.1% | 100.2% | 100.0% |
| Class D | 0.04% | -0.04% | -0.02% | 98.3% | 100.9% | 100.0% |
| Class F | -0.02% | -0.02% | 0.04% | 98.9% | 99.6% | 100.0% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! |

Conclusions

In this contribution, the derive intraTmpDimdMode input signal in IntraTMP is modified as the initial prediction signal according to BV. The dependency of deriving the final predictions and obtaining intraTmpDimdMode can be eliminated, which can be processed in parallel. Compared to ECM-10.0, the contribution achieves BD-rate saving of {0.00% (Y), -0.01% (U), -0.06% (V)} for the AI configuration. The encoding and decoding time is similar to the anchor scheme. It is suggested to investigate in EE.

- 5 References

- ECM-10.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

- M. Karczewicz, et. al., “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, February 2022.

- 6 Patent rights declaration(s)

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for the implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).