| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0110-v2 |
| --- | --- |

| Title: | EE2-6.3: Combination of EE2-6.1 and EE2-6.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jani Lainema Alireza Aminlou Pekka Astola Döne Bugdayci Sansli Ru-Ling Liao Yan Ye Jie Chen Xinwei Li | Tel: Email: Tel: Email: | - first.lastname@nokia.com - {ruling.lrl, yan.ye}@alibaba-inc.com |
| Source: | Nokia, Alibaba group |  |  |

_____________________________

# Abstract

This contribution reports results of EE2-6.3 which combines tests EE2-6.1 (spatial CABAC tuning) and EE2-6.2 (updating I-slices context model parameters). The combination is reported to be straight-forward and the software implementation provided in EE2 branch includes only enabling the two techniques at the same time. As EE2-6.1 includes two variants, the same a and b variants are tested in this combination test.

For test EE2-6.3a the impact on coding efficiency and runtimes over ECM-10.0 are reportedly {for Y, U, V, EncT, DecT, VmPeak}:

AI {-0.20%, -0.12%, -0.06%, 99.8%, 99.8%, 100.0%}
RA {-0.09%, -0.10%, -0.11%, 100.0%, 99.5%, 100.1%}
LB {-0.13%, -0.02%, -0.28%, 100.1%, 99.1%, 100.0%}

For test EE2-6.3b the impact on coding efficiency and runtimes over ECM-10.0 are reportedly {for Y, U, V, EncT, DecT, VmPeak}:

AI {-0.18%, -0.11%, -0.08%, 100.0%, 100.1%, 100.0%}
RA {-0.08%, -0.07%, -0.06%, 100.0%, 99.6%, 100.3%}
LB {-0.03%, 0.49%, -0.04%, 100.1%, 99.0%, 99.9%}

# Proposed methods

Descriptions of the proposed techniques can be found in the individual test reports for EE2-6.1 (JVET-AF0109) and EE2-6.2 (JVET-AF0133). Similarly, the individual results for those together with additional performance and complexity analysis are reported in those individual contributions. As EE2-6.1 includes two tests (EE2-6.1a and EE2-6.1b), the combination with EE2-6.2 is reported here for both as follows:

- EE2-6.3a: combination of EE2-6.1a and EE2-6.2

- EE2-6.3b: combination of EE2-6.1b and EE2-6.2

# Coding efficiency analysis

Coding efficiency results can be found in the attached Excel sheets.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Alibaba group does not have any current or pending patent rights relating to the technology described in this contribution.