| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0135-v1 |
| --- | --- |

| Title: | AHG12: Additional SbTMVP candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao Jie Chen Yan Ye Xinwei Li | Tel: Email: | {ruling.lrl, jiechen.cj, yan.ye, sid.lxw}@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

# Abstract

The contribution proposes to add additional SbTMVP candidates whose reference picture indices are set equal to 0 to subblock merge list. At most 4 additional SbTMVP candidates are added to the subblock merge list and are sorted together with existing SbTMVP candidates and other affine candidates. It is reported that on top of ECM-10.0, the overall coding performance and runtime for {Y, U, V, EncT, DecT, VmPeak} is {-0.05%, -0.03%, -0.01%, 100.0%, 101.5%, 100.4%} for RA and {-0.09%, -0.26%, -0.08%, 100.5%, 102.7%, 100.2%} for LB.

# Introduction

In the SbTMVP mode, a CU is split into 4x4 subblocks and each subblock obtains its own motion information from the motion field in a collocated picture. Each subblock can select its own reference picture according to scaling factor. Specifically, the selected reference picture is the one whose scaling factor is the closest to 1 among all reference pictures.

In addition, two collocated pictures are utilized to derive the SbTMVP candidates. For each collocated picture, a motion shift candidate list is firstly constructed using motions from spatial adjacent and non-adjacent neighboring blocks and a HMVP list. The candidates in the motion shift candidate list are sorted according to TM cost. Then, the SbTMVP candidates are derived and are added to a subblock merge list as follows:

- the SbTMVP candidate derived from the first candidate in the first motion shift candidate list is placed in the first entry;

- if current picture is a non-low delay picture and the two collocated pictures are different from each other, the SbTMVP candidate derived from the first candidate in the second motion shift candidate list is added;

- the SbTMVP candidates derived from the second and third candidates in the first motion shift candidate list are added.

At most 4 SbTMVP candidates are included in the subblock merge list. Except for the first SbTMVP candidate, the other SbTMVP candidates are sorted with affine candidates using ARMC.

# Proposed methods

In addition to the existing SbTMVP candidates allowing flexible reference pictures for subblocks, it is proposed to add at most 4 additional SbTMVP candidates whose reference picture indices of subblocks are all set equal to 0 to the subblock merge list. The proposed SbTMVP candidates are inserted right after the existing SbTMVP candidates. More specifically, the existing and proposed SbTMVP candidates are added to the subblock merge list as follows:

- the existing and the proposed SbTMVP candidates derived from the first candidate in the first motion shift candidate list are placed in the first and second entries, respectively;

- if current picture is a non-low delay picture and the two collocated pictures are different from each other, the existing and the proposed SbTMVP candidates derived from the first candidate in the second motion shift candidate list are added;

- the existing and the proposed SbTMVP candidates derived from the second candidate in the first motion shift candidate list are added;

- the existing and the proposed SbTMVP candidates derived from the third candidate in the first motion shift candidate list are added.

# Simulation results

The proposed methods are implemented on top of ECM-10.0 and are tested under CTC for enhanced compression tool testing [1] with random access and low delay B configurations. Table 1 shows the simulation results of the proposed method.

Table 1. Simulation results of the proposed method

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.03% | -0.06% | 0.00% | 100.3% | 101.6% | 100.6% |
| Class A2 | -0.03% | 0.04% | -0.05% | 100.3% | 102.3% | 100.2% |
| Class B | -0.04% | 0.00% | -0.02% | 100.0% | 101.5% | 100.4% |
| Class C | -0.08% | -0.12% | 0.05% | 99.7% | 100.7% | 100.3% |
| Class E |  |  |  |  |  |  |
| Overall | -0.05% | -0.03% | -0.01% | 100.0% | 101.5% | 100.4% |
| Class D | -0.06% | 0.03% | -0.05% | 99.3% | 100.3% | 100.4% |
| Class F | -0.08% | -0.11% | -0.09% | 99.6% | 101.0% | 100.1% |

|  | Low delay B Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.01% | -0.18% | -0.08% | 100.4% | 102.9% | 100.0% |
| Class C | -0.07% | -0.42% | -0.05% | 100.6% | 102.0% | 100.4% |
| Class E | -0.26% | -0.15% | -0.11% | 100.4% | 103.6% | 100.3% |
| Overall | -0.09% | -0.26% | -0.08% | 100.5% | 102.7% | 100.2% |
| Class D | -0.16% | -0.39% | -0.10% | 99.3% | 101.4% | 100.4% |
| Class F | -0.03% | -0.08% | 0.02% | 100.8% | 102.4% | 100.3% |

# Conclusion

This contribution proposes to add at most 4 additional SbTMVP candidates whose reference picture indices are set equal to 0 to subblock merge list. It is reported that 0.05% and 0.09% BD-rate reduction are achieved in RA and LB conditions with minor runtime impact. It is recommended to study this proposal in next round of EE2.

# References

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AE2017, Jul. 2023.

# Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).