| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0169-v1 |
| --- | --- |

| Title: | Non-EE2: Adaptive clipping with signaled lower and upper bounds |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Cui, Zhi Zhang, Han Huang, Vadim Seregin, Marta Karczewicz (Qualcomm) | Tel: Email: | kaicui@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes a method of applying adaptive clipping with signaled lower and upper bounds. For each picture, the minimum and maximum values of the Luma channel, which are derived from the original picture or MCTF pre-filtered picture, are signaled and used to perform clipping at the reconstruction stage. The proposed method reports the following BD-rate performance on top of ECM-10.0:

AI: {-0.06%, 0.06%, -0.02%}, EncT 100.4%, DecT 100.3%, VmPeak 100.0%.

RA: {-0.xx%, -0.xx%, -0.xx%}, EncT xx.x%, DecT xx.x%, VmPeak 100.0%.

# Introduction

In ECM-10.0, a bit-depth based clipping is applied to multiple places to avoid data overflow. The range of the bit-depth based clipping is defined as [0, 2^bitdepth-1]. The clipping operation is applied in filtering, sampling, interpolation, weighted prediction, weighted combination, reconstruction, and other stages, to ensure that the generated prediction and reconstruction samples remain in the defined dynamic range.

# Proposed Method

This contribution proposes to use the signaled minimum/maximum values of the luma channel as the clipping bounds. The min/max values are obtained by scanning the sample values in the original picture. If motion compensated temporal filter (MCTF) is applied to a picture, the min/max values are obtained by scanning the sample values in the MCTF pre-filtered picture. The delta values compared to [64, 940] of min/max values, respectively, are signaled in picture header. The min/max values are used as the lower and upper bounds for clipping at the reconstruction stage (when adding the residual signal to the prediction) and before saving the reconstructed picture into the decoded picture buffer.

When clipping is performed in luma mapping chroma scaling (LMCS) mapped domain, the min/max values are derived by applying forward LMCS look-up table to the signalled min/max values.

# Simulation Results

The proposed method was implemented on top of ECM-10.0. The performance was evaluated following the common test condition defined in [2]. The test results are reported in the following tables using ECM-10.0 as the anchor.

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.08% | -0.01% | -0.11% | 100.2% | 99.6% | 100.0% |
| Class A2 | -0.01% | 0.02% | -0.03% | 100.2% | 100.9% | 100.0% |
| Class B | -0.09% | 0.03% | -0.09% | 100.4% | 100.2% | 100.0% |
| Class C | -0.08% | 0.06% | 0.10% | 100.9% | 100.7% | 100.0% |
| Class E | -0.03% | 0.20% | 0.01% | 100.3% | 100.1% | 100.0% |
| Overall | -0.06% | 0.06% | -0.02% | 100.4% | 100.3% | 100.0% |
| Class D | -0.20% | -0.06% | -0.12% | 100.1% | 100.1% | 100.0% |
| Class F | -0.29% | -0.26% | 0.14% | 100.0% | 100.4% | 100.0% |
| Class TGM | -0.36% | -0.18% | -0.18% | 100.3% | 100.5% | 100.0% |

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.07% | 0.14% | -0.20% | 100.2% | 101.2% | 100.1% |
| Class C | -0.13% | -0.04% | -0.06% | 100.3% | 100.9% | 100.1% |
| Class E |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |
| Class D | -0.38% | 0.11% | -0.16% | 100.0% | 101.3% | 100.0% |
| Class F | -0.10% | 0.09% | 0.18% | 102.3% | 104.0% | 100.0% |
| Class TGM | -0.46% | -0.35% | -0.42% | 100.7% | 102.7% | 100.0% |

# Reference

[1] ECM-10.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

[2] M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing”, JVET-AE2017, July 2023.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).