| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0176-v1 |
| --- | --- |

| Title: | Non-EE2: Decoder Derived Cross-Component Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang, Po-Han Lin, Vadim Seregin, Jian-Liang Lin, Marta Karczewicz | Tel: Email: | yjchang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes a decoder derived cross-component prediction (DDCCP) to derive the best CCP mode by comparing the template costs between the evaluated CCPs. A decoder-derived CCP fusion mode is also introduced to derive CCP modes for fusion selected based on the template costs. The simulation results for 3 different test setting on top of ECM-10.0 are summarized as follows:

Test 1: CCP list constructed by {CCLM, CCCM, CCCMwMDF, GLCCCM, LBCCP} with the constraint of (CU_Width * CU_Height > 64)

AI: -0.08% (Y), -0.34% (Cb), -0.42% (Cr), 101.6% (EncT), 103.4% (DecT)

Test 2: CCP list constructed by {CCCM, CCCMwMDF, GLCCCM} with the constraint of (CU_Width * CU_Height > 8)

AI: -0.10% (Y), -0.34% (Cb), -0.34% (Cr), 102.3% (EncT), 105.5% (DecT)

Test 3: CCP list constructed by {CCLM, CCCM, CCCMwMDF, GLCCCM, LBCCP} with the constraint of (CU_Width * CU_Height > 8)

AI: -0.14% (Y), -0.16% (Cb), -0.16% (Cr)

- Introduction

Cross-component linear model (CCLM) prediction mode is used in VVC, for which the chroma samples are predicted based on the reconstructed luma samples around the collocated positions. ECM [1] further adopts various types of cross-component prediction (CCP) modes, which include slope adjustment of CCLM, gradient LM (GLM), convolutional cross-component intra prediction mode (CCCM), no-subsampled CCCM, CCCM with multiple downsampling filters (CCCMwMDF), gradient and location based CCCM (GLCCCM), block-vector guided CCCM (BVG-CCCM), local-boosting CCP (LB-CCP), and CCP merge (CCPmerge, also called non-local CCP). The newly introduced CCP modes improve the overall performance, especially in chroma components. However, the current CCP design causes an inefficient signaling while the best CCP mode for a chroma CU in a specific case is the one using longest bits. Therefore, this contribution provides a way to resolve the issue by signaling a decoder derived flag to derive a best CCP mode using a template.

- Proposal

A decoder derived cross-component prediction (DDCCP) is proposed with a DDCCP flag signalled. When this DDCCP flag is true, it requires to construct a CCP mode list, and derive the best CCP mode by using a template (as shown in Figure 1) to calculate the template costs to compare the reconstructed samples and the prediction values generated by the evaluated CCP mode. It is further proposed to introduce a decoder-derived CCP fusion mode. A DDCCP fusion may be signaled. If this flag is true, it derives CCP modes for fusion based on the template costs.

Figure 1. Example of template adjacent to the current chroma CU

- Simulation results

Three tests are performed for the proposed methods on top of reference software ECM-10.0. The differences between the two tests are on the CCP list construction and CU size constraint as follows:

Test 1: CCP list constructed by {CCCM, CCCMwMDF, GLCCCM} with the constraint of (CU_Width * CU_Height > 64).

Test 2: CCP list constructed by {CCCM, CCCMwMDF, GLCCCM} with the constraint of (CU_Width * CU_Height > 8).

Test 3: CCP list constructed by {CCLM, CCCM, CCCMwMDF, GLCCCM, LBCCP} with the constraint of (CU_Width * CU_Height > 8).

The simulations are run under Common Test Conditions for ECM [2]. The results are shown in the following tables.

Table 1. Simulation results of Test 1.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.74% | -0.84% | 101.0% | 105.9% |
| Class A2 | -0.14% | -0.64% | -0.78% | 102.0% | 103.9% |
| Class B | -0.08% | -0.18% | -0.30% | 101.3% | 104.4% |
| Class C | -0.05% | -0.06% | -0.05% | 102.2% | 98.4% |
| Class E | -0.10% | -0.32% | -0.35% | 102.0% | 105.7% |
| Overall | -0.08% | -0.34% | -0.42% | 101.6% | 103.4% |
| Class D | -0.01% | 0.01% | 0.00% | 101.8% | 102.5% |
| Class F | -0.01% | -0.01% | 0.03% | 103.5% | 105.3% |

Table 2. Simulation results of Test 2.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.75% | -0.79% | 101.3% | 107.1% |
| Class A2 | -0.15% | -0.64% | -0.74% | 103.2% | 106.0% |
| Class B | -0.09% | -0.24% | -0.34% | 101.7% | 106.0% |
| Class C | -0.10% | -0.01% | 0.11% | 104.0% | 103.5% |
| Class E | -0.10% | -0.25% | -0.08% | 101.2% | 105.2% |
| Overall | -0.10% | -0.34% | -0.34% | 102.3% | 105.5% |
| Class D | -0.06% | -0.04% | 0.12% | 100.0% | 105.6% |
| Class F | -0.09% | 0.00% | 0.02% |  |  |

Table 3. Simulation results of Test 3.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.64% | -0.64% |  |  |
| Class A2 | -0.19% | -0.53% | -0.59% |  |  |
| Class B | -0.11% | -0.03% | -0.12% |  |  |
| Class C | -0.14% | 0.28% | 0.36% |  |  |
| Class E | -0.18% | -0.14% | -0.03% |  |  |
| Overall | -0.14% | -0.16% | -0.16% |  |  |
| Class D | -0.10% | 0.42% | 0.22% |  |  |
| Class F | -0.11% | 0.13% | 0.36% |  |  |

- References

[1] M. Coban, R.-L. Liao, K. Naser, J. Ström, and L. Zhang, “Algorithm description of Enhanced Compression Model 10 (ECM 10),” JVET-AE2025, Oct. 2023.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AE2017, April 2023.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).