| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0178-v2 |
| --- | --- |

| Title: | Non-EE2: Applying fixed filters to output of the Gaussian filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Marta Karczewicz, Vadim Seregin 5775 Morehouse Drive, San Diego, CA 92121, USA | Tel: Email: | nanh@qti.qualcomm.com martak@qti.qualcomm.com vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In ECM-10.0, two Laplacian classifier-based fixed filters in adaptive loop filter (ALF) are applied to input to deblocking filters (DBF) and input to ALF. A third fixed filter, which is known as a Gaussian filter, is applied to input to DBF. The outputs of the three fixed filters are then used to derive ALF output. In this contribution, the first two fixed filters are proposed to apply to output of the Gaussian filter. On top of ECM-10.0 with common test condition, simulation results of the proposed method are reported as follows.

|  | BD-rate Y | BD-rate U | BD-rate V | EncT | DecT | VmPeak |
| --- | --- | --- | --- | --- | --- | --- |
| All intra | -0.07% | 0.00% | 0.00% | 100.7% | 104.0% | 100.1% |
| Random access | -0.11% | 0.02% | -0.04% | 100.2% | 102.8% | 100.1% |
| Low delay B | -0.12% | 0.13% | -0.26% | 100.2% | 103.0% | 100.1% |

# Introduction

In ECM-10.0, two Laplacian-based classifiers are applied to a 2x2 block. The derived class indices are used to select two fixed filters from two filter sets. Then both fixed filters are applied to deblocking filter (DBF) input samples and adaptive loop filter (ALF) input samples. A third fixed filter, which is also known as a Gaussian filter, is applied to DBF input samples. Finally, a signalled filter is applied to the ALF input samples, DBF input samples, outputs of the two Laplacian-based fixed filters, output of the Gaussian filter and the residual data.

# Proposed method

In this proposal, the Gaussian filter output is added as an additional input to both Laplacian-based fixed filters and diamond 9x9 filter is used for that new input.

# Simulation results

The proposed method was implemented on top of ECM-10.0 and tested following common test conditions described in JVET-AE2017.

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.03% | 0.00% | 0.00% | 100.9% | 102.9% | 100.2% |
| Class A2 | -0.06% | 0.00% | 0.00% | 100.2% | 101.3% | 100.0% |
| Class B | -0.05% | 0.00% | 0.00% | 102.2% | 107.0% | 100.0% |
| Class C | -0.12% | -0.01% | -0.01% | 99.6% | 102.7% | 100.1% |
| Class E | -0.07% | 0.00% | 0.00% | 100.0% | 105.0% | 100.1% |
| Overall | -0.07% | 0.00% | 0.00% | 100.7% | 104.0% | 100.1% |
| Class D | -0.06% | 0.00% | 0.00% | 100.0% | 102.4% | 100.1% |
| Class F | -0.08% | -0.01% | -0.01% | 100.3% | 104.0% | 100.1% |
|  | Random Access Main 10 |  |  |  |  |  |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.03% | -0.03% | -0.02% | 101.6% | 104.8% | 100.2% |
| Class A2 | -0.08% | 0.08% | -0.06% | 101.3% | 104.2% | 100.4% |
| Class B | -0.11% | 0.06% | -0.13% | 99.6% | 102.2% | 100.0% |
| Class C | -0.19% | -0.03% | 0.06% | 99.1% | 101.1% | 100.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.11% | 0.02% | -0.04% | 100.2% | 102.8% | 100.1% |
| Class D | -0.25% | 0.22% | 0.01% | 100.8% | 102.5% | 100.1% |
| Class F | -0.05% | -0.17% | -0.03% | 99.9% | 101.9% | 100.1% |
|  | Low delay B Main 10 |  |  |  |  |  |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.07% | 0.17% | -0.27% | 100.1% | 102.7% | 100.2% |
| Class C | -0.23% | -0.26% | -0.38% | 99.7% | 103.0% | 100.1% |
| Class E | -0.06% | 0.57% | -0.08% | 100.9% | 103.6% | 100.1% |
| Overall | -0.12% | 0.13% | -0.26% | 100.2% | 103.0% | 100.1% |
| Class D | -0.10% | -0.46% | -0.28% | 99.3% | 102.1% | 100.1% |
| Class F | 0.01% | -0.17% | 0.49% | 100.8% | 102.7% | 100.1% |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).