| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0179-v1 |
| --- | --- |

| Title: | Non-EE2: Fixed filter for Chroma ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Marta Karczewicz, Hongtao Wang, Vadim Seregin 5775 Morehouse Drive, San Diego, CA 92121, USA | Tel: Email: | nanh@qti.qualcomm.com martak@qti.qualcomm.com hongtaow@qti.qualcomm.com vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In ECM-10.0, for luma component, two Laplacian and variance classifier-based fixed filters in adaptive loop filter (ALF) are applied to input to deblocking filters (DBF) and input to ALF. The outputs of the two fixed filters are then used to derive ALF output. In this contribution, a Laplacian and variance classifier-based fixed filter is applied to chroma components. On top of ECM-10.0 with common test condition, simulation results of the proposed method are reported as follows.

|  | BD-rate Y | BD-rate U | BD-rate V | EncT | DecT | VmPeak |
| --- | --- | --- | --- | --- | --- | --- |
| All intra | % | % | % | % | % | % |
| Random access | % | % | % | % | % | % |
| Low delay B | % | % | % | % | % | % |

# Introduction

In ECM-10.0, based on Laplacian values and variance, two classifiers are applied to a 2x2 luma block. For each classifier, a class index is determined based on the values of activity, directionality and scaled variance, where the activity value is a scaled value of sum of horizontal and vertical Laplacian values. The derived class indices are used to select two fixed filters from two filter sets. Then both fixed filters are applied to deblocking filter (DBF) input samples and adaptive loop filter (ALF) input samples. Finally, a signalled filter is applied to the ALF input samples, DBF input samples, outputs of the two fixed filters, output of the Gaussian filter and the residual data. When ALF is applied to a chroma sample, a 9x9 diamond shape signalled filter is applied to the ALF input samples.

# Proposed method

In this contribution, fixed filter is extended to chroma components. Like the classifiers for luma component, a classifier based on Laplacian values and variance is applied to chroma components. Compared to the luma classifier of a fixed filter in ECM-10.0, the proposed chroma classifier is applied at sample level. In addition, when calculating the activity value, the sum of the chroma vertical and horizonal Laplacian values is multiplied by 2 before scaling. Similarly, the chroma variance is multiplied by 2 before scaling. The derived class index is then used to select a fixed filter from a chroma filter set. In the proposed method, a chroma fixed filter is applied to ALF input samples in a 13x13 diamond shape and DBF input samples in a 7x7 diamond shape. Finally, in a signalled chroma filter, one extra tap is introduced, which is applied to the fixed filter output of current sample.

# Simulation results

The proposed method was implemented on top of ECM-10.0 and tested following common test conditions described in JVET-AE2017.

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |
| Class C | -0.06% | -2.33% | -3.24% |  |  | 100.1% |
| Class E | -0.02% | -1.92% | -1.34% |  |  | 100.2% |
| Overall |  |  |  |  |  |  |
| Class D | 0.00% | -2.03% | -3.24% |  |  | 100.1% |
| Class F |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |
| Class C | -0.04% | -2.40% | -2.81% |  |  | 100.1% |
| Class E |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |
| Class D | -0.02% | -1.82% | -3.01% |  |  | 100.1% |
| Class F | -0.02% | -1.82% | -1.99% |  |  | 100.2% |
|  | Low delay B Main 10 |  |  |  |  |  |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |
| Class D | -0.02% | -1.70% | -2.03% |  |  | 100.0% |
| Class F |  |  |  |  |  |  |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).