| 2Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0311-v1 |
| --- | --- |

| Title: | Results of expert viewing for the spatial scalability category of the VVC multilayer VT |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | M. Wien | Email: | wien@lfb.rwth-aachen.de |
| Source: | BoG |  |  |

_____________________________

# Abstract

This document reports the results of an on-site expert viewing performed at the 32nd JVET meeting in Hannover, DE.

The evaluation targets at the spatial scalability category of the test plan document JVET-AE2021, comparing the compression performance of dual layer (DL) coding with a downscaled base layer at scaling ratio 2 to single layer coding at the low resolution with upscaling to the target resolution using the VVC interpolation filters (UP) as well as single layer coding at the target resolution (SL).

A number of 19 experts participated in the expert viewing. With a comparably low number of valid scores, the results should be considered as an indication of tendencies. They are not regarded as a necessarily conclusive result.

With respect to the goals of this experiment, the following observations are made:

- Since the UP sequences represent the extracted base layer of the DL streams, the impact of the enhancement layer can be studied. It can be seen that in most cases, the MOS only slightly increases with the inclusion of the enhancement layer. This might be attributed to the fact that other effects overlay the benefit of enhanced detail provided with the DL stream.

- The result show a similar relation of the SL, DL, and UP results compared to JVET-AE0288 for the sequences CorsaireJoy. For TallBuildings2, the results seem to indicate a visual improvement of the DL configuration compared to the UP configuration. This might be attributed to the encoder modifications reported in JVET-AF0186.

- The MOS results for the DrivingPOVLogo sequence seem to indicate that there might be a range of benefit for the DL scheme at rates at or below 1Mbit/s.

- The MOS results for the NxBoxeLogo sequence seem to indicate bitrate ranges where  UP, DL, and SL have benefits, similar to what might be chosen in a bitrate ladder setting.

It is recommended to take these results into account by reconsiderering the test setup with respect to bitrate ranges to be tested for certain resolutions and encoder configurations. The test sequence CorsaireJoy might not include sufficient detail to provide a suitable demonstration case for the targeted application.

# Introduction

This document reports the results of an on-site expert viewing performed at the 32nd JVET meeting in Hannover, DE. The evaluation targets at the spatial scalability category of the test plan document JVET-AE2021 [1], comparing the compression performance of dual layer (DL) coding with a downscaled base layer at scaling ratio 2 to single layer coding at the low resolution with upscaling to the target resolution using the VVC interpolation filters (UP) as well as single layer coding at the target resolution (SL).

# Test setup

## Logistics

The setup at the meeting site included a PC with a Decklink video board for HDMI connection and SSD drives capable of stable playout of the raw YUV data at the required frame rate.

| Test Site | On-site |
| --- | --- |
| Display, size, connection (resolution setting) | LG 65G 65”, HDMI (3840×2160), 10bit input |
| Viewing distance | 3 viewers sitting at 1.5H, 2 views standing at 1.6H |
| Viewing angle | ±75°, 90° (at screen center) |
| Total number of viewers | 19 (2 female, 17 male) |

Participants confirmed visual acuity and normal colour vision.

The setup was chosen with three viewers sitting and two viewers standing right behind them at about the same distance was chosen to maximize the number of collected scores per session. The tests were performed with 3 groups 5 and one group of 4 participants with the seats and the center standing position covered first. The viewers entered their scores into the test database after the viewing session.

## Test sequences and quantizer settings

For evaluation, a subset of test sequences and quantizer settings listed in JVET-AF0186 were tested. Four UHD sequences were tested, including the two test sequences CorsaireJoy and TallBuildings2 studied before and the test sequences DivingPOVLogo and NxBoxeLogo from the LCEVC verification test activity. Three coding configurations were tested: Single layer (SL) at full resolution, dual layer (DL) with an HD base layer (2× spatial scalability), and upscaled base resoltution (UP) where the base layer of the DL streams was upscaled using the VVC upscaling filters. As an additional measurement point, the original test sequences, downscaled using the SHVC downscaling process and re-upscaled using the VVC upscaling filters, were evaluated (origUP). The QP values evaluated in the expert viewing are shown in the table below.

Table 1 – Selected test sequences and QP settings for the viewing.

| Sequence | QP SL | QP UP | QP BL QP EL |
| --- | --- | --- | --- |
| CorsaireJoy | 33 | 27 | 27 34 |
| CorsaireJoy | 40 | 34 | 34 41 |
| TallBuildings2 | 34 | 28 | 28 35 |
| TallBuildings2 | 42 | 36 | 36 43 |
| DrivingPOV3Logo | 32 | 27 | 27 34 |
| DrivingPOV3Logo | 38 | 33 | 33 40 |
| DrivingPOV3Logo | 42 | 37 | 37 44 |
| BoxeLogo | 34 | 30 | 30 37 |
| BoxeLogo | 38 | 32 | 32 39 |
| BoxeLogo | 42 | 37 | 37 44 |

### Rate distortion plots for the selected test points

## Test method and test design

The assessment was performed with the reconstructed UHD output. A single DCR test session with 40 BTCs was presented to the viewers. The duration of the test session was about 21 min.

### Degradation category rating

#### Session setup

The test sequences were evaluated using an 11-grade scale shown in Figure 2 below.

Figure 2: Meaning of the used 11 grade numerical scale

The test session included 40 basic test cells (BTCs) showing the uncompressed video sequence and one processed video sequence. Each BTC was structured as followed (PVS denotes the processed video sequence under evaluation):

“Original” (1sec) – [uncompressed sequence] (10sec) – “A” (1sec) – [PVS] (10sec) – “Vote <N>” (5sec)

The session included a stabilization phase of four BTCs and two trapping BTCs where original full resolution test sequences were to be scored in the PVS.

#### Viewer information and training

The viewers were explained the viewing task, the organization of the session and the meaning of the grading scale. All questions re

For the training, the training method used in JVET-AD0399 and JVET-AE0288 was applied. The method is briefly summarized as follows:

- Explain procedure and scoring scale. 
- Run test session (4 BTCs here, covering examples of the impairment range) 
- Let viewers note their scores. 
- Let them read out their scores. If variation is larger than 3 on the grading scale, re-run BTC 
   - Viewers shall reconsider their scoring for that sequence (not enforced to change their mind)
   - No indications where to look allowed, viewers have to make it up by themselves.
   - Answering of remaining questions, no further communication of scores.

# Subjective results and analysis

## Data processing

As a first step, the complete set of votes experts was screened for the trapping sequences. Two viewers was excluded on this basis. The Pearson correlation coefficient of the viewers scores relative to the MOS was assessed. The scores of three more viewers were removed based on a PCC threshold of 0.75%. As a last step, the z-scores of the viewer scores were compared to a threshold of 3. The z-score is computed as

zij= vij-MOSiσi, 

where vij is the score of viewer j for PVS i, MOSi is the mean opinion score for that PVS for the viewers, and σi is the standard deviation associated with MOSi. No scores were removed from the dataset on this basis.

## Subjective results

The measured MOS values of the reconstructed video on the 11-grade scale are plotted over the bit rate of the corresponding bitstream. The ±95% confidence intervals for the MOS values are indicated.

Figure 3: MOS-over-rate plots for the DCR experiment

UP = upscaled single layer coded at base resolution, DL = Dual layer, SL = Single layer at full resolution, orig UP = down- and re-upscaled uncompressed sequence (no rate assigned, shown as a horizontal line).

# Discussion

The participating experts are thanked for the efforts.

With a comparably low number of valid scores, the results should be considered as an indication of tendencies. They are not regarded as a necessarily conclusive result.

With respect to the goals of this experiment, the following observations are made:

- Since the UP sequences represent the extracted base layer of the DL streams, the impact of the enhancement layer can be studied. It can be seen that in most cases, the MOS only slightly increases with the inclusion of the enhancement layer. This might be attributed to the fact that other effects overlay the benefit of enhanced detail provided with the DL stream.

- The result show a similar relation of the SL, DL, and UP results compared to JVET-AE0288 for the sequences CorsaireJoy. For TallBuildings2, the results seem to indicate a visual improvement of the DL configuration compared to the UP configuration. This might be attributed to the encoder modifications reported in JVET-AF0186.

- The MOS results for the DrivingPOVLogo sequence seem to indicate that there might be a range of benefit for the DL scheme at rates at or below 1Mbit/s.

- The MOS results for the NxBoxeLogo sequence seem to indicate bitrate ranges where  UP, DL, and SL have benefits, similar to what might be chosen in a bitrate ladder setting.

It is recommended to take these results into account by reconsiderering the test setup with respect to bitrate ranges to be tested for certain resolutions and encoder configurations. The test sequence CorsaireJoy might not include sufficient detail to provide a suitable demonstration case for the targeted application.

# References

- S. Iwamura, P. de Lagrange, M. Wien, “Verification test plan for VVC multilayer coding,” Doc. JVET-AE2022, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 5, 31st meeting, online, July 2023.

- P. de Lagrange, F. Urban, “AHG4: experiments related to VVC spatial scalability visual testing,” Doc. JVET-AF0186, 32nd meeting, Hannover, Oct. 2023.

- M. Wien, “Results of expert viewing for the spatial scalability category of the VVC multilayer VT,” Doc. JVET-AD0399, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 5, 30th meeting, online, Apr. 2023.

- M. Wien, “Results of expert viewing for the spatial scalability category of the VVC multilayer VT,” Doc. JVET-AE0288, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 5, 31st meeting, online, July 2023.

- Recommendation ITU-T P.910 (2008), Subjective video quality assessment methods for multimedia applications.

- Recommendation ITU-R BT.500-14 (2019), Methodologies for the subjective assessment of the quality of television images.